3GPP TSG-RAN5 Meeting #105	R5-247433

# Orlando, USA, 18 Nov – 22 Nov, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3146 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to 6.3E.4 to add intra-band concurrent operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 specified the requirement for intra-band concurrent operation, which needs to be captured in RAN5 test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding minimum requirement for intra-band concurrent operation. Existing NR and sidelink requirement are reused, therefore this test doesn’t need to be repeated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3E.4.0，6.3E.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3E.4	Power control for V2X

6.3E.4.0	Minimum conformance requirements

6.3E.4.0.1	General

When UE is configured for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions for NR V2X operating bands in Table 5.2E.1-1, the following requirements are applied for NR V2X sidelink transmission.

For NR V2X UE supporting SL MIMO, the power control tolerance for single carrier shall apply to the sum of output power at each transmit antenna connector.

If the UE transmits on one antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

6.3E.4.0.2	Absolute power tolerance

The requirements in clause 6.3.4.2.3 shall apply for NR V2X transmission.

6.3E.4.0.3	Power control for V2X con-current operationa

For the inter-band con-current NR V2X operation, the requirements specified in clause 6.3.4.3 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.4.0.1 and 6.3E.4.0.2 shall apply for the sidelink in licensed band or Band n47.

For the intra-band concurrent NR V2X operation, the requirements specified in clause 6.3.4 shall apply for the uplink in licensed band and the requirements specified in clause 6.3E.4 shall apply for the sidelink in licensed band.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3E.4.

6.3E.4.1	Absolute power tolerance for V2X

6.3E.4.1.1	Absolute power tolerance for V2X / non-concurrent operation

6.3E.4.1.1.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e. transmission gap is larger than 20ms.

6.3E.4.1.1.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication.

6.3E.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3E.4.0.

6.3E.4.1.1.4	Test description

6.3E.4.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2E.1-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3E.4.1.1.4.1-1. The details of the V2X reference measurement channels (RMCs) are specified in Annex A.7.5 and the GNSS configuration in TS 38.508-1 [5] subclause 4.11.

Table 6.3E.4.1.1.4.1-1: Test Configuration Table for Absolute Power Tolerance (PSSCH/PSCCH)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.8 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | V2X Configuration to Transmit |  |  |
|  |  | Modulation |  | PSCCH and PSSCH RB allocation (Note 1) |
| 1 | Default | CP-OFDM QPSK |  | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1E-1. |  |  |  |  |

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.1 for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10. Message content exceptions are defined in clause 6.3E.4.1.1.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.3E.4.1.1.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS 38.508-1 [5] Table 4.11.2-2. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.3E.4.1.1.4.2	Test procedure

1.	Ensure the UE is in state Out_of_Coverage with generic procedure parameters Sidelink On according to TS 38.508-1 [5] clause 4.5. The UE is synchronized to GNSS,

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the mean power of the UE in the channel bandwidth according to the test configuration from Table 6.2E.2.1.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.3E.4.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.10 with the following exceptions.

Table 6.3E.4.1.1.4.3-1: SL-ResourcePool for PSCCH/PSSCH Testing

| Derivation Path: TS 38.508-1 [5], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sl-TimeResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| sl-FreqResourcePSCCH-r16 | As defined in Table 6.1E-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-SubchannelSize-r16 | As defined in Table 6.1E-2 |  |  |
| sl-StartRB-Subchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-NumSubchannel-r16 | As defined in Table 6.1E-2 |  |  |
| sl-PowerControl-r16 SEQUENCE { |  |  |  |
| sl-MaxTransPower-r16 | -19 |  | Test point 1 |
|  | 10 |  | Test point 2 |
| sl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSSCH-PSCCH-r16 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r16 | Not present |  |  |
| dl-Alpha-PSFCH-r16 | Not present |  |  |
| dl-P0-PSFCH-r16 | Not present |  |  |
| dl-P0-PSSCH-PSCCH-r17 | Not present |  |  |
| sl-P0-PSSCH-PSCCH-r17 | Not present |  |  |
| dl-P0-PSFCH-r17 | Not present |  |  |
| } |  |  |  |
| sl-TimeResource-r16 | 1000000000000000000000000000000000000000 | 1st slot within 40 slots | 15kHz SCS |
|  | 11000000000000000000000000000000000000000000000000000000000000000000000000000000 | First 2 slots within 80 slots | 30kHz SCS |
|  | 1111000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000000 | First 4 slots within 160 slots | 60kHz SCS |
| } |  |  |  |

6.3E.4.1.1.5	Test requirement

The requirement for the power measured in steps 3 and 4 of the test procedures shall not exceed the values specified in Table 6.3E.4.1.1.5-1.

Table 6.3E.4.1.1.5-1: Absolute power tolerance

|  | Test point / expected output power (dBm) |  |
| --- | --- | --- |
|  | Test point 1 | Test point 2 |
| Expected Measured power Normal conditions | -19 dBm | 10 dBm |
| Power tolerance | ± (9.0+TT) dB | ± (9.0+TT) dB |
| NOTE 1: TT is specified in Table 6.3E.4.1.1.5-2 |  |  |

Table 6.3E.4.1.1.5-2: Test Tolerance for Absolute power tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.4 dB |

6.3E.4.1.1D	Absolute power tolerance for V2X / non-concurrent operation / SL-MIMO

6.3E.4.1.1D.1	Test purpose

Same test purpose as in 6.3E.4.1.1.1.

6.3E.4.1.1D.2	Test applicability

This test case applies to all types of UE release 16 and forward that support NR V2X sidelink communication and SL-MIMO.

6.3E.4.1.1D.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3E.4.0.

6.3E.4.1.1D.4	Test description

Same test description as in 6.3E.4.1.1.4 with following exceptions:

For initial conditions:

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.9.2 for TE diagram and section A.3.2.7 for UE diagram.

The test procedures are replaced by:

1.	Ensure the UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) and Transmit Mode with SL-MIMO. The UE is synchronized to GNSS,

2.	The UE starts to perform the NR sidelink communication according to SL-PreconfigurationNR with 2-layer MIMO codebook TPMI 0. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the NR sidelink RMC.

3.	Measure the sum of the mean power of the UE at each transmit antenna connector in the channel bandwidth according to the test configuration from Table 6.3E.4.1.1.4.1-1. The period of measurement shall be at least continuous duration of one active sub-frame (1ms) excluding guard symbols.

6.3E.4.1.1D.5	Test requirement

The requirement for the power measured in step 3 of the test procedures shall not exceed the values specified in Table 6.3E.4.1.1D.5-1.

Table 6.3E.4.1.1D.5-1: Absolute Power Tolerance

|  | Test point / expected output power (dBm) |  |
| --- | --- | --- |
|  | Test point 1 | Test point 2 |
| Expected Measured power Normal conditions | -19 dBm | 10 dBm |
| Power tolerance | ± (9.0+TT) dB | ± (9.0+TT) dB |
| NOTE 1: TT is specified in Table 6.3E.4.1.1D.5-2 |  |  |

Table 6.3E.4.1.1D.5-2: Test Tolerance for Absolute power tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.4 dB |

6.3E.4.1.2	Absolute power tolerance for V2X / concurrent operation

6.3E.4.1.2.1	Test purpose

Same test purpose as in 6.3E.4.1.1.1 for concurrent operation.

6.3E.4.1.2.2	Test applicability

No test case details are specified. The absolute power tolerance requirements for inter-band concurrent operation and intra-band concurrent operation apply and are tested in clauses 6.3.4.2 for NR Uu operation and 6.3E.4.1.1 for NR sidelink operation.

< End of Changes >