3GPP TSG-RAN5 Meeting #105	R5-247479

# Orlando, USA, November 18 - 22 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0531 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability of NES triggering conditional handover test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Applicability is missing for SA FR2 NES triggering conditional handover test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability has been corrected in Table 4.2-2 for NES triggering conditional handover test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases and the work plan will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

Table 4.2-4: Applicability of RRM NR SA FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Test Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 7.1 | RRC_IDLE state mobility |  |  |  |  |  |  |
| 7.1.1 | NR cell re-selection |  |  |  |  |  |  |
| 7.1.1.1 | NR SA FR2 cell re-selection | FFS | FFS | FFS |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | FFS | FFS | FFS |  |  |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  |
| 7.1.1.7 | NR SA FR2 cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | Rel-17 | C240 | UEs supporting 5GS NR SA FR2 and intra-frequency RRC_IDLE measurements in HST | NOTE 1 |  |  |
| 7.1.1.8 | NR SA FR2 inter-frequency cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | Rel-18 | C345 | UEs supporting 5GS NR SA FR2 and inter-frequency RRC_IDLE measurements in HST | NOTE 1 |  |  |
| 7.2 | RRC_INACTIVE state mobility |  |  |  |  |  |  |
| 7.2.1 | Small Data Transmission |  |  |  |  |  |  |
| 7.2.1.1 | TA Validation for CG-SDT in FR2 | Rel-17 | C095a | UEs supporting 5GS NR SA FR2 and TA Validation for CG-SDT in FR2 | NOTE 1 |  |  |
| 7.3 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 7.3.1 | Handover |  |  |  |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | Rel-16 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | Rel-16 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.3.2 | RRC connection mobility control |  |  |  |  |  |  |
| 7.3.2.1 | RRC re-establishment |  |  |  |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.3.2.1.2 | NR SA FR2 - FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 |  |  |
| 7.3.2.2 | Random access |  |  |  |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 |  |  |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 |  | Subtest 2: F007 |
| 7.3.2.2.4 | NR SA FR2 2-step non-contention based random access | Rel-16 | C160 | UEs supporting 5GS NR SA FR2 and 2-step RACH | NOTE 1 |  |  |
| 7.3.2.3 | RRC connection release with redirection |  |  |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2-FR2 RRC connection release with redirection | FFS | FFS | FFS |  |  |  |
| 7.3.3 | Conditional Handover |  |  |  |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover |  |  |  |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover | NOTE 1 |  |  |
| 7.3.3.3 | NR SA FR2 intra-frequency NES triggering conditional handover | Rel-18 | FFS |  | NOTE 1 |  |  |
| 7.3.3.4 | NR SA FR2-FR1 inter-frequency NES triggering conditional handover | Rel-18 | FFS |  | NOTE 1 |  |  |
| 7.4 | Timing |  |  |  |  |  |  |
| 7.4.1 | UE transmit timing |  |  |  |  |  |  |
| 7.4.1.1 | NR SA FR2 UE transmit timing accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 | Subtest 2: F008 |
| 7.4.1.3 | NR SA FR2 UE Transmit Timing Test with 2-TA for UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | Rel-18 | C349 | UEs supporting 5GS NR SA FR2 and 2 TAGs for multi-DCI multi-TRP operation | NOTE 1 |  |  |
| 7.4.2 | UE timer accuracy |  |  |  |  |  |  |
| 7.4.3 | Timing advance |  |  |  |  |  |  |
| 7.4.3.1 | NR SA FR2 timing advance adjustment accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 |  |
| 7.5 | Signalling characteristics |  |  |  |  |  |  |
| 7.5.1 | Radio Link Monitoring |  |  |  |  |  |  |

/// End of changes