3GPP TSG-RAN5 Meeting #105	R5-247627

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4783 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New testcase 7.1.1.14.2.3 NES Power domain adaptation / Semi-Periodic CSI reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no testcase for NES Power domain adaptation / Semi-Periodic CSI reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding testcase for NES Power domain adaptation / Semi-Periodic CSI reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NES Power domain adaptation / Semi-Periodic CSI reporting functionality is not verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.14.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0535 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a revision of R5-247234. |  |  |  |  |  |  |  |  |

--------------------------------------------------------- Start of change ---------------------------------------------------------

#### 7.1.1.14.2.3	NES Power domain adaptation / Semi-Periodic CSI reporting

7.1.1.14.2.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state configured for Semi-Periodic CSI-RS measurement reporting }

ensure that {

when { UE receives RRCReconfiguration message to report Semi-periodical CSI measurements with CSI-ReportSubconfig with multiple CSI-ReportSubconfigID }

then { UE applies CSI-ReportConfig and sends Semi-periodical CSI measurement reports for all CSI-ReportSubconfig present in CSI-ReportConfig }

}

7.1.1.14.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.2.1.5.2, 5.2.4 and 6.1.3.16. Unless otherwise stated these are Rel-18 requirements.

[TS 38.214, clause 5.2.1.5.2]

…

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. For a selected reporting setting for which the CSI-ReportConfig contains a list of sub-configurations provided by the higher layer parameter csi-ReportSubConfigToAddModList, the activation command can also select one or more sub-configurations to use by the UE as described in clause 6.1.3.16 of [10, TS 38.321]. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH

…

[TS 38.214, clause 5.2.4]

…

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH. The activation command will contain one or more Reporting Settings, with or without containing one or more sub-configurations for each Reporting Setting for which the CSI-ReportConfig contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1

…

[TS 38.321, clause 6.1.3.16]

The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	L: This field indciates whether the MAC CE applies to SP CSI reporting on PUCCH Activation/Deactivation for LTM or not. If ltm-CSI-ReportConfigToAddModList is not configured, R field is present instead (i.e. set to 0);

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within ltm-CSI-ReportConfigToAddModList if L field is set to 1, or csi-ReportConfigToAddModList if L field is set to 0, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId or LTM-CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated. If the Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigToAddModList, the Si field is set to 0 to additionally indicate that all sub-configurations within csi-ReportSubConfigToAddModList shall be deactivated;

-	R: Reserved bit, set to 0.

NOTE:	If a Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigToAddModList, the corresponding Si field is not set to 1.

SP CSI reporting on PUCCH Activation/Deactivation MAC CE

7.1.1.14.2.3.3	         Test description

7.1.1.14.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.14.2.3.3.2	Test procedure sequence

Table 7.1.1.14.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 including CSI-MeasConfig to setup measurement and semiPersistentOnPUCCH in CSI-ReportSubconfig with different powerOffset-r18 | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a SP CSI reporting on PUCCH Activation MAC-CE | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 for all CSI-ReportSubconfig configured in first periodicity? | --> | NR RRC: MeasurementReport | 1 | P |
| 5 | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 for all CSI-ReportSubconfig configured in second periodicity? | --> | NR RRC: MeasurementReport | 1 | P |
| 6 | The SS transmits a SP CSI reporting on PUCCH Deactivation MAC-CE | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 for all CSI-ReportSubconfig configured within two periods? | --> | NR RRC: MeasurementReport | 1 | F |
| 8 | The SS transmits an RRCReconfiguration including CSI-measConfig to remove measurements | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | The UE transmits an RRCReconfigurationComplete message to confirm the remove | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 10 | Check: Does the UE transmit a MeasurementReport message on NR Cell 1 for all CSI-ReportSubconfig configured within two periods? | --> | NR RRC: MeasurementReport | 1 | F |

7.1.1.14.2.3.3.3	Specific message contents

Table 7.1.1.14.2.3.3.3-1: RRCReconfiguration (Step 1 and Step 8, Table 7.1.1.14.2.3.3.2-1)

Derivation Path: TS 38.508-1 [4] table 4.6.1-13 with condition NR_MEAS

Table 7.1.1.14.2.3.3.3-2: ServingCellConfig (Table 7.1.1.14.2.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig |  |  |
| } |  |  |  |

Table 7.1.1.14.2.3.3.3-3: CSI-MeasConfig (Table 7.1.1.14.2.3.3.3-2)

Derivation Path: TS 38.508-1 [4], clause 4.6.3-38

Table 7.1.1.14.2.3.3.3-4: CSI-ReportConfig (Step 1, Table 7.1.1.14.2.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18 { | 2 entries |  |  |
| CSI-ReportSubConfig-r18[1] SEQUENCE{ |  | Entry 1 |  |
| reportSubConfigId-r18 | 0 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig |  |  |
| portSubsetIndicator-r18 CHOICE { | TBD |  |  |
| } |  |  |  |
| non-PMI-PortIndication-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks { | 1 entry |  |  |
| PortIndexFor8Ranks[1] | TBD |  |  |
| } |  |  |  |
| } |  |  |  |
| a2-parameters SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 { |  |  |  |
| NZP-CSI-RS-ResourceIndex-r18[1] | TBD |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| CSI-ReportSubConfig-r18[2] SEQUENCE{ |  | Entry 2 |  |
| reportSubConfigId-r18 | 1 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig |  |  |
| portSubsetIndicator-r18 CHOICE { | TBD |  |  |
| } |  |  |  |
| non-PMI-PortIndication-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks { | 1 entry | PortIndexFor8Ranks |  |
| PortIndexFor8Ranks[1] | TBD |  |  |
| } |  |  |  |
| } |  |  |  |
| a2-parameters SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 { |  |  |  |
| NZP-CSI-RS-ResourceIndex-r18[1] | TBD |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.3.3.3-5: CSI-ReportConfig (Step 8, Table 7.1.1.14.2.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 1 entry |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 |  |  |
| CSI-ReportSubConfigId-r18[2] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

--------------------------------------------------------- End of change ---------------------------------------------------------