3GPP TSG-RAN5 Meeting #105	R5-247640

# Orlando, US, 18th Nov 2024 – 22th Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5373 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case related to AC-Barring per PLMN 13.5.1d |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussion paper(R5-242949) mentioned, Japan will use following 3GPP features to add the affected user to Relieving Carrier. -Access Control per PLMN So we need to introduce test cases related to ac-BarringPerPLMN-List for stability of connectivity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case related to AC-Barring per PLMN 13.5.1d |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior after receiving some parameters on ac-BarringPerPLMN-List will not be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/36.523-2 CR 1475 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-246626 R5-246626r1 R5-246626r2 R5-247640 |  |  |  |  |  |  |  |  |

[Start of change]

13.5.1d	MTSI MO speech call / SSAC in Connected mode / 0% access probability for MTSI MO speech call / AC-Barring per PLMN

13.5.1d.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state having received a SysteminformationBlockType2 message including a ssac-BarringForMMTEL-Voice-r12 set to 0% accessibility }

ensure that {
  when { the user initiates a MTSI MO voice call }

then { the UE does not initiate a MTSI MO voice call }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state having received a SysteminformationBlockType2 message including a ssac-BarringForMMTEL-Voice-r12 and having failed to initiate a voice call }

ensure that {
  when { ssac-BarringForMMTEL-Voice-r12 is removed and the user initiates a MTSI MO voice call during ac-BarringTime is running }

then { the UE does not initiate a MTSI MO voice call }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state having received a SysteminformationBlockType2 message including a ssac-BarringForMMTEL-Voice-r12 and having failed to initiate a voice call }

ensure that {
  when { the user initiates a MTSI MO voice call afar expiring ac-BarringTime}

then { the UE initiates a MTSI MO voice call }

}

13.5.1d.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.173 clause J.2.1.1 and TS 36.331 clause 5.3.3.10. Unless otherwise stated these are Rel-12 requirements.

[TS 24.173, clause J.2.1.1]:

The following information is provided by lower layer:

-	BarringFactorForMMTEL-Voice: barring rate for MMTEL voice;

-	BarringTimeForMMTEL-Voice: barring timer for MMTEL voice;

…

Upon request from a user to establish a multimedia telephony communication session as described in subclause 5.2, the UE shall:

1)	if the multimedia telephony communication session to be established is an emergency session, then skip the rest of steps below and continue with session establishment as described in subclause 5.2;

2)	retrieve SSAC related information mentioned above from lower layers;

3)	if video is offered in the multimedia telephony communication session:

…

4)	if audio is offered in the multimedia telephony communication session:

A)	if back-off timer Ty is running, reject the multimedia telephony communication session establishment and skip the rest of steps below; or

B)	else, then;

I)	draw a new random number "rand3" that is uniformly distributed in the range 0 ≤ rand3 < 1; and

II)	if the random number "rand3" is lower than BarringFactorForMMTEL-Voice, then skip the rest of steps below and continue with session establishment as described in subclause 5.2;

III)	else, then;

i)	draw a new random number "rand4" that is uniformly distributed in the range 0 ≤ rand4 < 1; and

ii)	start timer Ty with the timer value calculated using the fomula:

Ty =  (0,7 + 0,6*rand4) * BarringTimeForMMTEL-Voice; and

iii)	reject the multimedia telephony communication session establishment;

NOTE:	If the multimedia telephony communication implemenation and the access stratum protocol implementation are located in separate physical entities, it is expected that the interconnecting protocol supports the transfer of information elements needed for the service specific access control enforcement.

Service Specific Access Control is not activated when the UE is in other radio accesses (e.g. UTRAN/GERAN). And when UE camping on E-UTRAN moves to other radio accesses (e.g. UTRAN/GERAN), back-off timer (Tx or Ty or both) shall be stopped if running.

[TS 36.331 clause 5.3.3.10]:

Upon request from the upper layers, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1>	else:

2>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

1>	set the local variables BarringFactorForMMTEL-Voice and BarringTimeForMMTEL-Voice as follows:

2>	if ssac-BarringForMMTEL-Voice is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

NOTE:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

3>	if, for at least one of these Access Classes, the corresponding bit in the ac-BarringForSpecialAC contained in ssac-BarringForMMTEL-Voice is set to zero:

4>	set BarringFactorForMMTEL-Voice to one and BarringTimeForMMTEL-Voice to zero;

3>	else:

4>	set BarringFactorForMMTEL-Voice and BarringTimeForMMTEL-Voice to the value of ac-BarringFactor and ac-BarringTime included in ssac-BarringForMMTEL-Voice, respectively;

2>	else set BarringFactorForMMTEL-Voice to one and BarringTimeForMMTEL-Voice to zero;

1>	set the local variables BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video as follows:

2>	if ssac-BarringForMMTEL-Video is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	if, for at least one of these Access Classes, the corresponding bit in the ac-BarringForSpecialAC contained in ssac-BarringForMMTEL-Video is set to zero:

4>	set BarringFactorForMMTEL-Video to one and BarringTimeForMMTEL-Video to zero;

3>	else:

4>	set BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video to the value of ac-BarringFactor and ac-BarringTime included in ssac-BarringForMMTEL-Video, respectively;

2>	else set BarringFactorForMMTEL-Video to one and BarringTimeForMMTEL-Video to zero;

1>	forward the variables BarringFactorForMMTEL-Voice, BarringTimeForMMTEL-Voice, BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video to the upper layers;

13.5.1d.3	Test description

13.5.1d.3.1	Pre-test conditions

System Simulator:

-	Cell 1 as specified in TS 36.508[18] clause 4.4.1.2 are configured as shown in Table 13.5.1d.3.1–1.

Table 13.5.1d.3.1-1: PLMN identifiers

| Cell | PLMN names | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN1 | 001 | 01 |
|  | PLMN2 | 002 | 11 |

UE:

-	The UE is equipped with a test USIM with PLMN2 is set in it as 'forbidden PLMN'.

Preamble:

-	The UE is in state Registered for PLMN1, Generic RB Established (state 3) on Cell 1 according to [18].

13.5.1d.3.2	Test procedure sequence

Table 13.5.1d.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt a MTSI MO Speech call | - | - | - | - |
| 2 | Check: Does the UE transmit an INVITE message with in 10s? | --> | INVITE | 1 | F |
| 3 | SS changes SIB2 according to TS 36.508, table 4.4.3.3-1 and transmits a Paging message including systemInfoModification. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 4 | Wait for 15 s (Note 1) to allow the new system information to take effect. | - | - | - | - |
| 5 | Make the UE attempt a MTSI MO Speech call | - | - | - | - |
| 6 | Check: Does the UE transmit an INVITE message within 10s? | --> | INVITE | 2 | F |
| 7 | Wait for 132 s (Note 2) to make the ac-BarringTime Ty 128s expire. | - | - | - | - |
| 8 | Make the UE attempt a MTSI MO Speech call | - | - | - | - |
| 9 | Check: Does the UE transmit an INVITE message within 10 s? | --> | INVITE | 3 | P |
| 10-20 | Steps 3 to 13 of the generic procedure for MTSI MO speech call establishment (TS 34.229-1 C.21) | - | - | - | - |
| Note 1: The wait time of 15 s in step 4 is to allow for the network to page the system information change during the next modification period, and update the system information at the subsequent modification period. UE should acquire the updated system information within 100ms of the start of modification period. Note 2: The UE starts ac-BarringTime 128s in step2. Maximum time is 166.4 sec ((0.7 + 0.6 * 1) * s128). At the end of step 6, 35 sec elapses from step 2. Therefore about 132 sec (166.4s - 35s = 131.4) is enough to wait timer expiry. |  |  |  |  |  |

13.5.1d.3.3	Specific message contents

Table 13.5.1d.3-3-1: SystemInformationBlockType1 for Cell 1 (preamble, Table 13.5.1d.3.2-1)

| Derivation path: TS 36.508[18], Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..6)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity | 00101 |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { |  | entry 2 |  |
| plmn-Identity | 00211 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.5.1d.3.3-2: SystemInformationBlockType2 for Cell 1 (preamble, Table 13.5.1d.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12 { | 1 entry |  |  |
| AC-BarringPerPLMN-r12[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| ac-BarringInfo-r12 SEQUENCE { |  |  |  |
| ac-BarringForEmergency-r12 | FALSE |  |  |
| } |  |  |  |
| ssac-BarringForMMTEL-Voice-r12 SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s128 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.5.1d.3.3-3: SystemInformationBlockType2 for Cell 1 (step 3, Table 13.5.1d.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ac-BarringPerPLMN-List-r12 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF AC-BarringPerPLMN-r12 { |  |  |  |
| AC-BarringPerPLMN-r12[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityIndex-r12 | 1 | PLMN1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.5.1d.3.3-4: Paging (step3, Table 13.5.1d.3.2-1)

| Derivation Path: TS 36.508[18], Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

[End of change]