3GPP TSG-RAN5 Meeting #105	R5-247713

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0753 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New pic definition for CA HST-SFN UE capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_HST_FR1_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test applicability for demod performance test cases 5.2A.2.4.1, 5.2A.3.4.1 is asking the NR UE to support enhanced demodulation processing for carrier aggregation for HST-SFN joint transmission scheme. This is UE capability defined in TS 38.306 clause 4.2.7.4. However, there is no entry in TS 38.508-2 for this capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To include new pic for UE capability to support CA HST-SFN joint transmission scheme in Table A.4.3.11-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA HST-SFN Test case will not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-247347 adding clauses affected in cover page |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### A.4.3.11	High Speed Capabilities

Table A.4.3.11-1: High Speed Capabilities

| Item | UE High Speed Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support the enhanced intra-NR and inter-RAT E-UTRAN measurement requirements to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-16 | pc_hst_meas_enh_r16 | No |  |  |
| 2 | Support the enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | 38.306, 4.2.19 | Rel-16 | pc_hst_demod_enh_r16 | No |  |  |
| 3 | Support the enhanced intra-NR RRM requirements to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-16 | pc_hst_intraNR_meas_enh_r16 | No |  | This PICS can only be set to true when pc_hst_meas_enh_r16 is set to false. Up to one PICS between pc_hst_intraNR_meas_enh_r16 and pc_hst_interRAT_meas_enh_r16 can be set to true |
| 4 | Supports the enhanced inter-RAT E-UTRAN RRM requirements to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-16 | pc_hst_interRAT_meas_enh_r16 | No |  | This PICS can only be set to true when pc_hst_meas_enh_r16 is set to false. Up to one PICS between pc_hst_intraNR_meas_enh_r16 and pc_hst_interRAT_meas_enh_r16 can be set to true |
| 5 | Support for enhanced inter-RAT NR measurement requirements in high-speed scenario | 36.306,4.3.33.7 | Rel-16 | pc_hst_interRAT_NR_meas_enh_r16 | No |  |  |
| 6 | Supports the enhanced RRM requirements for carrier aggregation to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-17 | pc_hst_RRM_CA_enh_r17 | No |  | FR1 only UE indicating support of this feature shall indicate support of measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |
| 7 | Supports the enhanced RRM requirements for inter-frequency measurements in connected mode to support high speed up to 500 km/h | 38.306, 4.2.19 | Rel-17 | pc_hst_RRM_interfreq_meas_enh_r17 | No |  | FR1 only UE indicating support of this feature shall indicate support of measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |
| 8 | Support the enhanced RRM requirements for inter-frequency IDLE/INACTIVE measurements to support high speed up to 500 km/h | 38.306, 5.6 | Rel-17 | pc_hst_RRM_interfreq_idle_inactive_meas_enh_r17 | No |  | FR1 only UE indicating support of this feature shall indicate support of measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. |
| 9 | Indicates whether the UE supports one shot large UL timing adjustment. | 38.306, 4.2.7.2 | Rel-17 | pc_hst_oneStep_UL_Timing_adj_r17 | No |  | FR2 only |
| 10 | Support the enhanced intra-NR RRM requirements to support high-speed up to 350 km/h for FR2 | 38.306, 4.2.7.2 | Rel-17 | pc_hst_MeasFlagFR2_r17 | No |  | FR2 only UE indicating support of this feature shall indicate support of ue-PowerClass-v1700 set to 'pc6'. |
| 11 | Support the enhanced one-shot large UL transmit timing adjustment requirement to support FR2 PC6 UEs and enhanced TCI state switching delay requirements in HST FR2 | 38.306, 4.2.7.2 | Rel-18 | pc_hst_tciStateSwitchInd_r18 | No |  | FR2 only UE indicating support of this feature shall indicate support of ue-PowerClass-v1700 set to 'pc6'. |
| 12 | Supports the enhanced demodulation processing for carrier aggregation for HST-SFN joint transmission scheme with velocity up to 500km/h | 38.306, 4.2.7.4 | Rel-17 | pc_hst_demod_CA_enh_r17 | No |  | FR1 only. UE indicating support of this feature shall indicate support of demodulationEnhancement-r16. |

## <<< END OF CHANGES >>>