3GPP TSG-RAN WG5 Meeting # 105	R5-247723

# Orlando, US, November 18 – 22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3136 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test configuration and test requirement for 3DL CA reference sensitivity exceptions for CA_n3A-n5A-n78A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update test configuration and test requirement for 3DL CA reference sensitivity exceptions for CA_n3A-n5A-n78A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include reference sensitivity power level for 3DL CA exceptions for CA_n3A-n5A-n78A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test of 3DL CA reference sensitivity exceptions for NR CA configuration CA_n3A-n5A-n78A will not be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.5, 7.3A.2.5, 7.3A.2_1.4.1, 7.3A.2_1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.905 CR 0970 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-247391r1: Undo the changes in Table 7.3A.2_1.5-1 and move the changes to R5-247015 to resolve the overlapping issue. |  |  |  |  |  |  |  |  |

<< Start of changes >>

7.3A.0.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands given in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a, the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a. For these test points the reference sensitivity requirement specified in Table 7.3.2.3-1 and Table 7.3.2.3-2 are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a.

<<Unchanged texts are omitted>>

Editor’s Note:

The n78 UL RB size of 10MHz BW in CA_n1-n3-n78 is different from the Table 7.3A.5-2 in TS 38.101-1 (V17.9.0), since the values in the latter cannot be applied to DFT-s-OFDM. The closest smaller valid value has been applied in Table 7.3A.0.5-2.

Table 7.3A.0.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n28 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | FDD | IMD4 |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD22 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 31.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 31.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD2 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3360 | 11.2 | TDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 27.9 | FDD | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n1-n5-n78 | n1 | N/A | 5 | N/A | 2122 | 18.1 | FDD | IMD3 |
|  | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 885 | 3.1 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3610 | 15.7 | TDD | IMD3 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n8-n78 | n1 | 1945 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3745 | 14.9 | TDD | IMD3 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 3.3 | FDD | IMD5 |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
| CA_n1-n28-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3416 | 15.7 | TDD | IMD32 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 4.2 | FDD | IMD5 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2150 | 15.7 | FDD | IMD3 |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 15.7 | FDD | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 4.2 | FDD | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3416 | 15.7 | TDD | IMD3 |
| CA_n1-n41-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3330 | 19.6 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | TDD | IMD41 |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 9.3 | FDD | IMD4 |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 15.6 | FDD | IMD31,2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n2-n5-n30 | n2 | 1870 | 5 | 25 | 1959 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 9.7 | FDD | IMD4 |
|  | n30 | 2310 | 10 | 50 | 2355 | N/A | FDD | N/A |
| CA_n2-n5-n48 | n2 | N/A | 5 | N/A | 1962 | 15.6 | FDD | IMD3 |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n48 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | 50 | 3593 | 16.6 | TDD | IMD3 |
| CA_n2-n5-n66 | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2140 | 7.2 | FDD | IMD4 |
| CA_n2-n5-n77 | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 887.5 | 3.8 | FDD | IMD55 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987 | 16.5 | FDD | IMD35 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3540 | 16.0 | TDD | IMD31 |
| CA_n2-n12-n77 | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD32 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3315 | 10 | 50 | 3315 | 16.0 | TDD | IMD31,2 |
| CA_n2-n14-n77 | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD3 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3466 | 10 | 50 | 3466 | 16.0 | TDD | IMD31 |
| CA_n2-n30-n77 | n2 | 1906 | 5 | 25 | 1986 | 8.6 | FDD | IMD4 |
|  | n30 | 2312 | 5 | 25 | 2357 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n30 | 2309 | 5 | 25 | 2354 | 10.6 | FDD | IMD41 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | TDD | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | 29.4 | TDD | IMD22 |
| CA_n2-n48-n66 | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | 50 | 3625 | 32.0 | TDD | IMD2 |
|  | n66 | 1770 | 5 | 25 | 2190 | N/A | FDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n48 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2155 | 12.1 | FDD | IMD4 |
|  | n2 | N/A | 5 | N/A | 1960 | 28.3 | FDD | IMD21 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n2-n66-n77 | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD25 |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3900 | 8.9 | TDD | IMD4 |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2115 | 29.2 | FDD | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2140 | 10.4 | FDD | IMD4 |
|  | n77 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2175 | 4.0 | FDD | IMD5 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 9.1 | FDD | IMD45 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 2.1 | FDD | IMD55 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| CA_n3-n5-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3408 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3512 | 4.5 | TDD | IMD5 |
|  | n3 | N/A | 5 | N/A | 1862 | 15.7 | FDD | IMD3 |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n3-n8-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3550 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3370 | 4.5 | TDD | IMD5 |
|  | n3 | N/A | 5 | N/A | 1820 | 15.7 | FDD | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
| CA_n3-n28-n41 | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | TDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 26.0 | FDD | IMD24 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
|  | n3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | FDD | IMD2 |
| CA_n3-n28-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | TDD | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | FDD | IMD3 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 15.3 | FDD | IMD3 |
| CA_n3-n28-n78 | n3 | 1755 | 5 | 25 | 1850 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3320 | 17.3 | TDD | IMD3 |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3764 | 4.5 | TDD | IMD5 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n41 | N/A | 5 | N/A | 2640 | 5.3 | TDD | IMD5 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1840 | 16.4 | FDD | IMD3 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 16.8 | TDD | IMD31 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12-n77 | n5 | 835 | 5 | 25 | 880 | 3.9 | FDD | IMD5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n12 | 710 | 5 | 25 | 740 | 4.4 | FDD | IMD5 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | 4.4 | TDD | IMD5 |
| CA_n5-n14-n77 | n5 | 835 | 5 | 25 | 880 | 3.9 | FDD | IMD5 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 4052 | 10 | 50 | 4052 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 11.6 | FDD | IMD41 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3298 | 10 | 50 | 3298 | 10.3 | TDD | IMD41 |
| CA_n5-n25-n66 | n5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1712 | 5 | 25 | 2132 | 7.2 | FDD | IMD4 |
| CA_n5-n25-n77 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | TDD | IMD3 |
|  | n5 | 844 | 5 | 25 | 889 | 3.8 | FDD | IMD5 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | 1907 | 5 | 25 | 1987 | 16.5 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| CA_n5-n30-n66 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n30 | 2307.5 | 5 | 25 | 2352.5 | N/A | FDD | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 4 | FDD | IMD5 |
| CA_n5-n30-n77 | n5 | 835 | 5 | 25 | 880 | 15.2 | FDD | IMD3 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | TDD | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | 16.1 | TDD | IMD3 |
| CA_n5-n48-n66 | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | N/A | 3622 | 3.6 | TDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
| CA_n5-n66-n77 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3465 | 16.1 | TDD | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4192 | 8.2 | TDD | IMD45 |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3535 | 3.3 | TDD | IMD5 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2142 | 13.2 | FDD | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n12-n30-n77 | n12 | 710 | 5 | 25 | 740 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3913 | 10 | 50 | 3913 | 16.0 | TDD | IMD3 |
| CA_n12-n66-n77 | n12 | 710 | 5 | 25 | 740 | 15.2 | FDD | IMD3 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n66 | 1746 | 5 | 25 | 2146 | 13.2 | FDD | IMD3 |
|  | n77 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
|  | n12 | 704 | 5 | 25 | 734 | N/A | FDD | N/A |
|  | n66 | 1723 | 5 | 25 | 2123 | N/A | FDD | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | 16.0 | TDD | IMD31,2 |
| CA_n14-n30-n77 | n14 | 793 | 5 | 25 | 763 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3896 | 10 | 50 | 3896 | 16.0 | TDD | IMD3 |
| CA_n14-n66-n77 | n14 | 793 | 5 | 25 | 763 | 15.2 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1755 | 5 | 25 | 2155 | 13.2 | FDD | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1755 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n77 | 3341 | 10 | 50 | 3341 | 16.0 | TDD | IMD31,2 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n66-n77 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | FDD | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | 10 | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD2 |
| CA_n25-n71-n77 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | 8.0 | TDD | IMD31,2 |
|  | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n77 | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD24 |
|  | n41 | 2567.5 | 10 | 50 | 2567.5 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n28 | 727.5 | 5 | 25 | 782.5 | 3.0 | FDD | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | 3323 | 10 | 50 | 3323 | 28.2 | TDD | IMD24 |
| CA_n28-n41-n79 | n28 | N/A | 5 | N/A | 780 | 13.0 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4600 | 10.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2645 | 10.4 | TDD | IMD4 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 52 | 3620 | N/A | N/A | n77 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A | n79 |
|  | n28 | 745 | 5 | 25 | 800 | 16.2 | IMD21,2 | n28 |
| CA_n30-n66-n77 | n30 | 2310 | 5 | 25 | 2355 | 29.2 | FDD | IMD21 |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 8.7 | FDD | IMD4 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4055 | 10 | 50 | 4055 | 28.4 | TDD | IMD21 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n66-n77 | n41 | 2560 | 5 | 25 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD31,2 |
|  | n41 | 2670 | 5 | 25 | 2670 | 5.2 | TDD | IMD5 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2530 | 5 | 25 | 2530 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21 |
|  | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | 16.3 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
|  | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2615 | 5 | 25 | 2615 | 15.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | N/A | TDD | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n71 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| CA_n48-n66-n70 | n48 | 3625 | 10 | 50 | 3625 | N/À | TDD | N/A |
|  | n66 | 1742.5 | 5 | 25 | 2142.5 | 2.8 | FDD | IMD5 |
|  | n70 | 1702.5 | 5 | 25 | 2002.5 | N/A | FDD | N/A |
| CA_n48-n66-n71 | n48 | 3552.5 | 10 | 50 | 3552.5 | N/A | TDD | N/A |
|  | n66 | 1761.5 | 5 | 25 | 2161.5 | 14.4 | FDD | IMD3 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n48 | 3695 | 10 | 50 | 3695 | 5.2 | TDD | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n48-n70-n71 | n48 | 3694 | 10 | 50 | 3694 | 9 | TDD | IMD41 |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n77 | 4108 | 10 | 50 | 4108 | 15.9 | TDD | IMD31,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | FDD | IMD3 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| CA_n66-n71-n78 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3724 | 9 | TDD | IMD41 |
|  | n66 | N/A | 5 | N/A | 2160 | 15.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |

<<Unchanged texts are omitted>>

7.3A.2	Reference sensitivity power level for 3DL CA without exceptions

7.3A.2.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 3DL CA.

7.3A.2.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.2.4	Test description

7.3A.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.2A.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2.4.1-1: Test Configuration Table for 3DL CA without exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For Inter-band CA: CA_nXA-nYA-nZA: Mid range for PCC and SCC with exceptions (Note 11): CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA :Low range, High Range for nXC and nXB, mid range for nYA for PCC and SCC with exceptions : CA configurations containing the following band combinations: CA_n1-n77: Mid in band n1 and Low in band n77 CA_n2-n77: UL and DL Mid in band n2 and band n77 at 3850MHz CA_n3-n77: TBD in band 3 and TBD in band 77. CA_n3-n78: Mid in band 3 and High in band 78. CA_n8-nX: Low range for PCC in Band 8 CA_n70-n71: High range for PCC in band 71 |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  | Refer to “NRB_PCC”, “NRB_SCC1” and “NRB_SCC2” columns |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0 to NOTE 5) |  |  |
|  | CA configuration |  |  |  |  |  |  |  | PCC NRB | SCC1 NRB | SCC2 NRB | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | Low CC1 | N/A | nX | Low CC2 | N/A | nX | Low CC3 | Highest NRB_agg | Highest NRB_agg | Highest NRB_agg | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nX | High CC1 | N/A | nX | High CC2 | N/A | nX | High CC3 | Highest NRB_agg | Highest NRB_agg | Highest NRB_agg | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0 to NOTE 5) |  |  |
|  | CA configuration |  |  |  |  |  |  |  | PCC NRB | SCC1 NRB | SCC2 NRB | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | N/A | nY | default | N/A | nZ | default | Highest (NOTE 12) | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nZ | default | N/A | nX | default | Highest (NOTE 12) | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 3 | nZ | default | N/A | nY | default | N/A | nX | default | Highest (NOTE 12) | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | N/A | nX | default | N/A | nY | default | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX | default | N/A | nX | default | Highest (NOTE 12) | Highest NRB_agg | Highest NRB_agg | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0 to NOTE 5) |  |  |
|  | CA configuration |  |  |  |  |  |  |  | PCC NRB | SCC1 NRB | SCC2 NRB | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 | N/A | nY | Mid | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | Mid | NA | nX | CC1 | Max (NOTE 7) | nX | CC2 | Highest (NOTE 12) | Highest NRB_agg | Highest NRB_agg | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nX(3A) Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 | Max (NOTE 7) | nX | CC3 | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nX(A-C) and CA_nX(A-B) Configuration (Intra-band contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 | N/A | nX | CC3 | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2.0: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: Inter-band: nX,nY,nZ correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, nX=n1, nY=n3, nZ=n8. Note 4: Intra-band contiguous + Inter-band: nX, nY correspond to the different bands in the CA Configuration, e.g. for CA_n1C-n3A, nX=n1, nY=n3 Note 5: Intra-band non-contiguous + Inter-band: nX and nY correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, nX=n1, nY =n8. Note 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 7: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration for Intra-band non-contiguous Note 8: For band combinations including operating bands without uplink band (as noted in Table 5.2-1), only the CA configurations where PCC band has uplink band shall be tested Note 9: The fallback configuration CA_nXA-nYA for 3CA configurations CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA does not need to be tested even if the test frequency differs Note 10: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. Note 11: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. Note 12: Each of Highest UL and Highest DL shall be selected according to clause 5.5A.3.2. DL channel bandwidth shall be selected first. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.2.4.1-2: Void

Table 7.3A.2.4.1-3: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.3A.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.2.1.4.3.

7.3A.2.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.3A.2.1.4.3.

3.	SS activates SCCs by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1-1 for C_RNTI to transmit the DL RMC according to Tables 7.3A.1.4.1-1 on both PCC and SCC.  The SS sends downlink MAC padding bits on the DL RMC

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3A.2.4.1-1 on PCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3.2.5-1 and 7.3.2.5-2 as appropriate. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the throughput measurement.  Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

7.3A.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.3A.2.5	Test requirement

For 3DL carrier aggregation, test parameters are specified in table 7.3A.2.4.1-1. For the CA configurations listed in table 7.3A.2.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1 for each non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2 for each non-SDL carrier for 4 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous CA with one uplink carrier and two or more downlink sub-blocks, the test requirement for SCC(s) shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.5-1: Reference sensitivity requirement for 3DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
| Intra-band non-contiguous 3DL CA | CA_n48(3A) | - |
|  | CA_n66(3A) | - |
| Inter-band 3DL CA | CA_n1A-n78C |  |
|  | CA_n1A-n78(2A) | - |
|  | CA_n1A-n8A-n78A |  |
|  | CA_n1A-n28A-n78A | - |
|  | CA_n1A-n78A-n79A | - |
|  | CA_n1A-n3A-n28A | - |
|  | CA_n1A-n3A-n77A |  |
|  | CA_n1A-n3A-n78A | - |
|  | CA_n1A-n5A-n78A |  |
|  | CA_n1A-n28A-n77A |  |
|  | CA_n1A-n28A-n78A | - |
|  | CA_n1A-n41A-n77A |  |
|  | CA_n2A-n5A-n48A |  |
|  | CA_n2A-n5A-n66A |  |
|  | CA_n2A-n5A-n77A |  |
|  | CA_n2A-n48A-n66A |  |
|  | CA_n2A-n48A-n77A |  |
|  | CA_n2A-n66A-n77A |  |
|  | CA_n2A-n48(2A) |  |
|  | CA_n2A-n48B |  |
|  | CA_n2A-n77C |  |
|  | CA_n3A-n5A-n78A |  |
|  | CA_n3A-n8A-n78A |  |
|  | CA_n3A-n28A-n41A |  |
|  | CA_n3A-n28A-n77A |  |
|  | CA_n3A-n28A-n78A | - |
|  | CA_n3A-n41A-n77A |  |
|  | CA_n3A-n77(2A) |  |
|  | CA_n5A-n48A-n66A |  |
|  | CA_n5A-n48A-n77A |  |
|  | CA_n5A-n66A-n77A |  |
|  | CA_n5A-n48(2A) |  |
|  | CA_n5A-n48B |  |
|  | CA_n5A-n77C |  |
|  | CA_n5B-n77A |  |
|  | CA_n25A-n66A-n77A | - |
|  | CA_n25A-n66A-n78A | - |
|  | CA_n26A-n66-n70A |  |
|  | CA_n26A-n66(2A) |  |
|  | CA_n28A-n41A-n77A |  |
|  | CA_n28A-n41C |  |
|  | CA_n28A-n41A-n79A | - |
|  | CA_n28A-n77(2A) |  |
|  | CA_n41A-n66A-n71A |  |
|  | CA_n41A-n79C |  |
|  | CA_n41C-n79A |  |
|  | CA_n48A-n66(2A) |  |
|  | CA_n48A-n71(2A) |  |
|  | CA_n48A-n77C |  |
|  | CA_n48B-n66A |  |
|  | CA_n48B-n70A |  |
|  | CA_n48B-n71A |  |
|  | CA_n48B-n77A |  |
|  | CA_n48(2A)-n66A |  |
|  | CA_n48(2A)-n70A |  |
|  | CA_n48(2A)-n71A |  |
|  | CA_n48(2A)-n77A |  |
|  | CA_n48A-n66A-n70A |  |
|  | CA_n48A-n66A-n71A |  |
|  | CA_n48A-n66A-n77A |  |
|  | CA_n48A-n70A-n71A |  |
|  | CA_n66A-n70A-n71A | - |
|  | CA_n66A-n71A-n77A | - |
|  | CA_n66A-n71A-n78A | - |
|  | CA_n66A-n71(2A) |  |
|  | CA_n66(2A)-n70A | - |
|  | CA_n66(2A)-n71A | - |
|  | CA_n66B-n70A | - |
|  | CA_n66B-n71A | - |
|  | CA_n66A-n77C |  |
|  | CA_n70A-n71A(2A) |  |
| SDL configuration | CA_n29A-n66A-n70A | - |
|  | CA_n29A-n66B | - |
|  | CA_n29A-n66(2A) | - |

7.3A.2_1	Reference sensitivity power level for 3DL CA exceptions

7.3A.2_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.2_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support inter-band NR 3DL CA with UL CA support.

7.3A.2_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.2_1.4	Test description

7.3A.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “NRB_PCC”and “NRB_SCC” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL allocation (Note 2) |  |  |
|  | CA configuration |  |  |  |  |  | PCC | SCC1 | SCC2 | CC Mod | PCC & SCC RB allocation |  |  | CC Mod | PCC, SCC1 or SCC2 RB allocation |  |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  |  |  | PCC | SCC1 | SCC2 |  |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n77 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1775/ DL 1870 | n77 | 3915 | n1 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n78 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n1 | UL 1950/ DL 2140 | n3 | UL 1770/ DL 1865 | n78 | 3360 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n78 | 3780 | n3 | UL 1770/ DL 1865 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n5A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/DL 874 | n78 | 3780 | n1 | DL 2122 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL1975/DL2165 | n78 | 3405 | n5 | DL 885 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n1A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL1950/DL2140 | n5 | UL830/DL875 | n78 | DL 3610 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1945/ DL 2135 | n8 | UL 900/ DL 945 | n78 | DL 3745 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1940/ DL 2130 | n78 | 3380 | n8 | DL 940 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n77 | 3630 | n1 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n41A-n77A_UL_n1A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1970/ DL 2160 | n41 | 2650 | n77 | 3330 | 5MHz | 10MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2640 | n77 | 3710 | n1 | DL 2140 | 10MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n1A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n28 | UL 733/ DL 788 | n78 | 3416 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1970/ DL 2160 | n78 | 3352 | n28 | DL 794 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n28A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n78 | 3630 | n1 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n66A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1990/ DL 1980 | n5 | UL 830/ DL 875 | n66 | DL 2140 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n5A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n48 | 3640 | n2 | DL 1962 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n5 | UL 844/ DL 1889 | n48 | DL 3593 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1907.5/ DL 1987.5 | n77 | 3305 | n5 | DL 887.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 846.5/ DL 891.5 | n77 | 3680 | n2 | DL 1987 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n5 | UL 830/ DL 875 | n77 | DL 3540 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n66 | UL 1770/ DL 2190 | n48 | DL 3625 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n48 | 3560 | n66 | DL 2155 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n48A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1735/ DL 2135 | n48 | 3695 | n2 | DL 1960 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3620 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3900 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n77 | 3970 | n66 | DL 2115 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n77 | 3500 | n66 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n2 | UL 1885/ DL 1965 | n77 | 3915 | n66 | DL 2175 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1760/ DL 2160 | n77 | 3720 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n66 | UL 1760/ DL 2160 | n77 | 3350 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n66 | UL 1760/ DL 2160 | n77 | 3620 | n2 | DL 1960 | 5Mhz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n5A-n78A_UL_n3A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL1825 | n5 | UL 839/ DL 884 | n78 | DL 3408 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1730/ DL1825 | n5 | UL 839/ DL 884 | n78 | DL 3512 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n5A-n78A_UL_n5A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n78 | 3540 | n3 | DL 1862 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n8A-n78A_UL_n3A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL 1825 | n8 | UL 910/ DL 955 | n78 | DL 3550 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1730/ DL 1825 | n8 | UL 910/ DL 955 | n78 | DL 3370 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n8A-n78A_UL_n8A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | UL 910/ DL 955 | n78 | 3640 | n3 | DL 1820 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2518 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2687 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2510 | n28 | UL 735/ DL 790 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 710.5/ DL 765.5 | n41 | 2543 | n3 | UL 1737.5/ DL 1832.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n28 | UL 733/ DL 788 | n77 | 4173 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n77 | 3320 | n3 | UL 1755/ DL 1850 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1712.5/ DL 1807.5 | n77 | 4195 | n28 | UL 715/ DL 770 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n78A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1755/ DL 1850 | n28 | UL 735/ DL 790 | n78 | 3320 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1750/ DL 1845 | n28 | UL 743/ DL 798 | n78 | 3764 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2580 | n77 | 3440 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2620 | n77 | 3400 | n3 | DL 1840 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n77 | 3900 | n41 | 2640 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n48A-n66A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/ DL 874 | n66 | UL 1760/ DL 2160 | n48 | DL 3622 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n5 | UL 845/ DL 890 | n66 | UL 1775/ DL 2175 | n77 | DL 3465 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n5 | UL 826.5/ DL 871.5 | n66 | UL 1712.5/ DL 2112.5 | n77 | DL 4192 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n5 | UL 835/ DL 880 | n66 | UL 1735/ DL 2135 | n77 | DL 3535 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 826.5/ DL 871.5 | n77 | 3795 | n66 | DL 2142 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 743/ DL 798 | n41 | 2580 | n77 | 3323 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 738/ DL 793 | n77 | 3380 | n41 | 2642 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2642 | n77 | 3440 | n28 | DL 798 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n41 | 2567.5 | n77 | 3460 | n28 | DL 782.5 | 10MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n41A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2600 | n28 | UL 725/ DL 780 | n79 | 4600 | 10MHz | 5 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 720/ DL 780 | n41 | 2600 | n79 | 4600 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n41 | 2645 | n79 | 4850 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n66A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n71 | UL 668/ DL 622 | n77 | 4108 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n77 | 4080 | n71 | DL 640 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n71A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 690/ DL 644 | n77 | 3530 | n66 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n66A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n71 | UL 668/ DL 622 | n78 | 3724 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n71A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 693/ DL 647 | n78 | 3546 | n66 | DL 2160 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference of 3DL/2UL according to Table 7.3A.0.5-2 for PC3. Note 3: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. Note 4: This step can be skipped for PC2 since no corresponding exceptions are defined for PC2 cases in Table 7.3A.0.5-2a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.3A.2_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.1_1.4.3.

7.3A.2_1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.3A.1_1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 7.3A.2_1.4.1-1on PCC and SCCs.  The SS sends downlink MAC padding bits on the DL RMC

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.1.1.4.1-1 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3A.2_1.5-1 for PC3 CA. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the throughput measurement.  Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

7.3A.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED and following exception:

For test points with “REFSENS_CA_3” UL configuration in table 7.3A.2_1.4.1-1, message exception in table 7.3A.1_1.4.3-1 applies.

7.3A.2_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 ans A.2.3 with parameters specified in Table 7.3A.2_1.5-1 for PC3.

Table 7.3A.2_1.5-1: Reference sensitivity requirement for PC3 interband 3DL CA

| CA configuration | Test ID | NR band | SCS kHz |  | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz (dBm) |  |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | 25 MHz (dBm) |  | 30 MHz (dBm) |  | 40 MHz (dBm) |
| DL_n1A-n3A-n77A_UL_n1A-n3A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+28.4+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.3+28.4+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n3A-n77A_UL_n3A-n77A | 1 | n1 | 15 |  | -100.0+31.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.0+31.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n3A-n78A_UL_n1A-n3A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+28.4+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+28.4+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  |  |  |  | -95.8+11.2+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+11.2+TT |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n3A-n78A_UL_n1A-n78A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 |  | -97.0+27.9+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+27.9+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n5A-n78A_UL_n5A-n78A | 1 | n1 | 15 |  | -100.0+18.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+18.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n5A-n78A_UL_n1A-n78A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+3.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.4+3.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n5A-n78A_UL_n1A-n5A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+15.7+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n8A-n78A_UL_n1A-n8A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 |  | -97.0 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+14.9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+14.9+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n8A-n78A_UL_n1A-n78A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 |  | -97.0 +3.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7 +3.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n28A-n78A_UL_n1A-n28A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+15.7+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n28A-n78A_UL_n1A-n78A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+4.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+4.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n28A-n78A_UL_n28A-n78A | 1 | n1 | 15 |  | -100.0+15.7+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n28A-n77A_UL_n28A-n77A | 1 | n1 | 15 |  | -100.0+15.7+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.0+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n41A-n77A_UL_n1A-n41A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  | -95.3+19.6+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -95.3+19.6+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n41A-n77A_UL_n41A-n77A | 1 | n1 | 15 |  | -100.0+9.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.0+9.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n48A_UL_n5A-n48A | 1 | n2 | 15 |  | -98.0+15.6+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+15.6+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n48A_UL_n2A-n5A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+16.6+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+16.6+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n66A_UL_n2A-n5A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+7.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+7.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0 +3.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 15 |  | -98.0+16.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+16.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n2A-n5A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+16.0 +TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+16.0 +TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n2A-n66A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+32+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+32+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n2A-n48A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+12.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+12.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n48A-n66A | 1 | n2 | 15 |  | -98.0+28.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+28.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n66A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+29.4 +TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+29.4 +TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.3+8.9 +TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+8.9 +TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+29.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+29.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  | -99.5+10.4+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+10.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 |  | -99.5+4.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+4.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 15 |  | -98.0+32.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+32.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 |  | -98.0+9.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+9.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n2 | 15 |  | -98.0+2.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+2.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n8A-n78A_UL_n3A-n8A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+16.1+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.0-2.2+16.1+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n8 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  |  |  |  | -95.8+4.5+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.0-2.2+4.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n8A-n78A_UL_n8A-n78A | 1 | n3 | 15 |  | -97.0+15.7+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.0-2.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n78A_UL_n3A-n28A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+17.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+17.3+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  |  |  |  | -95.8+4.5+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+4.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n3A-n28A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -94.8+27.4+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+27.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n41 | 15 |  | -94.8+15.9+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+15.9+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n3A-n41A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+26.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+26.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n28A-n41A | 1 | n3 | 15 |  | -97.0+26.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+26.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n3A-n28A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+15.9+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+15.9+ TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n28A-n77A | 1 | n3 | 15 |  | -97.0+17.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+17.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n3A-n77A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+15.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+15.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n41A-n77A_UL_n3A-n41A | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  | -95.3+16.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | -95.3+16.8+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n41A-n77A_UL_n41A-n77A | 1 | n3 | 15 | -97.0+16.4+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.0+16.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n41A-n77A_UL_n3A-n77A | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 | -94.8+5,3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -94.8+5.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n5A-n48A-n66A_UL_n5A-n66A | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+3.6+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+3.6+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n66A | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+16.1+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+16.1+ TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.3+8.2+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+8.2+ TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  |  |  |  | -95.3+3.3+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+3.3+ TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+13.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+13.2TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+ TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n28A-n41A-n77A_UL_n28A-n41A | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.36+28.2+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.36+28.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n28A-n41A-n77A_UL_n28A-n77A | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -98.0+29.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.0+29.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.36+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.56+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n28A-n41A-n77A_UL_n41A-n77A | 1 | n28 | 15 |  | -98.5+30.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5+30.8+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.36+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.56+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n28 | 15 |  | -98.5+3.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5+3.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n41 | 15 |  |  |  |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.36+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.56+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n28A-n41A-n79A_UL_n41A-n79A | 1 | n28 | 15 |  | -98.5+13+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+13+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  |  | -94.9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -94.9-2.7+TT |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+TT |
| DL_n28A-n41A-n79A_UL_n28A-n41A | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  |  | -94.9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -94.9-2.7+TT |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6+10.1+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+10.1+TT |
| DL_n28A-n41A-n79A_UL_n28A-n79A | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  |  | -94.9+10.4+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -94.9-2.7+10.4+TT |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+TT |
| DL_n66A-n71A-n77A_UL_n66A-n71A | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+15.9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.3+15.9+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n66A-n71A-n77A_UL_n66A-n77A | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+15.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2 +15.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.3-2.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n66A-n71A-n77A_UL_n71A-n77A | 1 | n66 | 15 |  | -99.5+15.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5+15.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.3-2.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n66A-n71A-n78A_UL_n66A-n71A | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.8+9+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n66A-n71A-n78A_UL_n71A-n78A | 1 | n66 | 15 |  | -99.5+15.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5+15.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.8-2.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 5: 4 Rx operation is targeted for FWA form factor. Note 6: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.2_1.5-1a: Reference sensitivity requirement for PC2 interband 3DL CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0 +13.6+TT |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 15 | -98.0+24.8+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+24.8+TT4 |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+34.7+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+34.7+TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 15 | -98.0+37.6+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+37.6+TT4 |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+22.2+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+22.2TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+ TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+ TT4 |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>