3GPP TSG-RAN5 Meeting #105	                            					    		R5-247730

Orlando, USA, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	5.4.20

Source:   Apple Benelux B.V.

Title:       On Common UL config for FR1 intra-band non-contiguous CA scenarios

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

The common uplink configurations defined in clause 6.1 of TS 38.521-1 [1] provides detailed uplink RB allocations which are used throughout the transmitter section of the FR1 RF test specification. An analysis of the configuration for intra-band non-contiguous CA scenarios has identified potential errors which impacts the test points. Specifically, the RB allocations are incorrectly defined and need to be corrected, or the type of RB allocation is incorrect.

# 2	Discussion

Uplink RB allocations for intra-band CA are provided in clause 6.1 of TS 38.521-1 [1] and are utilized across the transmitter RF test cases unless otherwise stated within the test.

However, several of the test configurations for CA non-contiguous RB allocations which are classified as inner allocations but appear to be outer allocations in Table 6.1A-1b of [1]. In looking back at the background for these tables, they were initially interested in [4]. Limited corrections were made in [5] and R5-231075. A complete overhaul was made in R5-235821. Some minor corrections were introduced with [6].

Observation 1: The uplink configuration and RB allocation for certain intra-band non-contiguous CA scenarios have potential errors that need correction where test parameters which are classified as inner allocations should be outer allocations.

# All the below values seem to be impacted by this issue, at least based on the analysis performed so far –

1. BW100 + BW40 SCS30 1@170 + 1@0

2. BW90 + BW60 SCS30 1@135 + 1@23

3. BW60 + BW80 SCS30 1@127 + 1@89

4. BW30 + BW90 SCS30 1@78 + 1@76

5. BW100 + BW15 SCS30 1@237 + 1@0

6. BW15 + BW100 SCS30 1@38 + 1@37

7. BW100 + BW10 SCS30 1@248 + 1@0

8. BW70 + BW40 SCS30 1@98 + 1@5

9. BW100 + BW50 SCS30 1@143 + 1@0

10. BW10 + BW100 SCS30 1@24 + 1@23

11. BW15 + BW90 SCS30 1@38 + 1@37

12. BW100 + BW60 SCS30 1@145 + 1@15

13. BW40 + BW70 SCS30 1@99 + 1@89

14. BW30 + BW80 SCS30 1@78 + 1@76

15. BW80 + BW30 SCS30 1@142 + 1@0

16. BW40 + BW100 SCS30 1@106 + 1@104

# The non-contiguous RB allocation in intra-band contiguous CA is defined in TS 38.521-1 section 6.2.A.2.

# The actual computation to determine the inner/outer RB allocations are further defined in the same clause.

A second issue seen is that the number of RBs listed is not in accordance with what is possible considering the maxRB for the specific SCS/channel bandwidth. For instance, BW10 with SCS30, the entry is 1@24, although BW10 has max NRB defined to be 24, the RB allocation cannot be more than 23 (as it starts with 0). This might be an oversight that needs to be correctly computed and corrected.

Observation 2: The number of RBs listed in the common uplink configuration for FR1 intra-band non-contiguous CA scenarios is not in accordance with what is possible considering the maxRB for the specific SCS/channel bandwidth.

A specific example can be from TS 38.521-1 [1], Table 6.1A-1b: Common uplink configuration for intra-band contiguous 2UL CA (non-contiguous RB allocation), BW_channel_ca = 110, channel bandwidth combination (10+100) MHz.

The calculation of inner/outer for contiguous CA with non-contiguous RB allocations is a complex task. Deriving some secondary parameters such as nominal channel spacing, Aggregated Channel BW or Foffset,low can be done by referring to TS 38.101-1 clause 5.3A.3.

This section uses the SCS and Guard Bands as defined by the yellow highlighted text below.

The Table 5.3.5-1 features the all the bands with the specified channels and SCS. According to the table the largest common SCS can be different. For n1 it would be 60kHz if any CBW other than 5MHz is used for the CA. In case of n28 it is 30kHz instead.

Due to the multiple test parameters impacted by the potential issues and errors identified as introduced in the test specifications, it is critical that RAN5 perform a thorough audit of the RB allocations specific to intra-band contiguous CA.

Proposal 1: Create an action point for RAN5 to re-check and analyze the RB allocation of FR1 intra-band contiguous CA (non-contiguous RB) in TS 38.521-1 Clause 6.1.

The associated change request in [7] provides some initial corrections but additional analysis is critical.

# 3	Summary

Observation 1: The uplink configuration and RB allocation for certain intra-band non-contiguous CA scenarios have potential errors that need correction where test parameters which are classified as inner allocations should be outer allocations.

Observation 2: The number of RBs listed in the common uplink configuration for FR1 intra-band non-contiguous CA scenarios is not in accordance with what is possible considering the maxRB for the specific SCS/channel bandwidth.

Proposal 1: Create an action point for RAN5 to re-check and analyze the RB allocation of FR1 intra-band contiguous CA (non-contiguous RB) in TS 38.521-1 Clause 6.1.

# 4	References

TS 38.521-1 – NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 standalone

R5-235821, RB allocations in table 6.1A-1b, Huawei, HiSilicon, 3GPP TSG-RAN5 Meeting #100, Toulouse , France, 21st – 25th Aug. 2023

R5-243127, Correction to general RB allocation in 6.1A, Huawei, HiSilicon , 3GPP TSG-RAN5 Meeting #103

R5-217487, Adding intra-band contiguous CA non-contiguous RB allocations in 6.1A, 3GPP TSG-RAN5 Meeting #93-e

R5-230234, Correction to RB allocation configuration for intra-band contiguous CA in Table 6.1A-1b, Huawei, HiSilicon, 3GPP TSG-RAN5 Meeting #98

R5-231075, “Correction to RB allocations for intra-band contiguous CA, Huawei, HiSilicon, 3GPP TSG-RAN5 Meeting #98

R5-247506, “Correction of RB allocation settings for FR1 intra-band non-contiguous CA scenarios