3GPP TSG-RAN WG5 Meeting #105	R5-247763

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3052 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to UE AMPR for UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, LTE_NR_B41_Bn41_PC29dBm-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PC1.5 UE supporting 2-layer codebook has not been covered in AMPR for UL MIMO test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AMPR for UL MIMO test cases is updated to cover PC1.5 UE supporting 2-layer codebook.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1.5 UE supporting 2-layer codebook can not be tested.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2D.3	UE additional maximum output power reduction for UL MIMO

Editor’s Note:

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U.

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 1.5, Power Class 2 and 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U to all types of NR Power Class 2 and 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.3.3 Additional Spurious Emissions for network signalling value NS_04, NS_47, NS_21, NS_56, NS_50 to all types of NR UE release 15 and forward that support UL-MIMO.

The requirements of this test apply in test case 6.5D.3_1.3 Additional Spurious Emissions for network signalling value NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed.

6.2D.3.3	Minimum conformance requirements

<< UNCHANGED PARTS SKIPPED >>

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

Table 6.2D.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0.7 | 1.0 |
| 40MHz < BW ≤ 100MHz | 1.0 | 1.0 |

Table 6.2D.3.5-1: UE Power Class 2 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 2 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 3 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.0-TT |
| 4 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 5 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 6 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 7 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 8 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 9 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.0-TT |
| 10 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 11 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 12 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 13 | 26 | 4.5 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.0-TT |
| 14 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 15 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 16 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 17 | 26 | 4.5 | 0 | 1.52 | 20 | 6 | 3 | 28+TT | 14.0-TT |
| 18 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 19 | 26 | 8 | 10 | 1.52 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 20 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 21 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 22 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| 23 | 26 | 8 | 0 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 24 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.0-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-1a: UE Power Class 2 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.52 | 19 | 5 | 3 | 28+TT | 14.-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 10 | 26 | 1 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 11 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 12 | 26 | 1 | 0 | 0 | 25 | 3 | 3 | 28+TT | 22.-TT |
| 13 | 26 | 3.5 | 6 | 1.52 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 16 | 26 | 2 | 4.5 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 17 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 18 | 26 | 2 | 0 | 0 | 24 | 3 | 3 | 28+TT | 21.-TT |
| 19 | 26 | 3.5 | 6 | 1.52 | 18.5 | 5 | 3 | 28+TT | 13.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 21 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 22 | 26 | 2.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 23 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 24 | 26 | 2.5 | 0 | 0 | 23.5 | 3 | 3 | 28+TT | 20.5-TT |
| 25 | 26 | 3.5 | 6.5 | 1.52 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 26 | 26 | 3.5 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 27 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 28 | 26 | 3 | 5 | 0 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 29 | 26 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 30 | 26 | 3 | 0 | 0 | 23 | 3 | 3 | 28+TT | 20.-TT |
| 31 | 26 | 5.5 | 8 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 5.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 33 | 26 | 5.5 | 0 | 0 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 34 | 26 | 5.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 35 | 26 | 5.5 | 0 | 1.52 | 19 | 5 | 3 | 28+TT | 14.-TT |
| 36 | 26 | 5.5 | 0 | 0 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 37 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 38 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 39 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.-TT |
| 40 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 41 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 42 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 43 | 26 | 4 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 44 | 26 | 4 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 45 | 26 | 4 | 0 | 0 | 22 | 5 | 3 | 28+TT | 17.-TT |
| 46 | 26 | 3.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 47 | 26 | 4 | 0 | 1.52 | 20.5 | 6 | 3 | 28+TT | 14.5-TT |
| 48 | 26 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 49 | 26 | 4.5 | 7.5 | 1.52 | 17 | 5 | 3 | 28+TT | 12.-TT |
| 50 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 51 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 52 | 26 | 4.5 | 6.5 | 0 | 19.5 | 5 | 3 | 28+TT | 14.5-TT |
| 53 | 26 | 4.5 | 0 | 1.52 | 20 | 6 | 3 | 28+TT | 14.-TT |
| 54 | 26 | 4.5 | 0 | 0 | 21.5 | 5 | 3 | 28+TT | 16.5-TT |
| 55 | 26 | 8 | 10 | 1.52 | 14.5 | 6 | 3 | 28+TT | 8.5-TT |
| 56 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 57 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 58 | 26 | 8 | 7.5 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| 59 | 26 | 8 | 0 | 1.52 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 60 | 26 | 8 | 0 | 0 | 18 | 5 | 3 | 28+TT | 13.-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2: UE Power Class 3 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 2 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0-TT |
| 4 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 5 | 23 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 6 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0-TT |
| 7 | 23 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 8 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0-TT |
| 10 | 23 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 11 | 23 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 12 | 23 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.0-TT |
| 13 | 23 | 3.5 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.0-TT |
| 14 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 15 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 16 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 17 | 23 | 3.5 | 0 | 1.52 | 18 | 5 | 3 | 25+TT | 13.0-TT |
| 18 | 23 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 19 | 23 | 6.5 | 8 | 1.52 | 13.5 | 6 | 3 | 25+TT | 7.5-TT |
| 20 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 21 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 22 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 23 | 23 | 6.5 | 0 | 1.52 | 15 | 6 | 3 | 25+TT | 9.0-TT |
| 24 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 for BWchannel ≤ 40 MHz; TT=1.0 for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2a: UE Power Class 3 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 3.5 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 22.5 | 5 | 3 | 28+TT | 17.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.52 | 21 | 5 | 3 | 28+TT | 16.-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 | 3 | 3 | 28+TT | 21.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.52 | 18 | 5 | 3 | 25+TT | 13.-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 5 | 3 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.52 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 | 5 | 3 | 25+TT | 17.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.52 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 22 | 5 | 3 | 25+TT | 17.-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 17 | 23 | 0 | 1 | 0 | 1.52 | 20.5 | 6 | 3 | 25+TT | 14.5-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 | 5 | 3 | 25+TT | 17.-TT |
| 19 | 23 | 0 | 2 | 4 | 1.52 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 23 | 23 | 0 | 2 | 0 | 1.52 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 | 5 | 3 | 25+TT | 16.-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.52 | 17 | 5 | 3 | 25+TT | 12.-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 6 | 3 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.52 | 19 | 5 | 3 | 25+TT | 14.-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 | 6 | 3 | 25+TT | 14.5-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.52 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.52 | 17 | 5 | 3 | 25+TT | 12.-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 39 | 23 | 0 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.-TT |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 41 | 23 | 0 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.-TT |
| 43 | 23 | 0 | 3 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 45 | 23 | 0 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.-TT |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 47 | 23 | 0 | 3 | 0 | 1.52 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 | 6 | 3 | 25+TT | 14.-TT |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.52 | 16 | 5 | 3 | 25+TT | 11.-TT |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 53 | 23 | 0 | 3.5 | 0 | 1.52 | 18 | 5 | 3 | 25+TT | 13.-TT |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
| 55 | 23 | 0 | 6.5 | 8 | 1.52 | 13.5 | 6 | 3 | 25+TT | 7.5-TT |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 59 | 23 | 0 | 6.5 | 0 | 1.52 | 15 | 6 | 3 | 25+TT | 9.-TT |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.5 dB. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2b: UE Power Class 1.5 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 6.5 | 9 | 1.5 | 18.5 | 4.0 | 3 | 31+TT | 14.5-TT |
| 2 | 29 | 6.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 3 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 4 | 29 | 4.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 5 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 6 | 29 | 4.5 | 0 | 0 | 24.5 | 2.0 | 3 | 31+TT | 21.5-TT |
| 7 | 29 | 6.5 | 9 | 1.5 | 18.5 | 4.0 | 3 | 31+TT | 14.5-TT |
| 8 | 29 | 6.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 9 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 10 | 29 | 4.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 11 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 12 | 29 | 4.5 | 0 | 0 | 24.5 | 2.0 | 3 | 31+TT | 21.5-TT |
| 13 | 29 | 6.5 | 9 | 1.5 | 18.5 | 4.0 | 3 | 31+TT | 14.5-TT |
| 14 | 29 | 6.5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 15 | 29 | 6.5 | 0 | 0 | 22.5 | 2.0 | 3 | 31+TT | 19.5-TT |
| 16 | 29 | 5 | 8 | 0 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 17 | 29 | 6.5 | 0 | 1.5 | 21 | 2.0 | 3 | 31+TT | 18-TT |
| 18 | 29 | 5 | 0 | 0 | 24 | 2.0 | 3 | 31+TT | 21-TT |
| 19 | 29 | 8.5 | 11.5 | 1.5 | 16 | 5.0 | 3 | 31+TT | 11-TT |
| 20 | 29 | 8.5 | 9 | 0 | 20 | 2.5 | 3 | 31+TT | 17-TT |
| 21 | 29 | 8.5 | 0 | 0 | 20.5 | 2.5 | 3 | 31+TT | 17.5-TT |
| 22 | 29 | 8.5 | 9 | 0 | 20 | 2.5 | 3 | 31+TT | 17-TT |
| 23 | 29 | 8.5 | 0 | 1.5 | 19 | 3.5 | 3 | 31+TT | 15.5-TT |
| 24 | 29 | 8.5 | 0 | 0 | 20.5 | 2.5 | 3 | 31+TT | 17.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-2c: UE Power Class 1.5 test requirements (NS_04) for band n41 with supporting ULFPTx

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 6 | 7 | 1.5 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 2 | 29 | 6 | 6 | 0 | 23 | 3 | 3 | 31+TT | 20-TT |
| 3 | 29 | 6 | 6 | 0 | 23 | 3 | 3 | 31+TT | 20-TT |
| 4 | 29 | 2 | 5 | 0 | 24 | 3 | 3 | 31+TT | 21-TT |
| 5 | 29 | 6 | 6 | 1.5 | 21.5 | 5 | 3 | 31+TT | 16.5-TT |
| 6 | 29 | 2 | 2 | 0 | 27 | 3 | 3 | 31+TT | 24-TT |
| 7 | 29 | 6 | 7 | 1.5 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 8 | 29 | 6 | 6 | 0 | 23 | 3 | 3 | 31+TT | 20-TT |
| 9 | 29 | 6 | 6 | 0 | 23 | 3 | 3 | 31+TT | 20-TT |
| 10 | 29 | 6 | 5 | 0 | 23 | 3 | 3 | 31+TT | 20-TT |
| 11 | 29 | 6 | 6 | 1.5 | 21.5 | 5 | 3 | 31+TT | 16.5-TT |
| 12 | 29 | 2 | 2 | 0 | 27 | 3 | 3 | 31+TT | 24-TT |
| 13 | 29 | 6.5 | 7.5 | 1.5 | 20 | 5 | 3 | 31+TT | 15-TT |
| 14 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 15 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 16 | 29 | 2.5 | 6 | 0 | 23 | 3 | 3 | 31+TT | 20-TT |
| 17 | 29 | 6.5 | 6.5 | 1.5 | 21 | 5 | 3 | 31+TT | 16-TT |
| 18 | 29 | 2.5 | 2.5 | 0 | 26.5 | 3 | 3 | 31+TT | 23.5-TT |
| 19 | 29 | 6.5 | 7.5 | 1.5 | 20 | 5 | 3 | 31+TT | 15-TT |
| 20 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 21 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 22 | 29 | 3.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 23 | 29 | 6.5 | 6.5 | 1.5 | 21 | 5 | 3 | 31+TT | 16-TT |
| 24 | 29 | 3.5 | 3.5 | 0 | 25.5 | 3 | 3 | 31+TT | 22.5-TT |
| 25 | 29 | 6.5 | 8 | 1.5 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 26 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 27 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 28 | 29 | 4 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 29 | 29 | 6.5 | 6.5 | 1.5 | 21 | 5 | 3 | 31+TT | 16-TT |
| 30 | 29 | 4 | 4 | 0 | 25 | 3 | 3 | 31+TT | 22-TT |
| 31 | 29 | 6.5 | 9.5 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 32 | 29 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 33 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 34 | 29 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 35 | 29 | 6.5 | 6.5 | 1.5 | 21 | 5 | 3 | 31+TT | 16-TT |
| 36 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 37 | 29 | 6.5 | 9 | 1.5 | 18.5 | 5 | 3 | 31+TT | 13.5-TT |
| 38 | 29 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 39 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 40 | 29 | 4.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 41 | 29 | 6.5 | 6.5 | 1.5 | 21 | 5 | 3 | 31+TT | 16-TT |
| 42 | 29 | 4.5 | 4.5 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 43 | 29 | 6.5 | 9 | 1.5 | 18.5 | 5 | 3 | 31+TT | 13.5-TT |
| 44 | 29 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 45 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 46 | 29 | 4.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 47 | 29 | 6.5 | 6.5 | 1.5 | 21 | 5 | 3 | 31+TT | 16-TT |
| 48 | 29 | 4.5 | 4.5 | 0 | 24.5 | 3 | 3 | 31+TT | 21.5-TT |
| 49 | 29 | 6.5 | 9 | 1.5 | 18.5 | 5 | 3 | 31+TT | 13.5-TT |
| 50 | 29 | 6.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 51 | 29 | 6.5 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 52 | 29 | 5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 53 | 29 | 6.5 | 6.5 | 1.5 | 21 | 5 | 3 | 31+TT | 16-TT |
| 54 | 29 | 5 | 5 | 0 | 24 | 3 | 3 | 31+TT | 21-TT |
| 55 | 29 | 8.5 | 11.5 | 1.5 | 16 | 5 | 3 | 31+TT | 11-TT |
| 56 | 29 | 8.5 | 9 | 0 | 20 | 5 | 3 | 31+TT | 15-TT |
| 57 | 29 | 8.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 58 | 29 | 8.5 | 9 | 0 | 20 | 5 | 3 | 31+TT | 15-TT |
| 59 | 29 | 8.5 | 8.5 | 1.5 | 19 | 5 | 3 | 31+TT | 14-TT |
| 60 | 29 | 8.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-3: UE Power Class 3 test requirements (NS_35) for band n71

<< END OF CHANGES >>