3GPP TSG-RAN5 Meeting #105	R5-247917

# Orlando , US, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	5.3.28.9

Source: 	Apple

Title: 	On FR2 beam correspondence during initial access test configuration

Document for:	Discussion and Endorsement

# Introduction

There have been several discussions in RAN5 meetings in response to the RAN4 LS [1] for the UE Conformance testing for FR2 beam correspondence during initial access and RRC_Inactive.

The goal is to suggest the UE RACH configuration parameters for preamble transmissions and SSB configuration for serving cell.

This paper provides Apple’s view on the configuration parameters for beam correspondence test during initial access and RRC_Inactive.

# Discussion

## On RACH Transmission Configuration IE

The intention of this topic is to propose RACH configuration parameters for beam correspondence test.

PRACH Preamble Format and Configuration Index

It is recommended to use the longest preamble format for PRACH transmission to accelerate the testing as per the RAN4 LS [1].As specified in TS 38.211 Table 6.3.3.2-4 [2], Random access configurations for FR2, the available preamble formats A1, A2, A3, B1, B4, C0, C2 with PRACH duration is within the range from 2 to 12. The longest applicable preamble format for PRACH transmission with PRACH duration 12 is Format B4[1]. The PRACH configuration Index for format B4 is from index# 112 to index# 143. Preamble Index#143 have comparatively higher number of available RACH slots per radio frame than index#112 for RACH transmission occasion.

Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum, and for FR2-NTN and paired spectrum.

Observation 1: RACH longest preamble format is recommended in RAN4 LS [1].

Observation 2: As specified in TS 38.211 Table 6.3.3.2-4  [2], PRACH Configuration Index 143 has the maximum number of RACH occasion and Index 112 has the lowest number of RACH occasion per radio frame for UE to transmit preambles.

Observation 3: The PRACH Configuration index #123 has ‘10’ RACH occasions slots for UE to send preambles, which is adequate compared to other applicable configuration indices.

Proposal 1: To configure UE for ‘B4’ PRACH preamble Format for beam correspondence test during initial access.

Proposal 2: To configure UE with PRACH configuration index# 123 for beam correspondence test during initial access.

Number of RACH Preamble Retransmission Counter

After initial UE Tx beam refinement based on the beam correspondence, UE will keep transmitting PRACH preamble till the expiry of PremableTransmax counter. This counter should be configured to maximum allowed value as specified in TS 38.331 is n200 [3], to avoid RACH failure before SS completes preamble measurements.

Observation 4: RACH failures during test can be avoided using maximum applicable value of PremableTransmax.

Proposal 3: Information element (IE) ‘preambleTransmax’ counter should be set to n200.

## On SSB Configuration for Serving Cell

The intention of this topic is to propose SSB configuration on serving cell for beam correspondence test.

Number of SSBs for NR FR2 serving cell

The common parameter settings for the serving cell are setup as specified in TS 38.508-1 Table 4.6.3-169 [4] for tests: UE maximum output power – EIRP, UE maximum output power – Spherical coverage and beam correspondence -EIRP. ssb-PositionsinBurst IE is configured for a single SSB for UE to measure and perform beam correspondence during initial access.

Observation 5:  Serving Cell is configured with single SSB beam for tests: UE maximum output power- Spherical coverage and beam correspondence - EIRP[4].

Proposal 4: To configure ssb-PositionsinBurst IE with value ‘0100 0000’ for a single SSB#1 transmission.

# Conclusion

The following observation and proposal were made in this contribution.

Observation 1: RACH longest preamble format is recommended in RAN4 LS [1].

Observation 2: As specified in TS 38.211 Table 6.3.3.2-4  [2], PRACH Configuration Index#143 has the maximum number of RACH occasion and Index#112 has the lowest number of RACH occasion per radio frame for UE to transmit preambles.

Observation 3: The PRACH Configuration index #123 has ‘10’ RACH occasions slots for UE to send preambles, which is adequate compared to other applicable configuration indices.

Observation 4: RACH failures during test can be avoided using maximum applicable value of PremableTransmax.

Observation 5:  Serving Cell is configured with single SSB beam for tests: UE maximum output power- Spherical coverage and beam correspondence - EIRP [4].

Proposal 1: To configure ‘B4’ PRACH preamble Format for beam correspondence test during initial access.

Proposal 2: To configure UE with PRACH configuration index#123 for beam correspondence test during initial access.

Proposal 3: Information element (IE) ‘preambleTransmax’ IE counter should be set to n200.

Proposal 4: To configure ssb-PositionsinBurst IE with value ‘0100 0000’ for a single SSB#1 transmission.

# References

- R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, October 9 – October 13, 2023

- TS 38.211,” Physical channels and modulation”, V18.4.0 (2024-09)

- TS 38.331, Radio Resource Control (RRC); Protocol specification, V18.3.0 (2024-09)

- TS 38.508-1, User Equipment (UE) conformance specification; Part 1: Common test environment. V18.4.0 (2024-10)