3GPP TSG-RAN5 Meeting #105	R5-247935

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1115 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the Beam Correspondence in RRC_Inactive and Initial Access test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Incomplete test details for the 6.6.3 Beam Correspondence in RRC_Inactive and Initial Access test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test configuration table to the test case 6.6.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test configuration will be missing from the test case 6.6.3 for Beam Correspondence in RRC_Inactive and Initial Access |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.6.3	Beam Correspondence in RRC_INACTIVE and initial access

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The test case is incomplete for Test procedure, Message Contents and Test Requirement.

6.6.3.1	Test purpose

The purpose of this test is to verify the UE RACH MSG1 performance and uplink spatial coverage of the UE in expected directions is acceptable.

Transmission of the wrong power increases interference or transmission errors in the uplink channel.

6.6.3.2	Test applicability

This test case applies to all types of NR UEs release 18 and forward.

6.6.3.3	Minimum conformance requirements

The minimum conformance requirements for beam correspondence in RRC_INACTIVE and initial access are same as specified in section 6.2.1.1.3.3.

For the beam correspondence requirement for UEs in initial access and in RRC_INACTIVE, the following applicability rules apply:

-	If a UE meets UE beam correspondence requirements in initial access, it is considered to have met the beam correspondence requirements in RRC_INACTIVE

The normative reference for this requirement is TS 38.101-2 [3] clause 6.6.3.

6.6.3.4	Test description

6.6.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.6.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3.4.1-1: Test Configuration Table

| Default Conditions |  |
| --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 | Normal |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 | Low range, Mid Range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 | 100 MHz |
| Test SCS as specified in Table 5.3.5-1 | 120 kHz |
| PRACH preamble format |  |
| PRACH Configuration Index | FFS |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.6.3.4.3.

6.6.3.4.2	Test procedure

FFS

6.6.3.4.3	Message contents

FFS

6.6.3.5	Test requirement

FFS

## <<< END OF CHANGES >>>