3GPP TSG-RAN5 Meeting #105	R5-246219r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1558 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SRVCC related generic test procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SRVCC Call Release procedures are all MT: step 1 is sent by the SS and step 2 expects a reply from the UE. If a UE does not support IMS over UTRAN, these step 2, and any subsequent steps will never get executed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Start a timer after step 1 and allow the test to continue if the timer expires. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.34, C35, C.36 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: modified text in EXCEPTIONs. |  |  |  |  |  |  |  |  |

# C.34	Generic test procedure for removal of early dialog of origination call after successful aSRVCC handover - EPS

Test procedure:

1)	SS sends 404 Not found to the UE

4a1)	SS waits for UE to send ACK

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | 404 Not Found | SS sends 404 Not Found |
| 2 |  |  | Void |  |
| 3 |  |  | Start Timer T1 = 5 seconds |  |
| - |  |  | EXCEPTION: Steps 4a1 to 4b1 describe optional behaviour depending on UE configuration. |  |
| 4a1 |  |  | ACK | UE sends ACK. |
| 4a2 |  |  | Stop timer T1 |  |
| 4b1 |  |  | Timer T1 expires |  |

ACK (step 4a1)

Use the default message " ACK” in annex A.2.7 with condition A4.

# C.35	Generic test procedure for removal of early dialog of incoming call after successful aSRVCC handover - EPS

Test procedure:

1)	SS sends CANCEL to the UE

6a1)	SS waits for UE to send 200 OK

6a2)	SS waits for UE to send 487 Request terminated

6a3)	SS responds with ACK to the UE

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | CANCEL | SS sends CANCEL |
| 2-4 |  |  | Void |  |
| 5 |  |  | Start Timer T1 = 5 seconds |  |
| - |  |  | EXCEPTION: Steps 6a1 to 6b1 describe optional behaviour depending on UE configuration. |  |
| 6a1 |  |  | 200 OK | UE sends 200 OK. |
| 6a2 |  |  | 487 Request Terminated | UE sends 487 Request Terminated |
| 6a3 |  |  | ACK | SS sends ACK |
| 6a4 |  |  | Stop timer T1 |  |
| 6b1 |  |  | Timer T1 expires |  |

ACK (step 6a3)

Use the default message " ACK” in annex A.2.6 with condition A4.

# C.36	Generic test procedure for removal IMS session release after SRVCC CS+PS Handover - EPS

Test procedure:

1)	SS sends BYE to UE

4a1)	SS waits for UE to send 200 OK

4b1)	SS waits for UE to send 481 Call/Transcation Does Not Exist

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | BYE | SS sends BYE |
| 2A – 2B |  |  | Void |  |
| 3 |  |  | Start Timer T1 = 5 seconds |  |
| - |  |  | EXCEPTION: Steps 4a1 to 4c1 describe optional behaviour depending on UE configuration. Based on the UE implementation, either IMS session is removed internally or will be terminated explicitly. If UE does not delete the IMS session, then UE sends 200 OK, otherwise ‘481 Call/Transcation Does Not Exist |  |
| 4a1 |  |  | 200 OK | UE sends 200 OK. |
| 4a2 |  |  | Stop timer T1 |  |
| 4b1 |  |  | SIP 481 Call/Transaction Does Not Exist | UE sends 481 Call/Transcation Does Not Exist |
| 4b2 |  |  | Stop timer T1 |  |
| 4c1 |  |  | Timer T1 expires |  |