3GPP TSG-RAN5 Meeting #105	R5-246253r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2994 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of delta RIBc and reference sensitivity for new 3CC NRCA combos within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Delta RIB,c for CA_n3A-n28A-n40A,CA_n3A-n40A-n77A and CA_n28A-n40A-n77A needs to be introduced. - Test requirements of reference sensitivity for CA_n3A-n28A-n40A,CA_n3A-n40A-n77A and CA_n28A-n40A-n77A need to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding delta RIB,c for the CA combo into Table 7.3A.0.3.2.3-1. - Adding reference sensitivity exceptions for the CA combo into Table Table 7.3A.0.5-2. - Adding reference sensitivity requirements for the CA combo into Table 7.3A.2.5-1. - Adding minimum conformance requirement of the CA combo into Table 7.3A.2_1.4.1-1 and Table 7.3A.2_1.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Delta RIB,c for the CA combo will remain incomplete. - The test configurations and requirements of reference sensitivity for the CA combo will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.3.2.3, 7.3A.0.5, 7.3A.2.5, 7.3A.2_1.4.1, 7.3A.2_1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0947 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Core specification reference: 38.101-1 V18.7.0 r1: remove changes of test requirements in Table 7.3A.2_1.5-1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

7.3A.0.3.2.3	ΔRIB,c for three bands

Table 7.3A.0.3.2.3-1: ΔRIB,c due to CA (three bands)

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1-n3-n28 | n28 | 0.2 |
| CA_n1-n3-n77 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n3-n78 | n1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n5-n78 | n1 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n8-n78 | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n28-n77 | n1 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n1-n41-n77 | n1 | 0.2 |
|  | n77 | 0.5 |
| CA_n1-n78-n79 | n78 | 0.5 |
| CA_n2-n5-n48 | n2 | 0.2 |
|  | n5 | 0..0 |
|  | n48 | 0.5 |
| CA_n2-n5-n66 | n2 | 0.3 |
|  | n5 | - |
|  | n66 | 0.3 |
| CA_n2-n5-n77 | n2 | 0.2 |
|  | n5 | 0.5 |
|  | n77 | 0.5 |
| CA_n2-n48-n66 | n2 | 0.3 |
|  | n48 | 0.5 |
|  | n66 | 0.3 |
| CA_n2-n48-n77 | n2 | 0.2 |
|  | n48 | 0.5 |
|  | n77 | 0.5 |
| CA_n2-n66-n77 | n2 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n8-n78 | n3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n28-n41 | n41 | 01/0.52 |
| CA_n3-n28-n77 | n3 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n3-n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| CA_n3-n41-n77 | n3 | 0.2 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| CA_n5-n48-n66 | n5 | 0 |
|  | n48 | 0.5 |
|  | n66 | 0.2 |
| CA_n5-n48-n77 | n5 | 0.2 |
|  | n48 | 0.5 |
|  | n77 | 0.5 |
| CA_n5-n66-n77 | n5 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n25-n66-n77 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |
| CA_n25-n66-n78 | n25 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |
| CA_n28-n40-n77 | n77 | 0.5 |
| CA_n28-n41-n77 | n28 | 0.2 |
|  | n77 | 0.5 |
| CA_n28-n41-n79 | n28 | 0.2 |
|  | n41 | 0.5 |
|  | n79 | 0.5 |
| CA_n41-n66-n71 | n41 | 0.51 |
|  |  | 12 |
|  | n66 | 0.5 |
| CA_n48-n66-n70 | n48 | 0.5 |
|  | n66 | 0.2 |
|  | n70 | 0.2 |
| CA_n48-n66-n71 | n48 | 0.2 |
|  | n70 | 0.2 |
|  | n71 | 0.2 |
| CA_n48-n66-n77 | n48 | 0.5 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |
| CA_n48-n70-n71 | n48 | 0.2 |
|  | n70 | 0.2 |
|  | n71 | 0.2 |
| CA_n66-n71-n77 | n66 | 0.2 |
|  | n71 | 0.2 |
|  | n77 | 0.5 |
| CA_n66-n71-n78 | n66 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

-------- Skip sections not changed --------

7.3A.0.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands given in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a, the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a. For these test points the reference sensitivity requirement specified in Table 7.3.2.3-1 and Table 7.3.2.3-2 are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a.

Table 7.3A.0.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) |  | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 |  | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 |  | N/A | FDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 |  | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 |  | N/A | FDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 |  | 8.0 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 |  | N/A | TDD | N/A |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 |  | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 |  | N/A | TDD | N/A |
| CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 |  | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 |  | N/A | FDD | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 |  | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 |  | 4 | FDD | IMD5 |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 |  | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 |  | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987.5 |  | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 RBSTART=10 | 3455 |  | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 1 RBSTART=0 | 3945 |  |  |  |  |
| CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 |  | 4 | FDD | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 |  | N/A | FDD | N/A |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 |  | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 |  | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 |  | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 |  | N/A | FDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 |  | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 |  | N/A | TDD | N/A |
| CA_n3-n5 | n3 | 1721 | 10 | 50 | 1816 |  | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 |  | 24 | FDD | IMD23 |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 |  | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 |  | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 |  | 8.0 | FDD | IMD44 |
|  | n77 | 3435 | 10 | 50 | 3435 |  | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 |  | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 |  | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 |  | 8.0 | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 |  | N/A | TDD | N/A |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 |  | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 |  | N/A | FDD | N/A |
| CA_n5-n776 | n5 | 844 | 5 | 25 | 889 |  | 8.3 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 |  | N/A | TDD | N/A |
|  | n5 | 829 | 5 | 25 | 874 |  | 5.5 | FDD | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 |  | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 |  | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 |  | N/A | TDD | N/A |
| CA_n14A-n77A | n14 | 793 | 5 | 20 | 763 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3935 | 10 | 50 | 3935 |  | N/A | TDD |  |
| CA_n20A-n78A | n20 | 850 | 5 | 25 | 809 |  | 11 | FDD | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 |  | N/A | TDD | N/A |
| CA_n24-n7710 | n24 | N/A | N/A | N/A | N/A |  | N/A | FDD | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A |  | N/A | TDD | N/A |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 |  | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 |  | 20 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 |  | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 |  | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 |  | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 |  | N/A | FDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 |  | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 |  | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 |  | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 |  | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 831 | 5 | 25 | 876 |  | 30 | FDD | IMD24 |
|  | n70 | 1707.5 | 5 | 25 | 2007.5 |  | N/A | FDD | N/A |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 |  | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 |  | N/A | TDD | N/A |
| CA_n41-n77 | n41 | 2545 | 60 | 1 (RBSTART=0) | 2545 | N/A |  | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART=272) | 2625 |  |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 |  | FDD | IMD9 |
| CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 |  | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 |  | 5.0 | FDD | IMD5 |
| CA_n48-n70 | n70 | 1697.5 | 25/15 | 25 | 1997.5 |  | 26 | FDD | IMD24 |
|  | n48 | 3695 | 10 | 50 | 3695 |  | N/A | TDD | N/A |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 |  | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 |  | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 |  | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 |  | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 |  | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 |  | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBSTART=0) | 3305 |  | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBSTART=8) | 3855 |  |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 |  | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 |  | N/A | TDD | N/A |
|  |  | 3875 | 10 | 1 (RBSTART=0) | 3875 |  |  |  |  |
| CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 |  | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 |  | N/A | TDD | N/A |
| CA_n70-n71 | n70 | 1697.5 | 5 | 25 | 1997.5 |  | 5 | FDD | IMD4 |
|  | n71 | 695.5 | 5 | 25 | 649.5 |  | N/A | FDD | N/A |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3309 | 10 | 50 | 3309 |  | N/A | TDD | N/A |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 |  | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 |  | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in subclause 6.2A.4 NOTE 2: RBSTART = 0, 15kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: TBD NOTE 7: TBD NOTE 8: TBD NOTE 9: TBD NOTE 10: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. NOTE 11: TBD NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

Table 7.3A.0.5-1a: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 17.8 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 32.10 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 19.10 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 18.4 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
| CA_n3-n774 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 13.6 | FDD | IMD7 |
|  | n7712 | 3427.5 | 10 | 1 (RBstart=10) | 3427.5 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 31.9 | FDD | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 18.5 | FDD | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
| CA_n5-n77 | 5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 17.6 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
| CA_n41-n77 | n41 | N/A | 10 | N/A | 2565 | 32 | TDD | IMD515 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 34.33 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+23 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 7: Void. NOTE 8: Void. NOTE 9: Void. NOTE 10: Void. NOTE 11: Void. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: Void. NOTE 14: Void. NOTE 15: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Editor’s Note:

The n78 UL RB size of 10MHz BW in CA_n1-n3-n78 is different from the Table 7.3A.5-2 in TS 38.101-1 (V17.9.0), since the values in the latter cannot be applied to DFT-s-OFDM. The closest smaller valid value has been applied in Table 7.3A.0.5-2.

Table 7.3A.0.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n28 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | FDD | IMD5 |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | FDD | IMD4 |
| CA_n1-n3-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD22 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1807.5 | 31.5 | FDD | IMD21,2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 31.0 | FDD | IMD21 |
|  | n3 | 1775 | 5 | 25 | 1870 | N/A | FDD | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3700 | 28.4 | TDD | IMD2 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3360 | 11.2 | TDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1830 | 27.9 | FDD | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
| CA_n1-n5-n78 | n1 | N/A | 5 | N/A | 2122 | 18.1 | FDD | IMD3 |
|  | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 885 | 3.1 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3610 | 15.7 | TDD | IMD3 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n8-n78 | n1 | 1945 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3745 | 14.9 | TDD | IMD3 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n8 | N/A | 5 | N/A | 940 | 3.3 | FDD | IMD5 |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
| CA_n1-n28-n77 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3416 | 15.7 | TDD | IMD32 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 790 | 4.2 | FDD | IMD5 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2150 | 15.7 | FDD | IMD3 |
| CA_n1-n28-n78 | n1 | N/A | 5 | N/A | 2150 | 15.7 | FDD | IMD3 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | N/A | 794 | 4.2 | FDD | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3416 | 15.7 | TDD | IMD3 |
| CA_n1-n41-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3330 | 19.6 | TDD | IMD31,2 |
|  | n1 | 1975 | 5 | 10 | 2165 | N/A | FDD | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n41 | N/A | 10 | N/A | 2515 | 11.5 | TDD | IMD41 |
|  | n41 | 2640 | 10 | 50 | 2640 | N/A | TDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 9.3 | FDD | IMD4 |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 15.6 | FDD | IMD31,2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n2-n5-n30 | n2 | 1870 | 5 | 25 | 1959 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 9.7 | FDD | IMD4 |
|  | n30 | 2310 | 10 | 50 | 2355 | N/A | FDD | N/A |
| CA_n2-n5-n48 | n2 | N/A | 5 | N/A | 1962 | 15.6 | FDD | IMD3 |
|  | n5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n48 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | 50 | 3593 | 16.6 | TDD | IMD3 |
| CA_n2-n5-n66 | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2140 | 7.2 | FDD | IMD4 |
| CA_n2-n5-n77 | n2 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 887.5 | 3.8 | FDD | IMD55 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987 | 16.5 | FDD | IMD35 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3540 | 16.0 | TDD | IMD31 |
| CA_n2-n12-n77 | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD32 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3315 | 10 | 50 | 3315 | 16.0 | TDD | IMD31,2 |
| CA_n2-n14-n77 | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD3 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3466 | 10 | 50 | 3466 | 16.0 | TDD | IMD31 |
| CA_n2-n30-n77 | n2 | 1906 | 5 | 25 | 1986 | 8.6 | FDD | IMD4 |
|  | n30 | 2312 | 5 | 25 | 2357 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n30 | 2309 | 5 | 25 | 2354 | 10.6 | FDD | IMD41 |
|  | n77 | 3361 | 10 | 50 | 3361 | N/A | TDD | N/A |
|  | n2 | 1870 | 5 | 25 | 1950 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | 29.4 | TDD | IMD22 |
| CA_n2-n48-n66 | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | 50 | 3625 | 32.0 | TDD | IMD2 |
|  | n66 | 1770 | 5 | 25 | 2190 | N/A | FDD | N/A |
|  | n2 | 1905 | 5 | 25 | 1985 | N/A | FDD | N/A |
|  | n48 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2155 | 12.1 | FDD | IMD4 |
|  | n2 | N/A | 5 | N/A | 1960 | 28.3 | FDD | IMD21 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n2-n66-n77 | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD25 |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3900 | 8.9 | TDD | IMD4 |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2115 | 29.2 | FDD | IMD2 |
|  | n77 | 3970 | 10 | 50 | 3970 | N/A | TDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2140 | 10.4 | FDD | IMD4 |
|  | n77 | 3500 | 10 | 50 | 3500 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2175 | 4.0 | FDD | IMD5 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 9.1 | FDD | IMD45 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1960 | 2.1 | FDD | IMD55 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
| CA_n3-n8-n78 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3550 | 16.1 | TDD | IMD3 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3370 | 4.5 | TDD | IMD5 |
|  | n3 | N/A | 5 | N/A | 1820 | 15.7 | FDD | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
| CA_n3-n28-n41 | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | TDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 26.0 | FDD | IMD24 |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |
|  | n41 | 2543 | 10 | 50 | 2543 | N/A | TDD | N/A |
|  | n3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | FDD | IMD2 |
| CA_n3-n28-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | FDD | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | TDD | IMD3 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | FDD | IMD3 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | TDD | N/A |
|  | n28 | 715 | 5 | 25 | 770 | 15.3 | FDD | IMD3 |
| CA_n3-n28-n78 | n3 | 1755 | 5 | 25 | 1850 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3320 | 17.3 | TDD | IMD3 |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3764 | 4.5 | TDD | IMD5 |
| CA_n3-n40-n77 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n40 | 2320 | 5 | 25 | 2320 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 4050 | 19.0 | TDD | IMD21 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2310 | 29.4 | TDD | IMD21 |
|  | n77 | 4030 | 10 | 50 | 4030 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1820 | 29.9 | FDD | IMD22 |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n77 | 4130 | 10 | 50 | 4130 | N/A | TDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n77 | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | TDD | N/A |
|  | n41 | N/A | 5 | N/A | 2640 | 5.3 | TDD | IMD5 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | TDD | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1840 | 16.4 | FDD | IMD3 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3440 | 16.8 | TDD | IMD31 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12-n77 | n5 | 835 | 5 | 25 | 880 | 3.9 | FDD | IMD5 |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n12 | 710 | 5 | 25 | 740 | 4.4 | FDD | IMD5 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n77 | 3905 | 10 | 50 | 3905 | 4.4 | TDD | IMD5 |
| CA_n5-n14-n77 | n5 | 835 | 5 | 25 | 880 | 3.9 | FDD | IMD5 |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 4052 | 10 | 50 | 4052 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 11.6 | FDD | IMD41 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n77 | 3298 | 10 | 50 | 3298 | 10.3 | TDD | IMD41 |
| CA_n5-n25-n66 | n5 | 834 | 5 | 25 | 879 | N/A | FDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1712 | 5 | 25 | 2132 | 7.2 | FDD | IMD4 |
| CA_n5-n25-n77 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | TDD | IMD3 |
|  | n5 | 844 | 5 | 25 | 889 | 3.8 | FDD | IMD5 |
|  | n25 | 1907 | 5 | 25 | 1987 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n25 | 1907 | 5 | 25 | 1987 | 16.5 | FDD | IMD3 |
|  | n77 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |
| CA_n5-n30-n66 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n30 | 2307.5 | 5 | 25 | 2352.5 | N/A | FDD | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 4 | FDD | IMD5 |
| CA_n5-n30-n77 | n5 | 835 | 5 | 25 | 880 | 15.2 | FDD | IMD3 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3740 | 10 | 50 | 3740 | N/A | TDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 4025 | 10 | 50 | 4025 | N/A | TDD | N/A |
|  | n5 | 840 | 5 | 25 | 885 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | 16.1 | TDD | IMD3 |
| CA_n5-n48-n66 | n5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n48 | N/A | 10 | N/A | 3622 | 3.6 | TDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
| CA_n5-n66-n77 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3465 | 16.1 | TDD | IMD3 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 4192 | 8.2 | TDD | IMD45 |
|  | n5 | 835 | 5 | 25 | 880 | N/A | FDD | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n77 | N/A | 10 | N/A | 3535 | 3.3 | TDD | IMD5 |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n66 | N/A | 5 | N/A | 2142 | 13.2 | FDD | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n12-n30-n77 | n12 | 710 | 5 | 25 | 740 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
|  | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3913 | 10 | 50 | 3913 | 16.0 | TDD | IMD3 |
| CA_n12-n66-n77 | n12 | 710 | 5 | 25 | 740 | 15.2 | FDD | IMD3 |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n12 | 707 | 5 | 25 | 737 | N/A | FDD | N/A |
|  | n66 | 1746 | 5 | 25 | 2146 | 13.2 | FDD | IMD3 |
|  | n77 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
|  | n12 | 704 | 5 | 25 | 734 | N/A | FDD | N/A |
|  | n66 | 1723 | 5 | 25 | 2123 | N/A | FDD | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | 16.0 | TDD | IMD31,2 |
| CA_n14-n30-n77 | n14 | 793 | 5 | 25 | 763 | 15.2 | FDD | IMD31 |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3857 | 10 | 50 | 3857 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | 13.2 | FDD | IMD3 |
|  | n77 | 3941 | 10 | 50 | 3941 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n77 | 3896 | 10 | 50 | 3896 | 16.0 | TDD | IMD3 |
| CA_n14-n66-n77 | n14 | 793 | 5 | 25 | 763 | 15.2 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n77 | 4188 | 10 | 50 | 4188 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1755 | 5 | 25 | 2155 | 13.2 | FDD | IMD3 |
|  | n77 | 3741 | 10 | 50 | 3741 | N/A | TDD | N/A |
|  | n14 | 793 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n66 | 1755 | 5 | 25 | 2155 | N/A | FDD | N/A |
|  | n77 | 3341 | 10 | 50 | 3341 | 16.0 | TDD | IMD31,2 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41-n66 | n25 | 1860 | 5 | 25 | 1940 | 11.0 | FDD | IMD4 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n66-n77 | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | FDD | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.4 | FDD | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | 10 | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | FDD | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 9.1 | FDD | IMD4 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | TDD | IMD2 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | TDD | IMD4 |
| CA_n25-n66-n78 | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3620 | 29.4 | TDD | IMD2 |
| CA_n25-n71-n77 | n25 | 1907.5 | 5 | 25 | 1987.5 | N/A | FDD | N/A |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n77 | 3305 | 10 | 50 | 3305 | 8.0 | TDD | IMD31,2 |
|  | n25 | 1874 | 5 | 25 | 1954 | 16.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
| CA_n28-n40-n77 | n28 | N/A | 5 | N/A | 800.5 | 11 | FDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
|  | n28 | 708 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | TDD | N/A |
|  | n77 | N/A | 10 | N/A | 3736 | 16.0 | TDD | IMD32 |
|  | n28 | 708 | 5 | 25 | 763 | N/A | FDD | N/A |
|  | n40 | N/A | 5 | N/A | 2134 | 15.7 | TDD | IMD3 |
|  | n77 | 3550 | 10 | 50 | 3550 | N/A | TDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n77 | n41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD24 |
|  | n41 | 2567.5 | 10 | 50 | 2567.5 | N/A | TDD | N/A |
|  | n77 | 3460 | 10 | 50 | 3460 | N/A | TDD | N/A |
|  | n28 | 727.5 | 5 | 25 | 782.5 | 3.0 | FDD | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | TDD | N/A |
|  | n41 | 2642 | 5 | 25 | 2642 | 29.5 | TDD | IMD2 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n77 | 3323 | 10 | 50 | 3323 | 28.2 | TDD | IMD24 |
| CA_n28-n41-n79 | n28 | N/A | 5 | N/A | 780 | 13.0 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4600 | 10.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2645 | 10.4 | TDD | IMD4 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| CA_n28-n77-n79 | n77 | 3620 | 10 | 52 | 3620 | N/A | N/A | n77 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A | n79 |
|  | n28 | 745 | 5 | 25 | 800 | 16.2 | IMD21,2 | n28 |
| CA_n30-n66-n77 | n30 | 2310 | 5 | 25 | 2355 | 29.2 | FDD | IMD21 |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4100 | 10 | 50 | 4100 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 8.7 | FDD | IMD4 |
|  | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n30 | 2310 | 5 | 25 | 2355 | N/A | FDD | N/A |
|  | n66 | 1745 | 5 | 25 | 2145 | N/A | FDD | N/A |
|  | n77 | 4055 | 10 | 50 | 4055 | 28.4 | TDD | IMD21 |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n66-n77 | n41 | 2560 | 5 | 25 | 2560 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3390 | 10 | 50 | 3390 | 16.1 | TDD | IMD31,2 |
|  | n41 | 2670 | 5 | 25 | 2670 | 5.2 | TDD | IMD5 |
|  | n66 | 1715 | 5 | 25 | 2115 | N/A | FDD | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
|  | n41 | 2530 | 5 | 25 | 2530 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 9.0 | FDD | IMD4 |
|  | n77 | 3610 | 10 | 50 | 3610 | N/A | TDD | N/A |
| CA_n41-n71-n77 | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | 29.1 | TDD | IMD21 |
|  | n41 | 2615 | 5 | 25 | 2615 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | 16.3 | TDD | IMD31 |
|  | n41 | 2580 | 5 | 25 | 2580 | N/A | TDD | N/A |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3774 | 10 | 50 | 3774 | 10.3 | TDD | IMD41 |
|  | n41 | 2615 | 5 | 25 | 2615 | 28.7 | TDD | IMD2 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3308 | 10 | 50 | 3308 | N/A | TDD | N/A |
|  | n41 | 2615 | 5 | 25 | 2615 | 15.5 | TDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 4001 | 10 | 50 | 4001 | N/A | TDD | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | TDD | N/A |
|  | n71 | 743 | 5 | 25 | 798 | 30.8 | FDD | IMD2 |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | TDD | N/A |
| CA_n48-n66-n70 | n48 | 3625 | 10 | 50 | 3625 | N/À | TDD | N/A |
|  | n66 | 1742.5 | 5 | 25 | 2142.5 | 2.8 | FDD | IMD5 |
|  | n70 | 1702.5 | 5 | 25 | 2002.5 | N/A | FDD | N/A |
| CA_n48-n66-n71 | n48 | 3552.5 | 10 | 50 | 3552.5 | N/A | TDD | N/A |
|  | n66 | 1761.5 | 5 | 25 | 2161.5 | 14.4 | FDD | IMD3 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n48 | 3695 | 10 | 50 | 3695 | 5.2 | TDD | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n48-n70-n71 | n48 | 3694 | 10 | 50 | 3694 | 9 | TDD | IMD41 |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
| CA_n66-n71-n77 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n77 | 4108 | 10 | 50 | 4108 | 15.9 | TDD | IMD31,2 |
|  | n66 | 1760 | 5 | 25 | 2160 | 15.5 | FDD | IMD32 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 686 | 5 | 25 | 640 | 15.3 | FDD | IMD3 |
|  | n77 | 4080 | 10 | 50 | 4080 | N/A | TDD | N/A |
| CA_n66-n71-n78 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3724 | 9 | TDD | IMD41 |
|  | n66 | N/A | 5 | N/A | 2160 | 15.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |

-------- Skip sections not changed --------

7.3A.2.5	Test requirement

For 3DL carrier aggregation, test parameters are specified in table 7.3A.2.4.1-1. For the CA configurations listed in table 7.3A.2.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference power level specified in table 7.3.2.5-1 for each non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2 for each non-SDL carrier for 4 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous CA with one uplink carrier and two or more downlink sub-blocks, the test requirement for SCC(s) shall be increased by ΔRIBNC given in Table 7.3A.0.2.2-1. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.5-1: Reference sensitivity requirement for 3DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
| Intra-band non-contiguous 3DL CA | CA_n48(3A) | - |
|  | CA_n66(3A) | - |
| Inter-band 3DL CA | CA_n1A-n78C |  |
|  | CA_n1A-n78(2A) | - |
|  | CA_n1A-n8A-n78A |  |
|  | CA_n1A-n28A-n78A | - |
|  | CA_n1A-n78A-n79A | - |
|  | CA_n1A-n3A-n28A | - |
|  | CA_n1A-n3A-n77A |  |
|  | CA_n1A-n3A-n78A | - |
|  | CA_n1A-n5A-n78A |  |
|  | CA_n1A-n28A-n77A |  |
|  | CA_n1A-n28A-n78A | - |
|  | CA_n1A-n41A-n77A |  |
|  | CA_n2A-n5A-n48A |  |
|  | CA_n2A-n5A-n66A |  |
|  | CA_n2A-n5A-n77A |  |
|  | CA_n2A-n48A-n66A |  |
|  | CA_n2A-n48A-n77A |  |
|  | CA_n2A-n66A-n77A |  |
|  | CA_n2A-n48(2A) |  |
|  | CA_n2A-n48B |  |
|  | CA_n2A-n77C |  |
|  | CA_n3A-n8A-n78A |  |
|  | CA_n3A-n28A-n40A |  |
|  | CA_n3A-n28A-n41A |  |
|  | CA_n3A-n28A-n77A |  |
|  | CA_n3A-n28A-n78A | - |
|  | CA_n3A-n40A-n77A |  |
|  | CA_n3A-n41A-n77A |  |
|  | CA_n3A-n77(2A) |  |
|  | CA_n5A-n48A-n66A |  |
|  | CA_n5A-n48A-n77A |  |
|  | CA_n5A-n66A-n77A |  |
|  | CA_n5A-n48(2A) |  |
|  | CA_n5A-n48B |  |
|  | CA_n5A-n77C |  |
|  | CA_n5B-n77A |  |
|  | CA_n25A-n66A-n77A | - |
|  | CA_n25A-n66A-n78A | - |
|  | CA_n26A-n66-n70A |  |
|  | CA_n26A-n66(2A) |  |
|  | CA_n28A-n40A-n77A |  |
|  | CA_n28A-n41A-n77A |  |
|  | CA_n28A-n41C |  |
|  | CA_n28A-n41A-n79A | - |
|  | CA_n28A-n77(2A) |  |
|  | CA_n41A-n66A-n71A |  |
|  | CA_n41A-n79C |  |
|  | CA_n41C-n79A |  |
|  | CA_n48A-n66(2A) |  |
|  | CA_n48A-n71(2A) |  |
|  | CA_n48A-n77C |  |
|  | CA_n48B-n66A |  |
|  | CA_n48B-n70A |  |
|  | CA_n48B-n71A |  |
|  | CA_n48B-n77A |  |
|  | CA_n48(2A)-n66A |  |
|  | CA_n48(2A)-n70A |  |
|  | CA_n48(2A)-n71A |  |
|  | CA_n48(2A)-n77A |  |
|  | CA_n48A-n66A-n70A |  |
|  | CA_n48A-n66A-n71A |  |
|  | CA_n48A-n66A-n77A |  |
|  | CA_n48A-n70A-n71A |  |
|  | CA_n66A-n70A-n71A | - |
|  | CA_n66A-n71A-n77A | - |
|  | CA_n66A-n71A-n78A | - |
|  | CA_n66A-n71(2A) |  |
|  | CA_n66(2A)-n70A | - |
|  | CA_n66(2A)-n71A | - |
|  | CA_n66B-n70A | - |
|  | CA_n66B-n71A | - |
|  | CA_n66A-n77C |  |
|  | CA_n70A-n71A(2A) |  |
| SDL configuration | CA_n29A-n66A-n70A | - |
|  | CA_n29A-n66B | - |
|  | CA_n29A-n66(2A) | - |

-------- Skip sections not changed --------

7.3A.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “NRB_PCC”and “NRB_SCC” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL allocation (Note 2) |  |  |
|  | CA configuration |  |  |  |  |  | PCC | SCC1 | SCC2 | CC Mod | PCC & SCC RB allocation |  |  | CC Mod | PCC, SCC1 or SCC2 RB allocation |  |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  |  |  | PCC | SCC1 | SCC2 |  |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n77 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1775/ DL 1870 | n77 | 3915 | n1 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n78 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n1 | UL 1950/ DL 2140 | n3 | UL 1770/ DL 1865 | n78 | 3360 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n78 | 3780 | n3 | UL 1770/ DL 1865 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n5A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/DL 874 | n78 | 3780 | n1 | DL 2122 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL1975/DL2165 | n78 | 3405 | n5 | DL 885 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n5A-n78A_UL_n1A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL1950/DL2140 | n5 | UL830/DL875 | n78 | DL 3610 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1945/ DL 2135 | n8 | UL 900/ DL 945 | n78 | DL 3745 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n8A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1940/ DL 2130 | n78 | 3380 | n8 | DL 940 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n77 | 3630 | n1 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n41A-n77A_UL_n1A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1970/ DL 2160 | n41 | 2650 | n77 | 3330 | 5MHz | 10MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2640 | n77 | 3710 | n1 | DL 2140 | 10MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n1A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n28 | UL 733/ DL 788 | n78 | 3416 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1970/ DL 2160 | n78 | 3352 | n28 | DL 794 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n28A-n78A_UL_n28A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 740/ DL 795 | n78 | 3630 | n1 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n66A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1990/ DL 1980 | n5 | UL 830/ DL 875 | n66 | DL 2140 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n5A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n48 | 3640 | n2 | DL 1962 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n5 | UL 844/ DL 1889 | n48 | DL 3593 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1907.5/ DL 1987.5 | n77 | 3305 | n5 | DL 887.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 846.5/ DL 891.5 | n77 | 3680 | n2 | DL 1987 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n5 | UL 830/ DL 875 | n77 | DL 3540 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n66 | UL 1770/ DL 2190 | n48 | DL 3625 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n48 | 3560 | n66 | DL 2155 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n48A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1735/ DL 2135 | n48 | 3695 | n2 | DL 1960 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3620 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3900 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n77 | 3970 | n66 | DL 2115 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n77 | 3500 | n66 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n2 | UL 1885/ DL 1965 | n77 | 3915 | n66 | DL 2175 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1760/ DL 2160 | n77 | 3720 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n66 | UL 1760/ DL 2160 | n77 | 3350 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n66 | UL 1760/ DL 2160 | n77 | 3620 | n2 | DL 1960 | 5Mhz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n8A-n78A_UL_n3A-n8A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL 1825 | n8 | UL 910/ DL 955 | n78 | DL 3550 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1730/ DL 1825 | n8 | UL 910/ DL 955 | n78 | DL 3370 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n8A-n78A_UL_n8A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | UL 910/ DL 955 | n78 | 3640 | n3 | DL 1820 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2518 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2687 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n3A-n40A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1730/ DL 1825 | n40 | 2320 | n77 | 4050 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n77 | 4030 | n40 | 2310 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n40A-n77A_UL_n40A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2310 | n77 | 4130 | n3 | 1820 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2510 | n28 | UL 735/ DL 790 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 710.5/ DL 765.5 | n41 | 2543 | n3 | UL 1737.5/ DL 1832.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n28 | UL 733/ DL 788 | n77 | 4173 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n77 | 3320 | n3 | UL 1755/ DL 1850 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1712.5/ DL 1807.5 | n77 | 4195 | n28 | UL 715/ DL 770 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n78A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1755/ DL 1850 | n28 | UL 735/ DL 790 | n78 | 3320 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1750/ DL 1845 | n28 | UL 743/ DL 798 | n78 | 3764 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2580 | n77 | 3440 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2620 | n77 | 3400 | n3 | DL 1840 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n41A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n77 | 3900 | n41 | 2640 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n48A-n66A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/ DL 874 | n66 | UL 1760/ DL 2160 | n48 | DL 3622 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n5 | UL 845/ DL 890 | n66 | UL 1775/ DL 2175 | n77 | DL 3465 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n5 | UL 826.5/ DL 871.5 | n66 | UL 1712.5/ DL 2112.5 | n77 | DL 4192 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n5 | UL 835/ DL 880 | n66 | UL 1735/ DL 2135 | n77 | DL 3535 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 826.5/ DL 871.5 | n77 | 3795 | n66 | DL 2142 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n28A-n40A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 708/ DL 2120 | n40 | 2310 | n77 | 3736 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 708/ DL 763 | n77 | 3550 | n40 | 2134 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n40A-n77A_UL_n40A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2302.5 | n77 | 3795 | n28 | 800.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 743/ DL 798 | n41 | 2580 | n77 | 3323 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 738/ DL 793 | n77 | 3380 | n41 | 2642 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n77A_UL_n41A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2642 | n77 | 3440 | n28 | DL 798 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n41 | 2567.5 | n77 | 3460 | n28 | DL 782.5 | 10MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n41A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2600 | n28 | UL 725/ DL 780 | n79 | 4600 | 10MHz | 5 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 720/ DL 780 | n41 | 2600 | n79 | 4600 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n41 | 2645 | n79 | 4850 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n66A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n71 | UL 668/ DL 622 | n77 | 4108 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n77 | 4080 | n71 | DL 640 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n77A_UL_n71A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 690/ DL 644 | n77 | 3530 | n66 | DL 2150 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n66A-n71A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1720/ DL 2120 | n71 | UL 668/ DL 622 | n78 | 3724 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n66A-n71A-n78A_UL_n71A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | UL 693/ DL 647 | n78 | 3546 | n66 | DL 2160 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference of 3DL/2UL according to Table 7.3A.0.5-2 for PC3. Note 3: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. Note 4: This step can be skipped for PC2 since no corresponding exceptions are defined for PC2 cases in Table 7.3A.0.5-2a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-------- Skip sections not changed --------

7.3A.2_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 ans A.2.3 with parameters specified in Table 7.3A.2_1.5-1 for PC3.

Table 7.3A.2_1.5-1: Reference sensitivity requirement for PC3 interband 3DL CA

| CA configuration | Test ID | NR band | SCS kHz |  | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 5 MHz (dBm) |  |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) |  | 25 MHz (dBm) |  | 30 MHz (dBm) |  | 40 MHz (dBm) |
| DL_n1A-n3A-n77A_UL_n1A-n3A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+28.4+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.3+28.4+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n3A-n77A_UL_n3A-n77A | 1 | n1 | 15 |  | -100.0+31.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.0+31.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n3A-n78A_UL_n1A-n3A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+28.4+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+28.4+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  |  |  |  | -95.8+11.2+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+11.2+TT |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n3A-n78A_UL_n1A-n78A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n3 | 15 |  | -97.0+27.9+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+27.9+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n5A-n78A_UL_n5A-n78A | 1 | n1 | 15 |  | -100.0+18.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+18.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n5A-n78A_UL_n1A-n78A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+3.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.4+3.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n5A-n78A_UL_n1A-n5A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+15.7+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n8A-n78A_UL_n1A-n8A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 |  | -97.0 +TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7 +TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+14.9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+14.9+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n8A-n78A_UL_n1A-n78A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 |  | -97.0 +3.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7 +3.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n28A-n78A_UL_n1A-n28A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+15.7+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n28A-n78A_UL_n1A-n78A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+4.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+4.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n28A-n78A_UL_n28A-n78A | 1 | n1 | 15 |  | -100.0+15.7+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n28A-n77A_UL_n28A-n77A | 1 | n1 | 15 |  | -100.0+15.7+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.0+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n41A-n77A_UL_n1A-n41A | 1 | n1 | 15 |  | -100.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  | -95.3+19.6+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -95.3+19.6+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| DL_n1A-n41A-n77A_UL_n41A-n77A | 1 | n1 | 15 |  | -100.0+9.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.0+9.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n48A_UL_n5A-n48A | 1 | n2 | 15 |  | -98.0+15.6+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+15.6+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n48A_UL_n2A-n5A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+16.6+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+16.6+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n66A_UL_n2A-n5A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+7.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+7.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0 +3.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 15 |  | -98.0+16.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+16.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n2A-n5A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+16.0 +TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+16.0 +TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n2A-n66A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+32+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+32+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n2A-n48A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+12.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+12.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n48A-n66A_UL_n48A-n66A | 1 | n2 | 15 |  | -98.0+28.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+28.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n66A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+29.4 +TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+29.4 +TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.3+8.9 +TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+8.9 +TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+29.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+29.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  | -99.5+10.4+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+10.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n2 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 |  | -99.5+4.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+4.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 15 |  | -98.0+32.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+32.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n2 | 15 |  | -98.0+9.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+9.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n2 | 15 |  | -98.0+2.1+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+2.1+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n8A-n78A_UL_n3A-n8A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+16.1+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.0-2.2+16.1+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n8 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  |  |  |  | -95.8+4.5+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.0-2.2+4.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n8A-n78A_UL_n8A-n78A | 1 | n3 | 15 |  | -97.0+15.7+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+15.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n8 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.0-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.0-2.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n78A_UL_n3A-n28A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+17.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+17.3+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n78 | 15 |  |  |  |  | -95.8+4.5+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -98.0+4.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n3A-n28A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -94.8+27.4+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+27.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n41 | 15 |  | -94.8+15.9+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+15.9+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n3A-n41A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+26.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+26.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n41A_UL_n28A-n41A | 1 | n3 | 15 |  | -97.0+26.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+26.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n3A-n28A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+15.9+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+15.9+ TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n28A-n77A | 1 | n3 | 15 |  | -97.0+17.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+17.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n28A-n77A_UL_n3A-n77A | 1 | n3 | 15 |  | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n28 | 15 |  | -98.5+15.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+15.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n41A-n77A_UL_n3A-n41A | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  | -95.3+16.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | -95.3+16.8+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n41A-n77A_UL_n41A-n77A | 1 | n3 | 15 | -97.0+16.4+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.0+16.4+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 | -94.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n3A-n41A-n77A_UL_n3A-n77A | 1 | n3 | 15 | -97.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -99.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 | -94.8+5,3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | -94.8+5.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n5A-n48A-n66A_UL_n5A-n66A | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n48 | 15 |  | -99.0+3.6+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -101.2+3.6+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n66A | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+16.1+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+16.1+ TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.3+8.2+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+8.2+ TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | n77 | 15 |  |  |  |  | -95.3+3.3+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+3.3+ TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n66 | 15 |  | -99.5+13.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -102.2+13.2TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+ TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+ TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n28A-n41A-n77A_UL_n28A-n41A | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.36+28.2+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.36+28.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n28A-n41A-n77A_UL_n28A-n77A | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -98.0+29.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.0+29.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.36+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.56+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n28A-n41A-n77A_UL_n41A-n77A | 1 | n28 | 15 |  | -98.5+30.8+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5+30.8+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  | -98.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.36+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.56+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n28 | 15 |  | -98.5+3.0+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5+3.0+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n41 | 15 |  |  |  |  | -94.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |  |  |  |  |  |
|  | 2 | n77 | 15 |  |  |  |  | -95.36+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -97.56+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n28A-n41A-n79A_UL_n41A-n79A | 1 | n28 | 15 |  | -98.5+13+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+13+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  |  | -94.9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -94.9-2.7+TT |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+TT |
| DL_n28A-n41A-n79A_UL_n28A-n41A | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  |  | -94.9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -94.9-2.7+TT |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6+10.1+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+10.1+TT |
| DL_n28A-n41A-n79A_UL_n28A-n79A | 1 | n28 | 15 |  | -98.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -98.5-2.75+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n41 | 15 |  |  |  |  | -94.9+10.4+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -94.9-2.7+10.4+TT |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6+TT |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | -89.6-2.2+TT |
| DL_n66A-n71A-n77A_UL_n66A-n71A | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+15.9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.3+15.9+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n66A-n71A-n77A_UL_n66A-n77A | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+15.3+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2 +15.3+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.3-2.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n66A-n71A-n77A_UL_n71A-n77A | 1 | n66 | 15 |  | -99.5+15.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5+15.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  |  |  |  | -95.3+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.3-2.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n66A-n71A-n78A_UL_n66A-n71A | 1 | n66 | 15 |  | -99.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+9+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.8+9+TT4 |  |  |  |  |  |  |  |  |  |  |
| DL_n66A-n71A-n78A_UL_n71A-n78A | 1 | n66 | 15 |  | -99.5+15.5+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -99.5+15.5+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n71 | 15 |  | -97.2+TT |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | -97.2-2.7+TT4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | n78 | 15 |  |  |  |  | -95.8+TT |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | -95.8-2.2+TT4 |  |  |  |  |  |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 5: 4 Rx operation is targeted for FWA form factor. Note 6: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.2_1.5-1a: Reference sensitivity requirement for PC2 interband 3DL CA

| CA configuration | Test ID | NR band | SCS kHz | Channel Bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) |
| DL_n2A-n5A-n77A_UL_n2A-n77A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0 +13.6+TT |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n5A-n77A_UL_n5A-n77A | 1 | n2 | 15 | -98.0+24.8+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+24.8+TT4 |  |  |  |  |  |  |
|  | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n2A-n77A | 1 | n2 | 15 | -98.0+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+TT4 |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+34.7+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+34.7+TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n2A-n66A-n77A_UL_n66A-n77A | 1 | n2 | 15 | -98.0+37.6+TT |  |  |  |  |  |  |
|  |  |  |  | -100.7+37.6+TT4 |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+TT4 |  |  |  |  |  |
| DL_n5A-n66A-n77A_UL_n5A-n77A | 1 | n5 | 15 | -98.0+TT |  |  |  |  |  |  |
|  | 1 | n66 | 15 | -99.5+22.2+TT |  |  |  |  |  |  |
|  |  |  |  | -102.2+22.2TT4 |  |  |  |  |  |  |
|  | 1 | n77 | 15 |  | -95.3+ TT |  |  |  |  |  |
|  |  |  |  |  | -97.5+ TT4 |  |  |  |  |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

-------- End of changes --------