3GPP TSG-RAN5 Meeting #105	R5-246752r2

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4746 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SL SIG test case 12.2.10.2.4 - Concurrent TBS selection SL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add NR SL SIG test case 12.2.10.2.4 - Inter-concurrent operation / MAC / SL-SCH data reception / SL-SCH Transport block size selection for SL MIMO |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR SL SIG test case 12.2.10.2.4 - Inter-concurrent operation / MAC / SL-SCH data reception / SL-SCH Transport block size selection for SL MIMO is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.10.2.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0528 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Updated based on comments received: - Test procedure and message contents are updated. 2nd revision: Change PDCU SDU in test procedure to “PDCP SDU containing a V2X packet” |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.2.10.2.4	Inter-concurrent operation / MAC / SL-SCH data reception / SL-SCH Transport block size selection for SL MIMO

12.2.10.2.4.1	Test Purpose (TP)

(1)

with { UE is configured to perform sidelink reception }

ensure that {

when { UE receives 1st stage SCI indicating a SL-SCH TB corresponding to Frequency resource assignment, Time resource assignment, Modulation and coding scheme, PSFCH overhead indication and 2 layers spatial multiplexing }

then { UE decodes the received SL-SCH TB transport block of size correctly and forwards it to higher layers }

}

12.2.10.2.4.2	Conformance requirements

Same as the conformance requirements given in subclause 12.2.10.2.3.2.

12.2.10.2.4.3	Test description

12.2.10.2.4.3.1	Pre-test conditions

Same as the pre-test conditions given in subclause 12.2.10.2.3.3.1.

12.2.10.2.4.3.2	Test procedure sequence

Same as the test procedure sequence given in subclause 12.2.10.2.3.3.2 with following exceptions:

- Table 12.2.10.2.3.3.2-2 is replaced by Table 12.2.10.2.4.3.2-1;

- Table 12.2.10.2.3.3.2-3 is replaced by Table 12.2.10.2.4.3.2-2;

Table 12.2.10.2.4.3.2-1: Number of sidelink PDCP SDUs and PDCP SDU size used as test data

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 152 ≤ TBS ≤ 12152 (note 2) | 1 | 8*FLOOR((TBS -152) / 8) |
| 12153 ≤ TBS ≤ 24224 | 2 | 8*FLOOR((TBS -224) / 16) |
| 24225 ≤ TBS ≤ 36296 | 3 | 8*FLOOR((TBS -296) / 24) |
| 36297 ≤ TBS ≤ 48368 | 4 | 8*FLOOR((TBS -368) / 32) |
| 48369 ≤ TBS ≤ 60440 | 5 | 8*FLOOR((TBS -440) / 40) |
| 60441 ≤ TBS ≤ 72512 | 6 | 8*FLOOR((TBS -512) / 48) |
| 72513 ≤ TBS ≤ 84584 | 7 | 8*FLOOR((TBS -584) / 56) |
| 84585 ≤ TBS ≤ 96656 | 8 | 8*FLOOR((TBS -656) / 64) |
| 96657 ≤ TBS ≤ 108728 | 9 | 8*FLOOR((TBS -728) / 72) |
| 108729 ≤ TBS ≤ 120800 | 10 | 8*FLOOR((TBS -800) / 80) |
| 120801 ≤ TBS ≤ 132872 | 11 | 8*FLOOR((TBS -872) / 88) |
| 132873 ≤ TBS ≤ 144944 | 12 | 8*FLOOR((TBS -944) / 96) |
| 144945 ≤ TBS ≤ 145416 | 13 | 8*FLOOR((TBS -1016) / 104) |
| 145417 ≤ TBS ≤ 169088 | 14 | 8*FLOOR((TBS -1088) / 112) |
| 169089 ≤ TBS ≤ 181160 | 15 | 8*FLOOR((TBS -1160) / 120) |
| 181161 ≤ TBS ≤ 193232 | 16 | 8*FLOOR((TBS -1232) / 128) |
| 193233 ≤ TBS ≤ 205304 | 17 | 8*FLOOR((TBS -1304) / 136) |
| 205305 ≤ TBS ≤ 217376 | 18 | 8*FLOOR((TBS -1376) / 144) |
| 217377 ≤ TBS ≤ 229448 | 19 | 8*FLOOR((TBS -1448) / 152) |
| 229449 ≤ TBS ≤ 241520 | 20 | 8*FLOOR((TBS -1520) / 160) |
| 241521 ≤ TBS ≤ 253592 | 21 | 8*FLOOR((TBS -1592) / 168) |
| 253593 ≤ TBS ≤ 265664 | 22 | 8*FLOOR((TBS -1664) / 176) |
| 265665 ≤ TBS ≤ 277736 | 23 | 8*FLOOR((TBS -1736) / 184) |
| 277737 ≤ TBS ≤ 289808 | 24 | 8*FLOOR((TBS -1808) / 192) |
| 289809 ≤ TBS ≤ 290896 | 25 | 8*FLOOR((TBS -1880) / 200) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = 8 * FLOOR((TBS - SL-SCH subheader size - RLC Status PDU size - subheader for RLC Status PDU size) / 8N) - (PDCP header size + AMD PDU header size + subheader for MAC subPDU size) (in bits) where PDCP header size is 32 bits (12-bit PDCP SN case); AMD PDU header size is 16 bits (12 bit RLC SN case); SL-SCH subheader size is 32 bits. subheader for MAC subPDU or RLC status PDU = 24 bits (consider the worst L = 16 bits case). RLC Status PDU size = 24 bits (only 1 ACK_SN case). This gives: PDCP SDU size = 8 * FLOOR((TBS - 80) / 8N) - 72 Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 152 bits. |  |  |

Table 12.2.10.2.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 7 are repeated for allowed values of Nsubchannel from 1 to NRBSL BWPnsubch in BWP and IMCS from 0 to 28. NOTE: Skip the execution of steps for which the TBS size results in coding rate exceeding 0.95. | - | - | - | - |
| 1 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of Nsubchannel and IMCS. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 7 are performed if TBS is less than or equal to UE capability "Maximum number of bits of a SL-SCH transport block received within a TTI" as specified in Table 12.2.10.2.3.3.2-1 and larger than or equal to 152 bits as specified in Table 12.2.10.2.4.3.2-1. | - | - | - | - |
| 2 | The SS triggers UE to close UE test loop mode E (Receive Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 3 | The SS creates one or more PDCP SDUs each containing a V2X packet, depending on TBS, in accordance with Table 12.2.10.2.4.3.2-1. | - | - | - | - |
| 4 | The SS transmits the PDCP SDUs concatenated into a MAC PDU in logical slot without presence of PSFCH, and sets the fields in associated SCI format 1-A as follows: - Frequency resource assignment: Nsubchannel - Modulation and coding scheme: IMCS - PSFCH overhead indication: “0” (doesn’t consider PSFCH when determining TBS) - Number of DMRS port: “1” (dual ports PSSCH transmission) | <-- | MAC PDU (N PDCP SDUs each containing a V2X packet) SCI: (SCI Format 1-A) |  |  |
| 5 | the SS requests the UE to report the counter of successful reception of packets. NOTE: Requesting the UE to report the counter of successful reception of packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | - | - |
| 6 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to N, where N is the PDCP SDUs sent in step 4? NOTE: The UE reporting the counter of successful reception of SDAP SDU/PSSCH/PSCCH packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | 1 | P |
| 7 | The SS triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| - | EXCEPTION: Steps 8 to 14 are repeated for allowed values of Nsubchannel from 1 to NRBSL BWPnsubch in BWP and IMCS from 0 to 28. NOTE: Skip the execution of steps for which the TBS size results in coding rate exceeding 0.95. | - | - | - | - |
| 8 | The SS calculates or looks up TBS in TS 38.214 [15] based on the value of Nsubchannel and IMCS. | - | - | - | - |
| - | EXCEPTION: Steps 9 to 14 are performed if TBS is less than or equal to UE capability "Maximum number of bits of a SL-SCH transport block received within a TTI" as specified in Table 12.2.10.2.3.3.2-1 and larger than or equal to 152 bits as specified in Table 12.2.10.2.4.3.2-1. | - | - | - | - |
| 9 | The SS triggers UE to close UE test loop mode E (Receive Mode). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 10 | The SS creates one or more PDCP SDUs each containing a V2X packet, depending on TBS, in accordance with Table 12.2.10.2.4.3.2-1. | - | - | - | - |
| 11 | The SS transmits the PDCP SDUs concatenated into a MAC PDU in logical slot with presence of PSFCH, and sets the fields in associated SCI format 1-A as follows: - Frequency resource assignment: Nsubchannel - Modulation and coding scheme: IMCS - PSFCH overhead indication: “1” (Consider PSFCH when determining TBS) - Number of DMRS port: “1” (dual ports PSSCH transmission) | <-- | MAC PDU (N PDCP SDUs each containing a V2X packet) SCI: (SCI Format 1-A) |  |  |
| 12 | the SS requests the UE to report the counter of successful reception of packets. NOTE: Requesting the UE to report the counter of successful reception of packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | - | - |
| 13 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to N, where N is the PDCP SDUs sent in step 11? NOTE: The UE reporting the counter of successful reception of SDAP SDU/PSSCH/PSCCH packets may be performed by MMI or AT command (+CUSPCREQ). | - | - | 1 | P |
| 14 | The SS triggers UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |

12.2.10.2.4.3.3	Specific message contents

Same as the specific message contents given in 12.2.10.2.3.3.3 except,

-	Table 12.2.10.2.3.3.2-2 is replaced by Table 12.2.10.2.4.3.2-1

<End of Change 1>