| 3GPP TSG-RAN5 Meeting #105 R5-246872 r1 # Orlando, US, 18th – 22nd November 2024 CR-Form-v12.3. CHANGE REQUEST. 37.544; CR; 0036; rev; 1; Current version:; 16.5.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; Radio Access Network; Core Network. |
| --- |
| Title: | CR to TS 37.544 on adding test method for UE over 72mm wide [OTA_WideHand] |  |  |  |  |  |  |  |  |  |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on ETSI request in LS, the missing LTE TRP TRS test method for UE over 72mm wide should be added. It was confirmed in RAN4 that LTE phantom definition and positioning guideline is same as NR, i.e., the phantom definition and positioning guideline is RAT agnostic. This formal CR is based on the endorsed content in CR R4-2415770 of RAN4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The descriptions for Wide Grip hand phantom and positioning guideline for UE over 72mm has been added into spec, by reference TS 38.161. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is not applicable to UE over 72mm wide. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.3.3.5, 4.3.4.5, A.2.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency in R4-2418220 and R4-2418226. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: update Cat B to Cat F |  |  |  |  |  |  |  |  |

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 25 101: "User Equipment (UE) radio transmission and reception (FDD)".

[3]	3GPP TR 25.914: "Measurements of radio performances for UMTS terminals in speech mode".

[4]	3GPP TS 34.108: "Common Test Environments for User Equipment (UE) Conformance Testing".

[5]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions”

[6]	3GPP TS 34.114: "User Equipment (UE) / Mobile Station (MS) Over The Air (OTA) antenna performance; Conformance testing".

[7]	3GPP TS 34.121-1: 3rd Generation Partnership Project; Technical Specification Group Terminals; Terminal conformance specification; Radio transmission and reception (FDD)

[8]	3GPP TS 34.122: "Terminal conformance specification; Radio transmission and reception (TDD)"

[9]	3GPP TS 36 101: "E-UTRA UE radio transmission and reception".

[10]	3GPP TS 36.508: "Common test environments for User Equipment (UE)".

[11]	3GPP TS 36.521-1: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: conformance testing".

[12]	3GPP TS 37.144: "User Equipment (UE) and Mobile Station (MS) GSM, UTRA, and E-UTRA over the air performance requirements".

[13]	3GPP TR 37.844: "User Equipment (UE) GSM, UTRA and E-UTRA over the air performance requirements".

[14]	3GPP TR 37.902: “Measurements of User Equipment (UE) radio performances for LTE/UMTS terminals; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology (Release 12)”

[15]	IEEE standard P1528; "Recommended Practice for Determining the Peak Spatial-Average Specific Absorption Rate (SAR) in the Human Head from Wireless Communications Devices: Experimental Techniques", April, 2003.

[16]	CENELEC Standard ENS 50361: "Basic Standard for the measurement of Specific Absorption Rate related to human exposure to electromagnetic fields from mobile phones (300 MHz-3 GHz)", Brussels, Belgium, CENELEC, July 2001, 51 p.

[17]	H. Knoess, M. Christensen, S. Svendsen, T. Hiegler, A. Friederich, "Investigation of Different Phantom Head Models Including Holder", COST 273 TD(04) 068, Gothenburg, June 2004, 6 p.

[18]	ETSI ETR 273: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement of radiated methods of measurement (using test sites) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 2: Examples and annexes".

[19]	M.Y. Kanda, M. Ballen, C.K, Chou. "Formulation and characterization of tissue simulating liquids used for SAR measurement (500-2000 MHz)" Asia-Pacific Radio Science Conference, Tokyo, Japan, August 1-4, 2001, p. 274.

[20]	V. Vigneras, "Elaboration and characterization of biological tissues equivalent liquids in the frequency range 0.9 - 3 GHz", Final report, PIOM Laboratory, University of Bordeaux, France, November 2001.

[21]	Dr. Michael D. Foegelle, ETS-Lindgren, Kevin Li, Nokia Corp., Andrew Pavacic and Paul Moller, Motorola, Inc. “Developing a Standard Hand Phantom for Wireless Testing” Part 1,Wireless design and Development

[22]	Gabriel, C., “Tissue Equivalent Material for Hand Phantoms,” Physics in Medicine and Biology, 52 (2007), pp. 4205 - 4210,

[23]	Dr. Michael D. Foegelle, ETS-Lindgren, Kevin Li, Nokia Corp., Andrew Pavacic and Paul Moller, Motorola, Inc. Wireless Design & Development, The Development of a Standard Hand Phantom for Wireless Performance Testing: Part 2

[24]	T. Laitinen, J. Ollikainen, P. Vainikainen, C. Icheln, "Rapid Spherical 3-D Field Measurement System for Mobile Terminal Antennas", COST273 TD(03)134

[25]	ETSI TR 100 028: "ElectroMagnetic Compatibility and Radio Spectrum Matters (ERM); Uncertainties in the measurement of mobile radio equipment characteristics Part 1".

[26]	ETSI TR 102 273-1-1: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement on Radiated Methods of Measurement (using test site) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 1: Introduction".

[27]	ETSI TR 102 273-1-2: "Electromagnetic compatibility and Radio spectrum Matters (ERM); Improvement on Radiated Methods of Measurement (using test site) and evaluation of the corresponding measurement uncertainties; Part 1: Uncertainties in the measurement of mobile radio equipment characteristics; Sub-part 2: Examples and annexes".

[28]	IEEE standard P1528; "Recommended Practice for Determining the Peak Spatial-Average Specific Absorption Rate (SAR) in the Human Head from Wireless Communications Devices: Experimental Techniques", April, 2003

[29]	3GPP TR 37.977: “Verification of radiated multi-antenna reception performance of User Equipment (UE)”

[30]	IEEE.149-1979.R2008: "IEEE Standard Test Procedures for Antennas," IEEE, October 2003.

[31]	Yanakiev, Boyan Radkov; Nielsen, Jesper Ødum; Christensen, Morten; Pedersen, Gert F, "Antennas In Real Environments", 2011 Proceedings of the Fifth European Conference on Antennas and Propagation (EuCAP). IEEE, 2011. p. 2766-2770.

[32]	CTIA: "Test Plan for Wireless Device Over-the-Air Performance"

[33]	TD(09) 742, COST 2100 SWG 2.2, Braunschweig, Germany, February 2009, J. Takada: "Handset MIMO Antenna Testing Using a RF-controlled Spatial Fading Emulator".

[34]	3GPP TS 36.509: “Special conformance testing functions for User Equipment (UE)”

[35]	3GPP TS 38.161:  " NR; User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios".

<<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

4.3.3.4	PDA

This procedure applies to DUTs that are from 56 to 72 mm wide. To help achieve a consistent positioning, the DUT is aligned to a PDA palm spacer. No alignment tool is required. The PDA spacer features side and bottom walls to ensure consistent alignment of DUTs of various sizes.

- Place the DUT on the PDA spacer between the fingers and align the DUT to the side wall of the PDA.

- If the DUT is shorter than 135 mm, then align the top of the DUT with the top of the PDA spacer. Otherwise, align the bottom of the DUT with the bottom wall of the PDA spacer.

- While keeping the DUT in the hand phantom in the position defined in previous steps, place the DUT and the hand phantom against the head phantom in such way that the DUT is in 6°tilt angle as described in Subclause 4.3.3.

Figure 4.3.3.4-1: Right-handed PDA hand phantom with a spacer

NOTE: Use left-handed (mirror-imaged) spacers with left-handed phantoms.

4.3.3.5	Wide Grip Hand Phantom

The DUT positioning guideline for DUTs of width >72mm and ≤92mm in the Wide Grip hand for data mode shall be the same as described in TS 38.161 Clause B.3.2.2 [35].

<<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

4.3.4.3	PDA

This procedure is suitable for use with DUTs of width 56-72 mm. The positioning of the DUT in the PDA hand for data mode is identical to that for speech mode described in Subclause 4.3.3.4.

4.3.4.5	Wide Grip Hand Phantom

The DUT positioning guideline for DUTs of width >72mm and ≤92mm in the Wide Grip hand for data mode shall be the same as described in TS 38.161 Clause B.3.1.1 [35].

<<< END OF CHANGES >>>

<<< Skip Unchanged Sections >>>

<<< START OF CHANGES >>>

A.2.2.5	Mechanical Requirements of Hand Phantoms

The hand phantoms shall be constructed of a material that is sufficiently flexible to accommodate the range of devices specified in A.2.2.4. The material shall also be made sufficiently stiff that the hand grip remains constant under rotation.

Adequate material stiffness of the hand phantom is necessary to maintain high repeatability of OTA measurements. The stiffness of the hand material shall be verified by measuring the deflection of the index finger of a moulded monoblock hand phantom under a given weight. Test procedure is following;

1. Position the hand phantom such that the index finger is horizontal.

2. Apply an indicator needle that extends horizontally 55 mm ±1 mm beyond the tip of the index finger.

3. Record the position of the indicator needle on a vertical scale.

4. Apply 20 g ±0.2 g of weight centred 6 mm ±0.5 mm from the tip of the index finger towards the hand.

5. Record the new position of the indicator needle on a vertical scale.

The deflection of the index finger of the hand phantom shall be between 2 and 5 mm. Deflection less than 2 mm per 20 g weight indicates a material that is too rigid. Deflection greater than 5 mm per 20 g weight indicates a material that is too soft.

Figure A.2.2.5-1: Hand phantom stiffness test

Figure A.2.2.5-2: A weight container designed for a hand phantom stiffness test

A weight container presented in Figure A.2.2.5-2 may be used in the stiffness test. It is assumed that other grips will have similar stiffness as the monoblock hand from the same material. Therefore, a stiffness test of the monoblock hand alone is considered sufficient.

The characteristics of position of the hand phantom such that the index finger is horizontal are specified in Table A.2.2.5-1.

Table A.2.2.5-1: Characteristics of position of the hand phantom

| ID | Hand Grip Placement | Characteristic Type |
| --- | --- | --- |
| 0 | Low | Shift Up |
| 1 | Medium | No Shift |
| 2 | High | Shift Down |

The characteristics of deflection of the index finger of the hand phantom are specified in Table A.2.2.5-2.

Table A.2.2.5-2: Characteristics of deflection of the index finger of the hand phantom

| ID | Deflection of Index Finger (df) Placement | Characteristic Type |
| --- | --- | --- |
| 0 | df ≥ 2 mm per 20g | Rigid |
| 1 | 2mm < df ≤ 5mm per 20g | Stable |
| 2 | df > 5mm per 20g | Soft |

A.2.2.6	Wide Grip Hand Phantom Definition

Wide Grip Hand phantom for DUTs with width >72mm and ≤92mm shall be the same as described in TS 38.161 Clause B.2.3 [35].

<<< END OF CHANGES >>>