3GPP TSG-RAN5 Meeting #105	R5-246970r1

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0902 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 6.3.2.1.7 2Rx FDD PMI reporting for multi-TRP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The message contents for 6.3.2.1.7 2Rx FDD PMI reporting for mTRP are not complete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding necessary message contents for mTRP, including measurement configuration and CSI report configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case can’t be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Correct the title number mistakes. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3.2.1.7	2Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA

6.3.2.1.7.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.7.2	Test applicability

This test applies to all types of NR UE release 17 and forward.

This test also applies to all types of EUTRA UE release 17 and forward supporting EN-DC.

6.3.2.1.7.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.2.3-1 and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.7.3-2.

Table 6.3.2.1.7-1: Test parameters (dual-layer)

| Parameter |  |  |  |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TRxP #1(Note 1) |  | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  |  | TRxP #1 |  |  |
| PDCCH configuration |  |  | TCI state |  |  | TCI State #1 |  |  |
|  |  |  | CORESETPoolIndex |  |  | 0 |  |  |
| CSI-RS for tracking |  |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 |  | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 |  | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  | 1 for CSI-RS resource 5,6,7,8 |
|  |  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |  |
|  |  |  | Density |  |  | 3 |  |  |
|  |  |  | CSI-RS periodicity |  | Slots | 20 |  |  |
|  |  |  | CSI-RS offset |  | Slots | 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 |  | 10 for CSI-RS resources 5 and 6 11 for CSI-RS resources 7 and 8 |
|  |  |  | QCL info |  |  | TCI state #0 |  |  |
| Duplex mode |  |  |  |  |  | FDD |  |  |
| Bandwidth |  |  |  |  | MHz | 10 |  |  |
| Subcarrier spacing |  |  |  |  | kHz | 15 |  |  |
| Active DL BWP index |  |  |  |  |  | 1 |  |  |
| Propagation channel |  |  |  |  |  | TDLA30-10 |  |  |
| Antenna configuration per TRxP |  |  |  |  |  | High XP 8 x 2 (N1,N2) = (4,1) |  |  |
| Beamforming Model |  |  |  |  |  | As specified in Annex B.4.1 (Note 4) |  |  |
| PDSCH configuration | Mapping type |  |  |  |  | Type A |  |  |
|  | k0 |  |  |  |  | 0 |  |  |
|  | Starting symbol (S) |  |  |  |  | 2 |  |  |
|  | Length (L) |  |  |  |  | 12 |  |  |
|  | PRB bundling type |  |  |  |  | Static |  |  |
|  | PRB bundling size |  |  |  |  | 2 |  |  |
|  | Resource allocation type |  |  |  |  | Type 1 |  |  |
|  | RBG size |  |  |  |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  |  |  |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  |  | N/A |  |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  |  | 1000 |  | 1002 |
|  | TCI state |  |  |  |  | TCI State #1 |  | TCI State #2 |
|  | DMRS Type |  |  |  |  | Type 1 |  |  |
|  | Number of additional DMRS |  |  |  |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  |  | 1 |  |  |
| TCI State #1 | Type 1 QCL information |  |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration |  | N/A |
|  |  |  |  | QCL Type |  | Type A |  | N/A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| TCI State #2 | Type 1 QCL information |  |  | CSI-RS resource |  | N/A |  | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  |  | QCL Type |  | N/A |  | Type A |
|  | Type 2 QCL information |  |  | CSI-RS resource |  | N/A |  | N/A |
|  |  |  |  | QCL Type |  | N/A |  | N/A |
| Resource allocation |  |  |  |  |  | Full-overlapping |  |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  |  | us | 0 |  |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  |  | Hz | 0 |  |  |
| Number of HARQ Processes |  |  |  |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  |  | 2 |  |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  |  |  | Periodic |  |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 4 |  |  |
|  |  | CDM Type |  |  |  | FD-CDM2 |  |  |
|  |  | Density (ρ) |  |  |  | 1 |  |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  |  | Row 5,(4) |  |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (9) |  |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | 5/1 |  |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource ID |  |  |  | Resource #9 | Resource #10 |  |
|  |  | CSI-RS resource Type |  |  |  | Aperiodic | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  |  |  | 8 | 8 |  |
|  |  | CDM Type |  |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |  |
|  |  | Density (ρ) |  |  |  | 1 | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  |  | Row 8, (4,6) | Row 8, (4,6) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  |  | (5) | (9) |  |
|  |  | CSI-RS periodicity and offset |  |  | slot | Not configured | Not configured |  |
|  |  | aperiodicTriggeringOffset |  |  |  | 0 | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  |  |  | Aperiodic |  |  |
|  |  | CSI-IM RE pattern |  |  |  | Pattern 0 |  |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  |  | (4,9) |  |  |
|  |  | CSI-IM timeConfig periodicity and offset |  |  | slot | Not configured |  |  |
| ReportConfigType |  |  |  |  |  | Aperiodic |  |  |
| CQI-table |  |  |  |  |  | Table 1 |  |  |
| reportQuantity |  |  |  |  |  | cri-RI-PMI-CQI |  |  |
| csi-ReportMode |  |  |  |  |  | Mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  | X=0 |  |  |
| CMR pairing and grouping |  |  |  |  |  | CMR group #1: {NZP CSI-RS resource #9}, with K1=1 CMR group #2: {NZP CSI-RS resource #10}, with K2=1 CMR paring: {NZP CSI-RS resource #9, NZP CSI-RS resource #10} |  |  |
| timeRestrictionForChannelMeasurements |  |  |  |  |  | Not configured |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  |  |  | Not configured |  |  |
| cqi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  |  |  | Wideband |  |  |
| Sub-band Size |  |  |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  |  |  | 1111111 |  |  |
| CSI-Report periodicity and offset |  |  |  |  | slot | Not configured |  |  |
| Aperiodic Report Slot Offset |  |  |  |  |  | 5 |  |  |
| CSI request |  |  |  |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |
| reportTriggerSize |  |  |  |  |  | 1 |  |  |
| CSI-AperiodicTriggerStateList |  |  |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |  |
| Codebook configuration |  | CodebookType |  |  |  | typeI-SinglePanel |  |  |
|  |  | CodebookMode |  |  |  | 1 |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  |  |  | (4,1) |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  |  |  | (4,1) |  |  |
|  |  | CodebookSubsetRestriction |  |  |  | 0x FFFF |  |  |
|  |  | RI Restriction |  |  |  | 00000001 (1 MIMO layer per TRxP) |  |  |
| Physical channel for CSI report |  |  |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  |  |  | ms | 8 |  |  |
| Maximum number of HARQ transmission |  |  |  |  |  | 4 |  |  |
| Measurement channel |  |  |  |  |  | R.PDSCH.1-6.4 |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 3: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 4: Randomization of the principle beam direction per TRxP shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |  |  |  |  |

Table 6.3.2.1.7.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.6 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.2.1.7.

6.3.2.1.7.4	Test description

6.3.2.1.7.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 6.1.2-1 and Table 6.3.2.1.7.3-1 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On for SA or (EN-DC, DC bearer MCG and SCG, Connected without Release On) for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.3.2.1.7.4.3.

6.3.2.1.7.4.2	Test procedure

1.	Set the parameters of bandwidth, the propagation condition, antenna configuration and measurement channel according to Table 6.3.2.1.7.3-1 as appropriate.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC with precoding matrix according to PMI report from the UE. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission with an UL RMC for CP-OFDM QPSK with 5 RBs allocated according to A.2.2.6 of TS 38.521-1 [21] to carry the PUSCH CSI feedback via PDCCH DCI format [0_1] with aperiodic CSI request triggered. No transport block is sent in parallel to the CQI feedback. Establish and  according to Annex G.3.2.

3.	Set SNR to. The SS shall transmit PDSCH with randomly selected precoding matrix from codebook (Table 5.2.2.2.1-6 in TS 38.214 [12]) every slot regardless of PMI reports from the UE. Note that each precoding matrix shall be selected in equal probabilities. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format [0_1] with aperiodic CSI request triggered. Measure according to Annex G.3.3.

4.	Calculate. If the ratio  which is specified in table 6.3.2.1.7.5-1, then the test is pass. Otherwise, the test is fail.

6.3.2.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.

6.3.2.1.7.4.3.1	Message exceptions for SA

Table 6.3.2.1.7.4.3.1-1: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | n=10 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for TRS (5) | entry 1 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | NZP-CSI-RS-Resource for TRS (6) | entry 2 CSI-RS resource 6 |  |
| NZP-CSI-RS-Resource[7] | NZP-CSI-RS-Resource for TRS (7) | entry 3 CSI-RS resource 7 |  |
| NZP-CSI-RS-Resource[8] | NZP-CSI-RS-Resource for TRS (8) | entry 4 CSI-RS resource 8 |  |
| NZP-CSI-RS-Resource[9] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 9 |  |
| NZP-CSI-RS-Resource[10] | NZP-CSI-RS-Resource for CSI Acquisition | entry 6 CSI-RS resource 10 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | n entries | n=2 |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1, Table 6.3.2.1.7.4.3.1-2 |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2, Table 6.3.2.1.7.4.3.1-6 |  |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { |  | 1 entry |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | Table 6.3.2.1.7.4.3.1-14 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-2: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 0 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 8 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-ResourceId[3] | 2 | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-ResourceId[4] | 3 | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-ResourceId[1] | 4 | entry 1 CSI-RS resource 5 |  |
| NZP-CSI-RS-ResourceId[2] | 5 | entry 2 CSI-RS resource 6 |  |
| NZP-CSI-RS-ResourceId[3] | 6 | entry 3 CSI-RS resource 7 |  |
| NZP-CSI-RS-ResourceId[4] | 7 | entry 4 CSI-RS resource 8 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | true |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-3: NZP-CSI-RS-Resource for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 0 | CSI-RS resource 1 |  |
|  | 1 | CSI-RS resource 2 |  |
|  | 2 | CSI-RS resource 3 |  |
|  | 3 | CSI-RS resource 4 |  |
|  | 4 | CSI-RS resource 5 |  |
|  | 5 | CSI-RS resource 6 |  |
|  | 6 | CSI-RS resource 7 |  |
|  | 7 | CSI-RS resource 8 |  |
| resourceMapping | CSI-RS-ResourceMapping for TRS | Table 6.3.2.1.7.4.3.1-4 |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for TRS | Table 6.3.2.1.7.4.3.1-5 |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId 0 |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-4: CSI-RS-ResourceMapping for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0000 | k0=0 for CSI-RS resource 1,2,3,4 |  |
|  | 0001 | k0=1 for CSI-RS resource 5,6,7,8 |  |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 6 | l0 = 6 for CSI-RS resource 1,3,5,7 |  |
|  | 10 | l0 = 10 for CSI-RS resource 2,4,6,8 |  |
| nrofPorts | p1 | 1 for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
| cdm-Type | noCDM |  |  |
| density CHOICE{ |  |  |  |
| three | Null |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-5: CSI-ResourcePeriodicityAndOffset for TRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots20 | 10 | Periodicity 20 slots and offset 10 for CSI-RS resource 1,2,3,4 |  |
| slots20 | 11 | Periodicity 20 slots and offset 11 for CSI-RS resource 5,6,7,8 |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-6: NZP-CSI-RS-ResourceSet for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 1 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 8 | entry 1 CSI-RS resource 9 |  |
| NZP-CSI-RS-ResourceId[2] | 9 | entry 2 CSI-RS resource 10 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | 0 |  |  |
| trs_Info | Not present |  |  |
| cmrGroupingAndPairing-r17 | CMRGroupingAndPairing-r17 | Table 6.3.2.1.7.4.3.1-7 |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-7: CMRGroupingAndPairing-r17

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CMRGroupingAndPairing-r17 ::= SEQUENCE { |  |  |  |
| nrofResourcesGroup1-r17 | 1 |  |  |
| pair1OfNZP-CSI-RS-r17 ::= SEQUENCE { |  | 2 entries |  |
| nzp-CSI-RS-ResourceId1-r17 | 1 |  |  |
| nzp-CSI-RS-ResourceId2-r17 | 1 |  |  |
| } |  |  |  |
| pair2OfNZP-CSI-RS-r17 | Not present |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-8: NZP-CSI-RS-Resource for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 8 | CSI-RS resource 9 |  |
|  | 9 | CSI-RS resource 10 |  |
| resourceMapping | CSI-RS-ResourceMapping for CSI Acquistion | Table 6.3.2.1.7.4.3.1-9 |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | Not present |  |  |
| qcl-InfoPeriodicCSI-RS | Not present |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-9: CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001100 | k0 = 4, k1 = 6, row8 |  |
| } |  |  |  |
| nrofPorts | p8 |  |  |
| firstOFDMSymbolInTimeDomain | 5 | l0 = 5 for CSI-RS resource 9 |  |
|  | 9 | l0 = 9 for CSI-RS resource 10 |  |
| cdm-Type | cdm4-FD2-TD2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-10: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 3 entries |  |  |
| TCI-State[1] SEQUENCE { |  | entry 1 |  |
| tci-StateId | TCI-StateId 0 |  |  |
| qcl-type1 SEQUENCE { | QCL Type is Type1 |  |  |
| Cell | Not present |  |  |
| Bwp-id | Not present | BWP ID |  |
| referenceSignal | ssb: 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[2] SEQUENCE { |  | entry 2 |  |
| tci-StateId | TCI-StateId 1 |  |  |
| qcl-type1 { | QCL Type is Type1 |  |  |
| Cell | Not present |  |  |
| Bwp-id | 0 | BWP ID |  |
| referenceSignal | csi-rs: 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| } |  |  |  |
| TCI-State[3] SEQUENCE { |  | entry 3 |  |
| tci-StateId | TCI-StateId 2 |  |  |
| qcl-type1 { | QCL Type is Type1 |  |  |
| Cell | Not present |  |  |
| Bwp-id | 0 | BWP ID |  |
| referenceSignal | csi-rs: 5 | CSI-RS # 5 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| ZP-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| ZP-CSI-RS-Resource[1] | ZP-CSI-RS-Resource | entry 1, Table 6.3.2.1.7.4.3.1-11 |  |
| } |  |  |  |
| p-ZP-CSI-RS-ResourceSet | p-ZP-CSI-RS-ResourceSet | Table 5.4.2.5-5A in TS 38.508-1 [6] |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-11: ZP-CSI-RS-Resource

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-204 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| zp-CSI-RS-ResourceId | 0 |  |  |
| resourceMapping | CSI-RS-ResourceMapping for ZP CSI-RS | Table 6.3.2.1.7.4.3.1-12 |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for ZP CSI-RS | Table 6.3.2.1.7.4.3.1-13 |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-12: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 5.4.2, Table 5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | k0 = 4, row5 |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-13: CSI-ResourcePeriodicityAndOffset for ZP CSI-RS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-14: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| resourcesForChannelMeasurement | 1 | CSI-ResourceConfigId for CSI Acquisition |  |
| csi-IM-ResourcesForInterference | 2 | CSI-ResourceConfigId for CSI-IM |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE { | 2 entries |  |  |
| reportSlotOffsetList[1] | 5 | entry 1 |  |
| reportSlotOffsetList[2] | 5 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| codebookConfig-r17 | CodebookConfig-r17 | Table 6.3.2.1.7.4.3.1-15 |  |
| csi-ReportMode-r17 | mode1 |  |  |
| numberOfSingleTRP-CSI-Mode1-r17 | n0 |  |  |
| codebookConfig-v1730 ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 ::= SEQUENCE { |  |  |  |
| codebookMode | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-15: CodebookConfig-r17

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig-r17 ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 ::= SEQUENCE { |  |  |  |
| typeI-SinglePanel-Group1-r17 SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo ::= SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| four-one-TypeI-SinglePanel-Restriction1-r17 | 0x FFFF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-Group2-r17 SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo ::= SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| four-one-TypeI-SinglePanel-Restriction2-r17 | 0x FFFF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-RestrictionSTRP-r17 | Not present |  |  |
| typeI-SinglePanel-ri-RestrictionSDM-r17 | ‘0001’B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.7.4.3.1-16: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 4.3.6.1.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | K1 = 2 | “010” |  |
| Antenna port(s) | DMRS port 0 and 2 | “1011” |  |
| Transmission configuration indication | Value 0 which is mapped to the first TCI codepoint with TCI state ID0,1 = TCI state ID#1 and TCI state ID0,2 = TCI state ID#2 as defined in TS 38.321 clause 6.1.3.24. | “000” |  |

Table 6.3.2.1.7.4.3.1-17: ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| tci-PresentInDCI | enabled |  |  |
| } |  |  |  |

6.3.2.1.7.4.3.2	Message exceptions for NSA

Same as in clause 6.3.2.1.7.4.3.1.

6.3.2.1.7.5	Test requirement

Table 6.3.2.1.7.5-1: Test requirement

| Parameter | Test 1 |
| --- | --- |
|  | 1.59 |

<End of Changes>