3GPP TSG-RAN5 Meeting #105	R5-247029r1

# Orlando, United States, Nov 18-22, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0080 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating the test description in NTN performance test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NTN performance test case 8.2.1.2.2.1.1_1 is not correctly aligned with the RAN4 requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating test procedure and SIB19 in test case 8.2.1.2.2.1.1_1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 8.2.1.2.2.1.1_1 is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.2.1.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related with discussion paper R5-247028 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: correct 3GU issue in the coversheet |  |  |  |  |  |  |  |  |

### <Start of Changes>

##### 8.2.1.2.2	2RX requirements

###### 8.2.1.2.2.1	FDD

### <Unchanged Skipped>

8.2.1.2.2.1.1_1	2Rx FDD FR1 PDSCH Mapping Type A for Satellite Access

8.2.1.2.2.1.1_1.1	Test Purpose

Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models and MCS for NTN capable UE receiving signal from earth based gNB via a satellite access node.

8.2.1.2.2.1.1_1.2	Test Applicability

This test applies to all types of NTN UE release 17 and forward supporting satellite access.

8.2.1.2.2.1.1_1.3	Test Description

8.2.1.2.2.1.1_1.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [2].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [12] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [12] clause 5.2.2.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.2.1.1.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to clauses C.0, C.1, C.2 and uplink signals according to clauses G.0, G.1, G.2, G.3.1 of TS 38.521-1 [2].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 8.2.1.2.2.1.1_1.3.3.

8.2.1.2.2.1.1_1.3.2	Test procedure

1.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through  any preconfigured means.

2	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508[12] table 5.6.2.1-2 for NGSO (LEO-600) if UE supports only NGSO satellites or both GSO and NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in clause 8.2.1.2.2.1.1_1.3.3.

3. Deactivate UE prediction of satellite trajectory by any preconfigured means.

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables Table 8.2.1.2.2.1.1-3. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 8.2.1.2.2.1.1-2 and Table 8.2.1.2.2.1.1-3 as appropriate.

6.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to [Annex G].

4.	Repeat steps from 1 to 3 for each subtest in Table 8.2.1.2.2.1.1-3 as appropriate.

8.2.1.2.2.1.1_1.3.3	Message contents

Message contents are according to TS 38.508-1 [12] clauses 4.6.1 and 5.4.2 with following exceptions:

SIB19 contents as TS 38.508-1 [12] clause 5.6.2.1 with exceptions in Table 8.2.1.2.2.1.1_1.3.3-4:

Table 8.2.1.2.2.1.1_1.3.3-1: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | Pos1 |  |  |
| } |  |  |  |

Table 8.2.1.2.2.1.1_1.3.3-2: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n16 | Test 1-1, Test 1-2 |  |
|  | n32 | Test 1-3 |  |
|  |  | Test 1-4 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes with re-Tx disable for all HARQ for Test 1-4 in which 4 disabled processes are randomly select at test configuration |  |
| } |  |  |  |

Table 8.2.1.2.2.1.1_1.3.3-3: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots20 | 10 (for CSI-RS resources 1 and 2) 11 (for CSI-RS resources 3 and 4) |  |  |
| } |  |  |  |

Table 8.2.1.2.2.1.1_1.3.3-4: SIB19

| Derivation Path: TS 38.508-1 [6], Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17::= SEQUENCE { |  |  |  |
| cellSpecificKoffset-r17 | 8 |  |  |
| } |  |  |  |
| } |  |  |  |

8.2.1.2.2.1.1_1.3.4	Test requirement

Table 8.2.1.2.2.1.1-2 and Table 8.2.1.2.2.1.1-3 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1.1 for each throughput test shall meet or exceed the specified value in Table 8.2.1.2.2.1.1_1.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 8.2.1.2.2.1.1_1.3.4-1: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.2 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 8.5 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 0.5 |
| 1-4 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70* | 2 |

### <End of Changes>