3GPP TSG-RAN5 Meeting #105 	R5-247177r1

# Orlando, USA, 18th Nov. 2024 - 22nd Nov. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3104 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test configuration of 7.3A.1_1 for Rel-17 CA configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Table 7.3A.1_1.4.1-1, test settings for the following CA configurations are different from the minimum requirements. - CA_n3A-n5A - CA_n30A-n77A |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 7.3A.1_1.4.1-1, test settings for the following CA configurations were corrected according to the minimum requirements. - CA_n3A-n5A - CA_n30A-n77A Editorial corrections to Table 7.3A.1_1.4.1-1, Table 7.3A.1_1.5-1, Table 7.3A.1_1.5-1a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CA configurations will not be tested correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Editorial corrections to CA_n25A-n78A and CA_n26A-n70A were removed from Table 7.3A.1_1.5-1 to solve overlap with R5-247369. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.1_1	Reference sensitivity power level for 2DL CA exceptions

Editor’s Note: The following aspects are either missing or not yet determined:

- Test point analysis for CA_n3A-n5A IMD2 and IMD4 is currently missing in TR 38.905.

- Test points related to CBW=5MHz for Band n41 are not completed yet.

7.3A.1_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.1_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2DL CA

7.3A.1_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.1_1.4	Test description

7.3A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  |  |  | Refer to “PCC”and “SCC” columns |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n3A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 1771 MHz (UL) | n5 | 838 MHz | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for CA_n30A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n30 |  | 2310 MHz (UL) |  | n66 |  | 2197.5 MHz |  | 10 MHz |  | 15 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_4 |  |  |  |
| Test Settings for CA_n30A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | n77 |  | 3532.5 MHz |  | n30 |  | Mid |  | 10 MHz |  | 5 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  |  |  | REFSENS_CA_2 |  |
| 2 |  | n77 |  | 3487.5 MHz |  | n30 |  | Mid |  | 10 MHz |  | 10 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  |  |  | REFSENS_CA_2 |  |
| 3 |  | n30 |  | Mid |  | n77 |  | 3487.5 MHz |  | 5 MHz |  | 10 MHz |  | CP-OFDM QPSK |  | Full RB |  |  | DFT-s-OFDM QPSK |  | REFSENS_CA_3 |  | REFSENS_CA_3 |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier indicating power class 2 in TS 38.508-2 table A.4.3.2A.4.1-3. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band CA with PC3 single UL carrier or PC3 2UL CA

| CA configuration | Test ID | NR band | CBW | REFSENS test requirement of NR band (dBm) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | 2Rx | 4Rx |
| CA_nXA-nYA |  |  |  | REFSENS+MSD+TT | REFSENS+MSD8+ΔRIB,4R +TT |
| <<Unchanged rows skipped>> |  |  |  |  |  |
| CA_n71A-n77A | 1 | n71 | 5 | -97.2 +5.5 +TT | -97.2+5.5 +TT |
|  |  | n77 | 10 | -95.3 +TT | -95.3 -2.2 +TT |
| CA_n71A-n78A | 1 | n71 | 5 | -97.2 +TT | -97.2-2.76 +TT -97.2-2.47 + TT |
|  |  | n78 | 10 | -95.8 +10.4 +TT | -95.8 +10.4 +TT |
|  | 2 | n71 | 5 | -97.2 +5.5 +TT | -97.2 +5.5 +TT |
|  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n78A-n79A | 1 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 40 | -89.6 +2+TT | -89.6 +2+TT |
|  | 2 | n78 | 10 | -96.1 +2.6+TT | -96.1+2.6+TT |
|  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 3 | n78 | 100 | -85.5+2.6+TT | -85.5+2.6+TT |
|  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. Note 5: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. Note 6: When 4 Rx operation is supported by FWA form factor. Note 7: When 4Rx operation is supported by handheld UE. Note 8: For operations with 4 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.2.3-2 when MSD > 0. |  |  |  |  |  |

Table 7.3A.1_1.5-1a: Reference sensitivity requirement for inter band CA with PC2 single UL carrier or PC2 2UL CA

| CA configuration | Test ID | NR band | CBW | REFSENS test requirement of NR band (dBm) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | (MHz) | 2Rx | 4Rx |
| CA_nXA-nYA |  |  |  | REFSENS+MSD+TT | REFSENS+MSD7+ΔRIB,4R +TT |
| CA_n1A-n78A | 1 | n1 | 5 | -100 +17.8+TT | -100 +17.8+TT |
|  |  | n78 | 10 | -95.8 +TT | -95.8 -2.2 +TT |
| CA_n2A-n77A | 4 | n2 | 5 | -98+9.1+TT | -98+9.1+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 5 | n2 | 5 | -91.8+6.7+TT | -91.8+6.7+TT |
|  |  | n774 | 20 | -92.2+TT | -92.2-2.2+TT |
|  | 6 | n2 | 5 | -98.0+32.1+TT | -98.0 +32.1+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 7 | n2 | 5 | -98.0 +19.1+TT | -98.0 +19.1+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 9 | n2 | 5 | -98.0 +1.0 +TT | -98.0+1.0 +TT |
|  |  | n774 | 100 | -85.1 +TT | -85.1-2.2 +TT |
| CA_n3A-n41A | 1 | n3 | 5 | -97.0+18.4+TT | -97.0 +18.4+TT |
|  |  | n41 | 10 | -94.9 +TT | -94.9 -2.7 +TT |
|  | 2 | n3 | 5 | -97.0+2.3+TT | -97.0+2.3+TT |
|  |  | n41 | 100 | -84.6+TT | -84.6 -2.7+TT |
| CA_n3A-n77A | 4 | n3 | 5 | -97.0+31.9+TT | -97.0+31.9+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 5 | n3 | 5 | -97.0+18.5+TT | -97.0+18.5+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 6 | n3 | 5 | -97.0+8.1+TT | -97.0+8.1+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 7 | n3 | 40 | -82.3+1.0+TT | -82.3+1.0+TT |
|  |  | n774 | 40 | -89.2+TT | -89.2-2.2+TT |
| CA_n3A-n78A | 6 | n3 | 5 | -97.0 + 8.1 +TT | -97.0 +8.1 +TT |
|  |  | n78 | 5 | -97.0 +TT | -97.0 -2.7 +TT |
|  | 7 | n3 | 40 | -89.4 + 1.0 +TT | -89.4 + 1.0+TT |
|  |  | n78 | 40 | -89.4 +TT | -89.4 -2.7 +TT |
| CA_n5A-n77A | 5 | n5 | 20 | -90.8 +4.3+TT | -90.8+4.3+TT |
|  |  | n77 | 20 | -92.2+TT | -94.4+TT |
|  | 6 | n5 | 10 | -98.0 +8.1+TT | -98.0+8.1+TT |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
|  | 7 | n5 | 5 | -98.0 +18.6+TT | -98.0+18.6+TT |
|  |  | n77 | 10 | -95.3+TT | -97.5+TT |
| CA_n14A_n77A | 4 | n14 | 5 | -97.0 + 34 +TT | -97.0 + 34 +TT |
|  |  | n77 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 5 | n14 | 10 | -93.8 + 31 +TT | -93.8 + 31 +TT |
|  |  | n77 | 10 | -95.3+TT | -95.3-2.2+TT |
| CA_n30A-n77A | 1 | n30 | 5 | -99 + 13.2 + TT | -99 + 13.2 + TT |
|  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
|  | 2 | n30 | 10 | -95.8 + 10.6 + TT | -95.8 + 10.6 + TT |
|  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
|  | 3 | n30 | 5 | -99 + 17.6 + TT | -99 + 17.6 + TT |
|  |  | n77 | 10 | -95.3 + TT | -95.3 -2.2 + TT |
| CA_n41A-n77A | 1 | n41 | 10 | -94.8+13.2+TT | -94.8+13.2+TT |
|  |  | n774 | 20 | -92.4+TT | -92.4-2.2+TT |
|  | 2 | n41 | 100 | -84.6+8.8+TT | -84.6+8.8+TT |
|  |  | n774 | 20 | -92.4+TT | -92.4-2.2+TT |
|  | 5 | n41 | 100 | -84.6+TT | -84.6-2.7+TT |
|  |  | n774 | 10 | -95.3+10.5+TT | -95.3+10.5+TT |
|  | 6 | n41 | 10 | -94.8+6.5+TT | -94.8+6.5+TT |
|  |  | n774 | 100 | -85.1+TT | -85.1-2.2+TT |
|  | 7 | n41 | 100 | -84.6+6.5+TT | -84.6+6.5+TT |
|  |  | n774 | 100 | -85.1+TT | -85.1-2.2+TT |
|  | 8 | n41 | 10 | -94.8+32+TT | -94.8+32+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
|  | 9 | n41 | 10 | -94.8+32+TT | -94.8+32+TT |
|  |  | n774 | 10 | -95.3+TT | -95.3-2.2+TT |
| CA_n41A-n79A | 1 | n41 | 100 | -84.6+TT | -84.6 -2.7 +TT |
|  |  | n79 | 10 | -98.9+3.1+TT | -98.9+3.1+TT |
|  | 2 | n41 | 10 | -94.8+3.5+TT | -94.8+3.5+TT |
|  |  | n79 | 100 | -85.6+TT | -85.6 -2.2+TT |
| CA_n66A-n77A | 4 | n66 | 5 | -99.5 +34.33 +TT | -99.5 +34.33 +TT |
|  |  | n77 | 10 | -95.3 +TT | -95.3-2.2+TT |
|  | 5 | n66 | 5 | -99.5 +11.27 +TT | -99.5+11.27 +TT |
|  |  | n77 | 10 | -95.3 +TT | -95.3-2.2+TT |
| CA_n78A-n79A | 1 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 40 | -89.6 +5+TT | -89.6+5+TT |
|  | 2 | n78 | 10 | -96.1+5.6+TT | -96.1+5.6+TT |
|  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 3 | n78 | 100 | -85.5+5.6+TT | -85.5+5.6+TT |
|  |  | n79 | 100 | -85.5+TT | -85.5-2.2+TT |
|  | 4 | n78 | 100 | -85.5+TT | -85.5-2.2+TT |
|  |  | n79 | 100 | -85.5+5+TT | -85.5+5+TT |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: TT for each frequency and channel bandwidth is specified in Table 7.3.2.5-3. Note 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. Note 5: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. Note 6: When 4 Rx operation is supported by FWA form factor. Note 7: For operations with 4 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.2.3-2 when MSD > 0. |  |  |  |  |  |

<<End of change>>