Title: 	Discussion on test point selection for Beam Correspondence in RRC_INACTIVE and initial access for FR2 SA

Source: 	Apple

# Introduction

This document provides considerations of test points selection of 6.6.3 Beam Correspondence in RRC_INACTIVE and initial access in SA FR2 [1].

# Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

As a reference, below is the test configurations used for 6.6.1 FR2 SA Beam correspondence – EIRP [2].

Table 6.6.1.4.1-1: Test Configuration Table for PC3

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full |
| 2 | 100 |  |  |  |  |  |
| 3 | 200 |  |  |  |  |  |
| 4 | 400 |  |  |  |  |  |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

As a reference, below is the test configurations used for 6.2.1.2 UE maximum output power - Spherical coverage [2].

Table 6.2.1.2.4.1-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid Range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120 kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | - | Modulation |  | RB allocation (NOTE 1) |
| 1 | 50 |  |  | DFT-s-OFDM QPSK |  | Inner_Full for PC2, PC3, |
| 2 | 100 |  |  |  |  | PC4, PC5 and PC7 |
| 3 | 200 |  |  |  |  | Inner_Full_Region1 for |
| 4 | 400 |  |  |  |  | PC1 |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 , PC4, PC5 and PC7 or Table 6.1-2 for PC1. |  |  |  |  |  |  |

## Test Environment

For FR2 SA the Beam Correspondence – EIRP is tested with Normal Temperature Condition (NTC). This test environment can also be leverage for 6.6.3 Beam Correspondence in RRC_INACTIVE and initial access.

Proposal 1: Define Test Environment as ‘Normal’ for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing.

## Test Frequencies

As specified in 38.521-2, the UE maximum output power – spherical coverage test defined for UE spherical coverage for power class 3 [2] selected test frequencies low range, mid-range, and high range for testing. The same approach can be adopted for 6.6.3 Beam Correspondence in RRC_INACTIVE and initial access.

Proposal 2: Define test frequencies as Low range, Mid-range and High range for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing.

## Test Subcarrier Spacing

For FR2, 6.6.1 Beam correspondence – EIRP test 120 kHz SCS is selected. The 120 kHz SCS can also be applied for 6.6.3 Beam Correspondence in RRC_INACTIVE and initial access.

Proposal 3: Select 120 kHz SCS supported for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing.

## Test Channel bandwidth

Channel BW for 6.6.1 Beam correspondence – EIRP test is Lowest and Highest. It is proposed to test with 100 MHz for 6.6.3 Beam Correspondence in RRC_INACTIVE and initial access.100 MHz is the commonly deployed channel bandwidth in FR2 networks.

Proposal 4: Select 100 MHz test channel bandwidth for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing.

## PRACH configuration

In NR, the default PRACH configuration index is defined in 38.508-1 [3] is as follows. PRACH configuration index number specific to the beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing is FFS.

Proposal 5: Select 1 PRACH configuration index for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing. PRACH configuration Index number is FFS for this test.

## Number of Test Points

Based on above proposal, the total test points are tabulated below:

Table 2.9-1 Total test points of Beam Correspondence

| Environment conditions |  |  |  | Test Frequencies |  | Test ChBW | Test SCS |  | Number of PRACH Configuration Index |  | Total Test Points |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | 3 |  | 1 | 1 |  | 1 |  | 3 |

# Conclusion

In conclusion, the following selection of test points have been proposed for beam correspondence in RRC_INACTIVE and initial access in NR FR2.

Proposal 1: Define Test Environment as ‘Normal’ for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing.

Proposal 2: Define test frequencies as Low range, Mid-range and High range for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing.

Proposal 3: Select 120 kHz SCS supported for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing.

Proposal 4: Select 100 MHz test channel bandwidth for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing.

Proposal 5: Select 1 PRACH configuration index for beam correspondence in RRC_INACTIVE and initial access for NR FR2 SA testing. PRACH configuration Index number is FFS for this test.

# References

[1]	R5-245998, "Updates to the Beam Correspondence in RRC_Inactive and Initial Access test", Apple.

[2]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[3]	3GPP TS 38.508-1: “User Equipment (UE) conformance specification; Part 1: Common test environment”