3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210002

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1457 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN-DC FR1 : Addition of NR MAC test case 7.1.1.4.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR MAC test case 7.1.1.4.2.5 to the 5G/ENDC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR MAC test case 7.1.1.4.2.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.5.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

3	Branches executed	7

4	Execution Log Files	7

4.1	Qualcomm SM8350	7

4.2	MediaTek MT6297	7

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.1.4.2.5 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.1.1.4.2.5.ENDC

ATS Version:	iwd-TTCN3-B2019-06_D20wk38

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM8350 and MediaTek MT6297

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody |
| --- | --- |
| Reason for change | When MCS = 27 and PRB < 4, the resulting TBS is very small leading to CRC error in decoding UL data |
| Summary of change | Skip when MCS is 27 and PRB is smaller than 4 inside the loop |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | The change requires prose change. Accepted in principle conditional to prose CR approval. Implemented rightly as if (v_I_MCS == 28v_Max_Imcs and v_N_PRB < 4) |

Before Change:

function f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_Max_Imcs := 28, //@sic R5s201144sic@ boolean p_TransformPrecoder := false) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS:=0; // Modulation and Coding Scehme Index var integer v_N_PRB:=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA ; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var SubFrameTiming_Type v_Timing; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var IntegerList_Type v_NoOfSymbols := {4,14}; var integer v_Max_Imcs :=p_Max_Imcs;//@sic R5s201144sic@ // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_Now, resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(0, 4), cs_NR_TransportBlockRetransmissionList_Def(v_Max_Imcs), -, -, cs_NR_DciFormat_1_0_Params(v_SCS)); /* @sic R5s190442: v_SCS sic@ */ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB if (p_TransformPrecoder) { if (f_NR_Check_This_N_PRB(v_N_PRB) == false) { // nPRB check in EXCEPTION Step continue; // nprb not allowed } } for (v_I_MCS := 0; v_I_MCS <= v_Max_Imcs ; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 //@sic R5s190827r1 sic@ for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 //@sic R5s190827 sic@ /* if(v_TDRA == 0) { v_NoOfSymbols := 4; } else{ v_NoOfSymbols := 14 } */ //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols[v_TDRA], v_N_PRB, v_I_MCS); //@sic R5-198884 sic@ if (v_I_MCS == 28 and v_N_PRB < 4) { //For = 28 and < 4, the resulting TBS is very small leading to CRC errors in decoding UL data continue; } // skip this nprb, Imcs, TDRA combination. if (v_TBSize < 136) { //this results in zero number of PDCP SDUs continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_NR(v_Timing), v_NR_PDCP_SDUList)); //@siclog "Step 4" siclog@ v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 60); f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), resourceAllocationType1); //@sic R5s190895 sic@ //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_2_1

After Change:

function f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_Max_Imcs := 28, //@sic R5s201144sic@ boolean p_TransformPrecoder := false) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS:=0; // Modulation and Coding Scehme Index var integer v_N_PRB:=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA ; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var SubFrameTiming_Type v_Timing; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var IntegerList_Type v_NoOfSymbols := {4,14}; var integer v_Max_Imcs :=p_Max_Imcs;//@sic R5s201144sic@ // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_Now, resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(0, 4), cs_NR_TransportBlockRetransmissionList_Def(v_Max_Imcs), -, -, cs_NR_DciFormat_1_0_Params(v_SCS)); /* @sic R5s190442: v_SCS sic@ */ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB if (p_TransformPrecoder) { if (f_NR_Check_This_N_PRB(v_N_PRB) == false) { // nPRB check in EXCEPTION Step continue; // nprb not allowed } } for (v_I_MCS := 0; v_I_MCS <= v_Max_Imcs ; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 //@sic R5s190827r1 sic@ for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 //@sic R5s190827 sic@ /* if(v_TDRA == 0) { v_NoOfSymbols := 4; } else{ v_NoOfSymbols := 14 } */ //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols[v_TDRA], v_N_PRB, v_I_MCS); //@sic R5-198884 sic@ if (v_I_MCS == v_Max_nPRB and v_N_PRB < 4) { //For = 28 and < 4, the resulting TBS is very small leading to CRC errors in decoding UL data continue; } // skip this nprb, Imcs, TDRA combination. if (v_TBSize < 136) { //this results in zero number of PDCP SDUs continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_NR(v_Timing), v_NR_PDCP_SDUList)); //@siclog "Step 4" siclog@ v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 60); f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), resourceAllocationType1); //@sic R5s190895 sic@ //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_2_1

…

# Branches executed

The test case was executed on EN-DC band combinations DC_3A_n41 and DC_3A_n78

# Execution Log Files

## Qualcomm SM8350

The Qualcomm SM8350 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_4_2_5_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_4_2_5_PIXIT.html

## MediaTek MT6297

The MediaTek MT6297 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_4_2_5_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_4_2_5_PIXIT.html

# References

| [1] | R5s210003 : EN-DC FR1 : Supporting information for addition of NR MAC test case 7.1.1.4.2.5 |
| --- | --- |