3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210009

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0659 | rev | - | Current version: | 15.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall over IMS test case 21.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS eCall test case 21.1 to the IMS/EUTRA ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS eCall test case 21.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 36.508 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	5

2.5	Change 5	7

2.6	Change 6	8

3	Branches Executed	10

4	Execution Log Files	10

4.1	Qualcomm SA415M	10

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 21.1 which is part of the IMS EUTRA test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	21.1

ATS Version:	iwd-TTCN3-B2019-06_D20wk38

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SA415M

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | TC_21_1 |
| --- | --- |
| Reason for change | IMS2 PTC is not started |
| Summary of change | Create IMS2 PTC and start test case |
| TTCN module | IMS_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before change

testcase TC_21_1() runs on MTC_IMS system SYSTEM_IMS {

// @purpose

//   eCall over IMS / Manual initiation / Normal registration / Emergency registration / Success / 200 OK with ACK

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(300);

v_IMS1 := IMS_PTC.create alive;

f_MTC_IMS_CreateMapAndConnectPTCs_EUTRA(system, v_IMS1, v_IMS2);

v_IMS1.start(f_TC_21_1_IMS1());

t_GuardTimer.start;

f_MTC_MainLoop_IMS(t_GuardTimer);

}

After change

testcase TC_21_1() runs on MTC_IMS system SYSTEM_IMS {

// @purpose

//   eCall over IMS / Manual initiation / Normal registration / Emergency registration / Success / 200 OK with ACK

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(300);

v_IMS1 := IMS_PTC.create alive;

v_IMS2 := IMS_PTC.create alive;

f_MTC_IMS_CreateMapAndConnectPTCs_EUTRA(system, v_IMS1, v_IMS2);

v_IMS1.start(f_TC_21_1_IMS1());

v_IMS2.start(f_TC_21_1_IMS2());

t_GuardTimer.start;

f_MTC_MainLoop_IMS(t_GuardTimer);

}

## Change 2

| Function name | f_GetDefaultValueNetworkFeatureSupport |
| --- | --- |
| Reason for change | IMS eCall scenario requires indication of emergency bearer services over S1 support |
| Summary of change | Enable msVoPS and emcBS for Rel-14 test cases Note : An associated prose CR on 36.508 will be raised at RAN5#90 |
| TTCN module | EPS_NAS_Templates.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval. |

Before change

function f_GetDefaultValueNetworkFeatureSupport() return template (value) EPS_NetworkFeatureSupport

{ // @sic R5s180648 sic@

var template (value) EPS_NetworkFeatureSupport v_NtwkFeatSupport := cs_EPS_NwkFtSup_IMSVoice;

// @sic R5s130681, R5-173000 sic@

select (f_GetTestcaseAttrib_Eutra_Release(testcasename())) { // @sic R5s130508 sic@

case ("", "rel_10") { // rel8 or rel-10 - 36.508 just says rel-9

}

case ("rel_9") {

v_NtwkFeatSupport.imsVoPS := '1'B; // @sic R5s130902 sic@

v_NtwkFeatSupport.emcBS := '1'B; // @sic R5s130977 sic@

}

case else {}

}

if (f_GetTestcaseAttrib_CIOT_CP(testcasename()) or f_GetTestcaseAttrib_CIOT_UP(testcasename())) { // @sic R5-173000 sic@

v_NtwkFeatSupport := cs_EPS_NwkFtSup_CIOT;

}

return v_NtwkFeatSupport;

}

After change

function f_GetDefaultValueNetworkFeatureSupport() return template (value) EPS_NetworkFeatureSupport

{ // @sic R5s180648 sic@

var template (value) EPS_NetworkFeatureSupport v_NtwkFeatSupport := cs_EPS_NwkFtSup_IMSVoice;

// @sic R5s130681, R5-173000 sic@

select (f_GetTestcaseAttrib_Eutra_Release(testcasename())) { // @sic R5s130508 sic@

case ("", "rel_10") { // rel8 or rel-10 - 36.508 just says rel-9

}

case ("rel_9") {

v_NtwkFeatSupport.imsVoPS := '1'B; // @sic R5s130902 sic@

v_NtwkFeatSupport.emcBS := '1'B; // @sic R5s130977 sic@

}

case ("rel_14") {

v_NtwkFeatSupport.imsVoPS := '1'B;

v_NtwkFeatSupport.emcBS := '1'B;

}

case else {}

}

if (f_GetTestcaseAttrib_CIOT_CP(testcasename()) or f_GetTestcaseAttrib_CIOT_UP(testcasename())) { // @sic R5-173000 sic@

v_NtwkFeatSupport := cs_EPS_NwkFtSup_CIOT;

}

return v_NtwkFeatSupport;

}

## Change 3

| Function name | New constant tsc_MimeType_MultipartyMixed1 |
| --- | --- |
| Reason for change | Adding new string constant for multipart/mixed media type |
| Summary of change | Added new string |
| TTCN module | IMS_CommonDefs.ttcn |
| MCC160 Comment | Rejected: The proposed constant is not a valid MIME type; "boundary" is parameter of the Content-Type header. In addition the value of the boundary parameter shall not be hard-coded as the UE may use any value (in accordance to 34.229-1 A.2.1) |

New constant

module IMS_CommonDefs {

import from CommonDefs all;

import from Parameters all;

const charstring tsc_MimeType_MultipartyMixed := "multipart/mixed";                                 /* @status    APPROVED (IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R12, LTE_IRAT) */

const charstring tsc_MimeType_MultipartyMixed1 := "multipart/mixed; boundary=boundary1";

const charstring tsc_MimeType_Pidf := "application/pidf+xml";                                       /* @status    APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT, POS) */

const charstring tsc_MimeType_XcapDiff := "application/xcap-diff+xml";                              /* @status    APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC, NR5GC_IRAT, POS) */

type enumerated IMS_SDP_AudioCodec_Type { AMR_8000, AMR_16000, EVS_16000, DTMF_8000, DTMF_16000 };  /* @status    APPROVED (IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R12, LTE_IRAT)

@sic R5-165942: DTMF sic@ @sic R5-172930: DTMF_8000, DTMF_16000 sic@ */

type record of IMS_SDP_AudioCodec_Type IMS_SDP_AudioCodecList_Type;

## Change 4

| Function name | f_IMS_InviteRequest_MessageHeaderRX |
| --- | --- |
| Reason for change | Changing the media type to multipart/mixed |
| Summary of change | Changed the media type |
| TTCN module | IMS_Procedures_CallControl.ttcn |
| MCC160 Comment | Rejected: proposed change breaks mostly all IMS call control test cases; other changes are needed as shown in MCC160 implementation for change 5. |

Before change

function f_IMS_InviteRequest_MessageHeaderRX(template (present) SipUrl p_ExpectedCalleeUri,

OptionTag_List p_AdditionalOptionTagsForSupported := tsc_OptionTagList_Empty,

INVITE_A_2_1_Context_Type p_Context := A_2_1_A4,

boolean p_A3 := pc_MultimediaTelephonyService,    /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */

boolean p_A8 := false,

boolean p_A11 := false)

runs on IMS_PTC return template (present) MessageHeader

{ ………………………………

else {                                       /* normal case: no re-invite */

// route body acc. to conditions in A.2.1

v_Pcscf := ?;                              /* @sic R5s140934 change 6: P-CSCF address is checked later on (f_IMS_A_2_1_Invite_CommonChecks -> f_IMS_MessageHeader_Request_CheckRoute) sic@ */

v_RouteBodyList[0] := cr_RouteBody(*, cr_SipUri_HostPort_lr(v_Pcscf, v_Port_ps_Route));  /* @sic R5s140873: v_Pcscf;  R5s140934 change 6 sic@

@sic R5-155174: "loose routing" parameter mandatory for any kind of call (i.e. emergency call too, in contrast to R5s150817) sic@ */

if (not match(p_Context, (A_2_1_A6, A_2_1_A7, A_2_1_A19))) {  // => no emergency call

v_RouteBodyList[1] := cr_RouteBody(*, cr_SipUri_HostPort_lr(tsc_IMS_Scscf));

}

if (p_A8) {

v_MessageHeader_Invite.geolocation := cr_GeolocationAny;

v_MessageHeader_Invite.geolocationRouting := cr_GeolocationRoutingSupported; // @sic R5-134798 sic@

v_MediaType := tsc_MimeType_MultipartyMixed;

}

v_CallId := cr_CallId(?);

v_From := cr_From(v_DisplayName, v_SipUrlFrom);

v_To := cr_ToDef(p_ExpectedCalleeUri);

}

………………………………………

}

After change

function f_IMS_InviteRequest_MessageHeaderRX(template (present) SipUrl p_ExpectedCalleeUri,

OptionTag_List p_AdditionalOptionTagsForSupported := tsc_OptionTagList_Empty,

INVITE_A_2_1_Context_Type p_Context := A_2_1_A4,

boolean p_A3 := pc_MultimediaTelephonyService,    /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */

boolean p_A8 := false,

boolean p_A11 := false)

runs on IMS_PTC return template (present) MessageHeader

{ …………………………………………

else {                                       /* normal case: no re-invite */

// route body acc. to conditions in A.2.1

v_Pcscf := ?;                              /* @sic R5s140934 change 6: P-CSCF address is checked later on (f_IMS_A_2_1_Invite_CommonChecks -> f_IMS_MessageHeader_Request_CheckRoute) sic@ */

v_RouteBodyList[0] := cr_RouteBody(*, cr_SipUri_HostPort_lr(v_Pcscf, v_Port_ps_Route));  /* @sic R5s140873: v_Pcscf;  R5s140934 change 6 sic@

@sic R5-155174: "loose routing" parameter mandatory for any kind of call (i.e. emergency call too, in contrast to R5s150817) sic@ */

if (not match(p_Context, (A_2_1_A6, A_2_1_A7, A_2_1_A19))) {  // => no emergency call

v_RouteBodyList[1] := cr_RouteBody(*, cr_SipUri_HostPort_lr(tsc_IMS_Scscf));

}

if (p_A8) {

v_MessageHeader_Invite.geolocation := cr_GeolocationAny;

v_MessageHeader_Invite.geolocationRouting := cr_GeolocationRoutingSupported; // @sic R5-134798 sic@

v_MediaType := tsc_MimeType_MultipartyMixed;

}

v_CallId := cr_CallId(?);

v_From := cr_From(v_DisplayName, v_SipUrlFrom);

v_To := cr_ToDef(p_ExpectedCalleeUri);

}

v_MediaType := tsc_MimeType_MultipartyMixed1;

…………………………………

}

## Change 5

| Function name | f_IMS_INVITE_ReceiveRequest_A_2_1_Optional |
| --- | --- |
| Reason for change | Currently TTCN is checking for SDP body at top level |
| Summary of change | Pass flag to check for SDP body as MIME part |
| TTCN module | IMS_Procedures_CallControl.ttcn |
| MCC160 Comment | Implemented differently (see below) NOTE: the proposed change would break mostly all IMS call control test cases Further changes are needed to fulfil requirements according to 34.229-1 A.2.1: - Call-Info header added (seems to be missing in the log too) - Accept header added - Recv-Info header added - Content-Type header modified to multipart/mixed (with any boundary) (see change 4) - According to 34.229-1 A.2.1 the MSD MIME body's Content-Disposition shall have additional parameter "handling=optional" |

Before change

function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context,

template (present) SipUrl p_ExpectedCalleeUri,

template (present) MessageHeader p_MessageHeader_Invite,

template (omit) float p_WaitDuration := omit,

boolean p_A3 := pc_MultimediaTelephonyService,    /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */

boolean p_A8 := false,

boolean p_A11 := false,

boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert,

boolean p_OriginatingIdentificationRestriction := false)

runs on IMS_PTC return template (omit) IMS_DATA_REQ

{ /* p_OtherCalleeUri .. if present to be used as CalleeUri; if omit default CalleeUri is used acc. to PIXITs and context */

/* @sic R5-133276, R5-133301 Cleanup of functions for MO Call Setup sic@ */

/* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */

/* @sic R5-140799: additional parameter p_MessageHeader to deal with Reason header sic@ */

/* @sic R5s140370 change 7: pc_IMS_SRVCCAlert default value for p_A12, p_A12 renamed to p_SrvccAlertingSupported  sic@ */

/* @sic R5s150180: p_A10 removed sic@ */

/* @sic R5s150177 change 9, R5s150204: p_OtherCalleeUri changed to "template SipUrl p_OtherCalleeUri"; NOTE: p_OtherCalleeUri shall either omit or "template (present)" (but not e.g. "*") sic@ */

/* @sic R5s150846: p_A8 := false per default sic@ */

/* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */

/* @sic R5s170305: p_A9 := pc_IMS_SessionIdSupported per default sic@ */

/* @sic R5-174544: SessionId removed sic@ */

/* @sic R5s190882: p_AdditionalOptionTagsForSupported removed sic@ */

var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ;

var template (present) MessageBody v_MessageBody := f_SIP_MessageBodySDP_RX(p_A8);  /* @sic R5w140112 sic@ */

var boolean v_IsReInvite := (p_Context == A_2_1_A5);

var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6);

v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration);    // @sic R5s190882 sic@

if (isvalue(v_IMS_DATA_REQ)) {

f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A8, -, p_A11, p_SrvccAlertingSupported, p_OriginatingIdentificationRestriction);  /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */

}

return v_IMS_DATA_REQ;

}

After change

function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context,

template (present) SipUrl p_ExpectedCalleeUri,

template (present) MessageHeader p_MessageHeader_Invite,

template (omit) float p_WaitDuration := omit,

boolean p_A3 := pc_MultimediaTelephonyService,    /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */

boolean p_A8 := false,

boolean p_A11 := false,

boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert,

boolean p_OriginatingIdentificationRestriction := false)

runs on IMS_PTC return template (omit) IMS_DATA_REQ

{ /* p_OtherCalleeUri .. if present to be used as CalleeUri; if omit default CalleeUri is used acc. to PIXITs and context */

/* @sic R5-133276, R5-133301 Cleanup of functions for MO Call Setup sic@ */

/* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */

/* @sic R5-140799: additional parameter p_MessageHeader to deal with Reason header sic@ */

/* @sic R5s140370 change 7: pc_IMS_SRVCCAlert default value for p_A12, p_A12 renamed to p_SrvccAlertingSupported  sic@ */

/* @sic R5s150180: p_A10 removed sic@ */

/* @sic R5s150177 change 9, R5s150204: p_OtherCalleeUri changed to "template SipUrl p_OtherCalleeUri"; NOTE: p_OtherCalleeUri shall either omit or "template (present)" (but not e.g. "*") sic@ */

/* @sic R5s150846: p_A8 := false per default sic@ */

/* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */

/* @sic R5s170305: p_A9 := pc_IMS_SessionIdSupported per default sic@ */

/* @sic R5-174544: SessionId removed sic@ */

/* @sic R5s190882: p_AdditionalOptionTagsForSupported removed sic@ */

var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ;

var template (present) MessageBody v_MessageBody := f_SIP_MessageBodySDP_RX(true);  /* @sic R5w140112 sic@ */

var boolean v_IsReInvite := (p_Context == A_2_1_A5);

var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6);

v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration);    // @sic R5s190882 sic@

if (isvalue(v_IMS_DATA_REQ)) {

f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A8, -, p_A11, p_SrvccAlertingSupported, p_OriginatingIdentificationRestriction);  /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */

}

return v_IMS_DATA_REQ;

}

## MCC160 Implementation

function f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(INVITE_A_2_1_Context_Type p_Context, template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (omit) float p_WaitDuration := omit, boolean p_A3 := pc_MultimediaTelephonyService, boolean p_A8 := false, boolean p_A11 := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert, boolean p_OriginatingIdentificationRestriction := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var boolean v_UseMIME := (ispresent(p_MessageHeader_Invite.contentType) and match(tsc_MimeType_MultipartyMixed, p_MessageHeader_Invite.contentType.mTypeSubtype)); var template (present) MessageBody v_MessageBody := f_SIP_MessageBodySDP_RX(v_UseMIME); /* @sic R5w140112 sic@ */ var boolean v_IsReInvite := (p_Context == A_2_1_A5); var boolean v_IsEmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_Optional(p_ExpectedCalleeUri, p_MessageHeader_Invite, v_MessageBody, p_WaitDuration, v_IsReInvite, v_IsEmergencyCallWithoutRegistration); // @sic R5s190882 sic@ if (isvalue(v_IMS_DATA_REQ)) { f_IMS_A_2_1_Invite_CommonChecks(valueof(v_IMS_DATA_REQ), p_Context, p_A3, p_A8, -, p_A11, p_SrvccAlertingSupported, p_OriginatingIdentificationRestriction); /* @sic R5-160975: p_OriginatingIdentificationRestriction sic@ */ } return v_IMS_DATA_REQ; }

function f_ECall_InitialInvite_AnnexC47(ECall_Type p_ECallType) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var IMS_DATA_REQ v_IMS_DATA_REQ; var MIME_Message v_MIME_Message; var MIME_Part v_MIME_Part; var SDP_Message v_SdpOffer; var template (present) SDP_Message v_SDP_Message_Step1; var template (present) SipUrl v_ECallPSAP_Rx := fl_ECallPSAP_Rx(p_ECallType); var template (present) MessageHeader v_MessageHeader; var template (present) AcceptBody_List v_AcceptBodyList := { cr_AcceptBody(tsc_MimeType_EmergencyCallDataControlXML), * }; // condition A20, A21 var template (present) SemicolonParam_List v_DispositionParams := { cr_GenericParam("handling", "optional"), * }; // condition A20, A21 var boolean v_A8 := pc_IMS_Geolocation; var INVITE_A_2_1_Context_Type v_Context := A_2_1_A7; var charstring v_CidUrl; // Step 1. Receive INVITE for an eCall v_MessageHeader := f_IMS_InviteRequest_MessageHeaderRX(v_ECallPSAP_Rx, -, v_Context, -, v_A8); v_MessageHeader.callInfo := cr_CallInfo(-, "emergencyCallData.eCall.MSD"); // condition A20, A21 v_MessageHeader.recvInfo := cr_RecvInfo_Def("EmergencyCallData.eCall.MSD"); // condition A20, A21 v_MessageHeader.accept := cr_Accept(v_AcceptBodyList); // condition A20, A21 v_MessageHeader.contentType := cr_ContentTypeMIME; // condition A20, A21 v_IMS_DATA_REQ := valueof(f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(v_Context, v_ECallPSAP_Rx, v_MessageHeader, -, -, v_A8)); v_MIME_Message := v_IMS_DATA_REQ.Request.Invite.messageBody.mimeMessageBody; // Check PIDF if (v_A8) { v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, tsc_MimeType_Pidf); f_IMS_DecodeAndCheckGeolocation(v_MIME_Part.body.xmlBody); } // Check SDP v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX(); // Build RX SDP template using codecs AMR and AMR_WB v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(v_MIME_Message, c_sdpApplication); v_SdpOffer := f_IMS_SIP_DecodeMatchAndCheckSDP(v_MIME_Part.body.sdpMessageBody, v_SDP_Message_Step1); // Check eCall MSD v_CidUrl := v_IMS_DATA_REQ.Request.Invite.msgHeader.callInfo.callInfoBody[0].url; f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_MIME_Message, tsc_MimeType_EmergencyCallDataMSD, v_CidUrl, v_DispositionParams); return f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ, v_SdpOffer); }

function f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(MIME_Message p_MIME_Message, charstring p_ContentType, charstring p_ContentId, template SemicolonParam_List p_DispositionParams := *, template EmergencyCallData_Control p_EmergencyCallDataXML := omit) runs on IMS_PTC { var MIME_Part v_MIME_Part := f_IMS_MIME_MessageBody_GetBody(p_MIME_Message, p_ContentType); var template (present) MIME_Part_Header v_HeaderTemplate := cr_MIME_Part_Header(cr_MimeContentType(p_ContentType), cr_MimeContentDisposition("by-reference", p_DispositionParams), cr_MimeContentId(p_ContentId)); if (not match(v_MIME_Part.header, v_HeaderTemplate)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid MIME header"); } if (ispresent(p_EmergencyCallDataXML)) { if (not ischosen(v_MIME_Part.body.xmlBody) or not f_IMS_ECall_EmergencyCallData_Control_DecodeAndMatch(v_MIME_Part.body.xmlBody, p_EmergencyCallDataXML)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid MIME body"); } } }

function f_IMS_ECall_EmergencyCallData_Control_DecodeAndMatch(charstring p_XmlBody, template EmergencyCallData_Control p_EmergencyCallData_Control) return boolean { var bitstring v_Bitstring := f_Char2Bit(p_XmlBody); var EmergencyCallData_Control v_EmergencyCallData_Control; var boolean v_Result := false; if (decvalue(v_Bitstring, v_EmergencyCallData_Control) == 0) { v_Result := match(v_EmergencyCallData_Control, p_EmergencyCallData_Control); } return v_Result; }

## Change 6

| Function name | f_IMS_OtherResponse_200_MessageHeaderTX |
| --- | --- |
| Reason for change | Accept and RecvInfo headers not being set in 200 OK |
| Summary of change | - Define constants for emergency call control application types - Set the Accept and RecvInfo headers in 200 OK |
| TTCN module | IMS_Procedures_CallControl.ttcn |
| MCC160 Comment | Accepted in principle (see change 2.7.1). NOTE: The changes are only applicable for eCall; prose CR is needed for correction of 34.229-1 A.3.1 |

Before change

function f_IMS_OtherResponse_200_MessageHeaderTX(MessageHeader p_MessageHeader_Request,

template (omit) ContentType p_ContentType := omit,

template (omit) charstring p_EmergencyTelURI := omit,

charstring p_ContactUri := px_IMS_CalleeContactUri,

IMS_PreconditionIndication_Type p_PreconditionIndication := NoPreconditionIndication,

IMS_CallType_Type p_CallType := NORMAL_CALL)

runs on IMS_PTC return template (value) MessageHeader

{ /* @sic R5-130683: common handling for INVITE and UPDATE sic@ */

/* p_EmergencyTelURI .. shall be set e.g. to tsc_IMS_Emergency_TelUri for condition A6 in A.3.1 */

/* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */

/* @sic R5s150237: parameter p_AdditionalOptionTagsForRequire sic@ */

/* @sic R5s150692 change 17: parameter p_CallType  sic@ */

/* @sic R5s150851: no consistency check for p_CallType and p_EmergencyTelURI sic@ */

/* @sic R5-160794: p_AdditionalOptionTagsForRequire replaced by p_PreconditionIndication sic@ */

/* @sic R5-174544: SessionId removed sic@ */

var template (value) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderTX(p_MessageHeader_Request);  /* @sic R5s190882: f_IMS_BaseResponse_MessageHeaderTX sic@ */

var charstring v_Method := p_MessageHeader_Request.cSeq.method;

var boolean v_A1A3 := false;

var boolean v_A18 := pc_IMS_Video_FeatureTag;

var boolean v_A19 := false;

var template (value) RouteBody_List v_RouteBodyList;

var template (omit) Require v_Require := omit;

var template (omit) SemicolonParam_List v_ContactParams := omit;

if ((p_PreconditionIndication == PreconditionRequired) and ispresent(p_ContentType)) {    /* @sic R5-160794 sic@ NOTE: responses use Require header even for session modifications */

v_Require := cs_Require(tsc_OptionTagList_precondition);

}

select (v_Method) {

case ("INVITE") {

v_A1A3 := true;

v_A19 := true;

}

case ("UPDATE") {

v_A1A3 := true;

}

}

v_MessageHeader_Response.require         := v_Require;                                          /* @sic R5-160794 sic@ */

v_MessageHeader_Response.contentType     := p_ContentType;

if (v_A1A3) {

v_RouteBodyList := f_IMS_RouteSet_MO_Call_TX(p_CallType);             /* @sic R5s150692 change 17 sic@ */

v_MessageHeader_Response.recordRoute   := cs_RecordRoute(v_RouteBodyList);

v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("audio"));                     /* @sic R5-165943 sic@ */

if (v_A18 and v_A19) {

v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("video"));                   /* @sic R5-191117 sic@ */

}

v_MessageHeader_Response.contact       := cs_Contact(f_SIP_BuildSipUri_TX(p_ContactUri), v_ContactParams);

}

if (ispresent(p_EmergencyTelURI)) {    // Condition A6 @sic R5s150851: in case of A6 p_EmergencyTelURI shall be handed over from the calling side; p_CallType is not a criteria sic@

v_MessageHeader_Response.pAssertedID   := cs_PAssertedId(f_SIP_BuildSipUri_lr_TX(valueof(p_EmergencyTelURI)));

}

return v_MessageHeader_Response;

}

After change

………

const charstring tsc_EmergencyCallDataControlApplication := "application/EmergencyCallData.Control+xml";

const charstring tsc_EmergencyCallDataMsdApplication := "application/EmergencyCallData.eCall.MSD";

…………

function f_IMS_OtherResponse_200_MessageHeaderTX(MessageHeader p_MessageHeader_Request,

template (omit) ContentType p_ContentType := omit,

template (omit) charstring p_EmergencyTelURI := omit,

charstring p_ContactUri := px_IMS_CalleeContactUri,

IMS_PreconditionIndication_Type p_PreconditionIndication := NoPreconditionIndication,

IMS_CallType_Type p_CallType := NORMAL_CALL)

runs on IMS_PTC return template (value) MessageHeader

{ /* @sic R5-130683: common handling for INVITE and UPDATE sic@ */

/* p_EmergencyTelURI .. shall be set e.g. to tsc_IMS_Emergency_TelUri for condition A6 in A.3.1 */

/* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */

/* @sic R5s150237: parameter p_AdditionalOptionTagsForRequire sic@ */

/* @sic R5s150692 change 17: parameter p_CallType  sic@ */

/* @sic R5s150851: no consistency check for p_CallType and p_EmergencyTelURI sic@ */

/* @sic R5-160794: p_AdditionalOptionTagsForRequire replaced by p_PreconditionIndication sic@ */

/* @sic R5-174544: SessionId removed sic@ */

var template (value) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderTX(p_MessageHeader_Request);  /* @sic R5s190882: f_IMS_BaseResponse_MessageHeaderTX sic@ */

var charstring v_Method := p_MessageHeader_Request.cSeq.method;

var boolean v_A1A3 := false;

var boolean v_A18 := pc_IMS_Video_FeatureTag;

var boolean v_A19 := false;

var template (value) RouteBody_List v_RouteBodyList;

var template (omit) Require v_Require := omit;

var template (omit) SemicolonParam_List v_ContactParams := omit;

if ((p_PreconditionIndication == PreconditionRequired) and ispresent(p_ContentType)) {    /* @sic R5-160794 sic@ NOTE: responses use Require header even for session modifications */

v_Require := cs_Require(tsc_OptionTagList_precondition);

}

select (v_Method) {

case ("INVITE") {

v_A1A3 := true;

v_A19 := true;

}

case ("UPDATE") {

v_A1A3 := true;

}

}

v_MessageHeader_Response.require         := v_Require;                                          /* @sic R5-160794 sic@ */

v_MessageHeader_Response.contentType     := p_ContentType;

if (v_A1A3) {

v_RouteBodyList := f_IMS_RouteSet_MO_Call_TX(p_CallType);             /* @sic R5s150692 change 17 sic@ */

v_MessageHeader_Response.recordRoute   := cs_RecordRoute(v_RouteBodyList);

v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("audio"));                     /* @sic R5-165943 sic@ */

if (v_A18 and v_A19) {

v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("video"));                   /* @sic R5-191117 sic@ */

}

v_MessageHeader_Response.contact       := cs_Contact(f_SIP_BuildSipUri_TX(p_ContactUri), v_ContactParams);

}

if (ispresent(p_EmergencyTelURI)) {    // Condition A6 @sic R5s150851: in case of A6 p_EmergencyTelURI shall be handed over from the calling side; p_CallType is not a criteria sic@

v_MessageHeader_Response.pAssertedID   := cs_PAssertedId(f_SIP_BuildSipUri_lr_TX(valueof(p_EmergencyTelURI)));

}

v_MessageHeader_Response.accept          := cs_Accept({cs_AcceptBody(c_sdpApplication), cs_AcceptBody(tsc_MimeType_Pidf), cs_AcceptBody(tsc_EmergencyCallDataControlApplication), cs_AcceptBody(tsc_EmergencyCallDataMsdApplication)});

v_MessageHeader_Response.recvInfo        := cr_RecvInfo_Def("emergencyCallData.eCall.MSD");

return v_MessageHeader_Response;

}

## Further Changes by MCC160

## Corrections of f_ECall_CallSetup_AnnexC47

- IMS_InviteRequestWithSdp_Type as return type of f_ECall_InitialInvite_AnnexC47

- Common function to build MIME body for eCall DL messages

- Additional SIP headers Accept and RecvInfo for 200 OK (see change 6)

template (value) CallInfo cs_CallInfo(charstring p_CidUrl, charstring p_Purpose):= { // To be used in Annex A.3.1 condition A12 fieldName := CALL_INFO_E, callInfoBody := { { url := p_CidUrl, infoParams := { cs_GenericParam("purpose", p_Purpose) } } } };

function f_IMS_ECall_MIME_MessageBodyTX(charstring p_Boundary, charstring p_XmlPartContentId, template (value) EmergencyCallData_Control p_EmergencyCallData_Control, template (omit) SDP_Message p_SDP_Message := omit) return template (value) MessageBody { var template (value) MIME_Part_List v_MIME_Part_List := {}; var XmlBody v_XmlBody := f_IMS_ECall_EmergencyCallData_Control_Encode(p_EmergencyCallData_Control); v_MIME_Part_List := f_MIME_Part_List_AddXML(v_MIME_Part_List, tsc_MimeType_EmergencyCallDataControlXML, cs_MimeContentDisposition("by-reference"), cs_MimeContentId(p_XmlPartContentId), v_XmlBody); if (isvalue(p_SDP_Message)) { v_MIME_Part_List := f_MIME_Part_List_AddSDP(v_MIME_Part_List, valueof(p_SDP_Message)); } return cs_MessageBody_MIME(cs_MIME_Message(p_Boundary, v_MIME_Part_List)); }

function f_ECall_CallSetup_AnnexC47(ECall_Type p_ECallType) runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) SipUrl v_CalleeContactSipUri; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader_InviteResponse_200; var template (value) AcceptBody_List v_AcceptBodyList; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) SDP_Message v_SDP_Message_Step2; var template (value) MessageBody v_MessageBody; var IMS_CallType_Type v_EmergencyType := REGULAR_EMERGENCY; var IMS_SDP_AudioCodecInfo_Type v_AudioCodecInfo; var charstring v_MSDContentIdRef; var charstring v_BoundaryStep2 := "boundary1"; var charstring v_CidUrl_Step2 := "psap@3gpp.org"; // Step 1. Receive INVITE for an eCall v_InviteRequestWithSdp := f_ECall_InitialInvite_AnnexC47 (p_ECallType); v_InviteRequest := v_InviteRequestWithSdp.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfoAMR(v_InviteRequestWithSdp.SdpOffer); v_MSDContentIdRef := f_ECall_GetMSD_ContentId_InMessageBody(v_InviteRequest.messageBody); // Step 2. SS sends 200 OK with SDP contents v_SDP_Message_Step2 := f_IMS_BuildSDP_EmergencyCallAnswer_TX(v_AudioCodecInfo.Fmt, v_AudioCodecInfo.Codec); v_MessageHeader_InviteResponse_200 := f_IMS_OtherResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, cs_ContentTypeMIME(v_BoundaryStep2), -, -, -, v_EmergencyType); v_AcceptBodyList := { cs_AcceptBody(c_sdpApplication), cs_AcceptBody(tsc_MimeType_Pidf), cs_AcceptBody(tsc_MimeType_EmergencyCallDataControlXML), cs_AcceptBody(tsc_MimeType_EmergencyCallDataMSD) }; v_MessageHeader_InviteResponse_200.accept := cs_Accept(v_AcceptBodyList); v_MessageHeader_InviteResponse_200.callInfo := cs_CallInfo(v_CidUrl_Step2, "EmergencyCallData.Control"); v_MessageHeader_InviteResponse_200.recvInfo := cr_RecvInfo_Def("emergencyCallData.eCall.MSD"); v_MessageBody := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep2, v_CidUrl_Step2, cs_EmergencyCallData_Control_Ack(true, v_MSDContentIdRef), v_SDP_Message_Step2); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader_InviteResponse_200, v_MessageBody))); // Step 3. UE sends ACK v_CalleeContactSipUri := f_MessageHeader_GetContactSipUrl(v_MessageHeader_InviteResponse_200); v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine200, v_EmergencyType); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeContactSipUri, v_MessageHeader_Ack))); }

## Common check for boundary parameter in Content-Type and MIME body

A common check for the boundary parameter is added to a_IMS_ReceiveRequest and a_IMS_ReceiveResponse.
 No such check needed in f_ECall_InitialInvite_AnnexC47 anymore.

function fl_IMS_RequestWithMimeBody_CheckBoundary(MessageHeader p_MessageHeader, MIME_Message p_MIME_Message) runs on IMS_PTC { var template (omit) GenericParam v_BoundaryParam := f_SIP_SemicolonParamList_GetParam(p_MessageHeader.contentType.mParams, "boundary"); if (not isvalue(v_BoundaryParam)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "MIME ContentType with no boundary parameter"); } else if (f_GenericParam_GetValue(v_BoundaryParam) != p_MIME_Message.boundary) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "different boundary in ContentType and message body"); } }

altstep a_IMS_ReceiveRequest(template (present) IMS_DATA_REQ p_IMS_DATA_REQ, out IMS_DATA_REQ p_IMS_DATA_REQ_ByRef) runs on IMS_PTC { var IMS_DATA_REQ v_IMS_DATA_REQ; var IMS_Request_Type v_Request; var MessageHeader v_MessageHeader; var template (omit) MessageBody v_MessageBody; [] IMS_Server.receive(p_IMS_DATA_REQ) -> value v_IMS_DATA_REQ { v_Request := v_IMS_DATA_REQ.Request; if (ischosen(v_Request.Register)) { v_MessageHeader := v_Request.Register.msgHeader; v_MessageBody := v_Request.Register.messageBody; } else if (ischosen(v_Request.Invite)) { v_MessageHeader := v_Request.Invite.msgHeader; v_MessageBody := v_Request.Invite.messageBody; } else if (ischosen(v_Request.Options)) { v_MessageHeader := v_Request.Options.msgHeader; v_MessageBody := v_Request.Options.messageBody; } else if (ischosen(v_Request.Bye)) { v_MessageHeader := v_Request.Bye.msgHeader; v_MessageBody := v_Request.Bye.messageBody; } else if (ischosen(v_Request.Cancel)) { v_MessageHeader := v_Request.Cancel.msgHeader; v_MessageBody := v_Request.Cancel.messageBody; } else if (ischosen(v_Request.Ack)) { v_MessageHeader := v_Request.Ack.msgHeader; v_MessageBody := v_Request.Ack.messageBody; } else if (ischosen(v_Request.Prack)) { v_MessageHeader := v_Request.Prack.msgHeader; v_MessageBody := v_Request.Prack.messageBody; } else if (ischosen(v_Request.Notify)) { v_MessageHeader := v_Request.Notify.msgHeader; v_MessageBody := v_Request.Notify.messageBody; } else if (ischosen(v_Request.Subscribe)) { v_MessageHeader := v_Request.Subscribe.msgHeader; v_MessageBody := v_Request.Subscribe.messageBody; } else if (ischosen(v_Request.Publish)) { v_MessageHeader := v_Request.Publish.msgHeader; v_MessageBody := v_Request.Publish.messageBody; } else if (ischosen(v_Request.Update)) { v_MessageHeader := v_Request.Update.msgHeader; v_MessageBody := v_Request.Update.messageBody; } else if (ischosen(v_Request.Refer)) { v_MessageHeader := v_Request.Refer.msgHeader; v_MessageBody := v_Request.Refer.messageBody; } else if (ischosen(v_Request.Message)) { v_MessageHeader := v_Request.Message.msgHeader; v_MessageBody := v_Request.Message.messageBody; } else if (ischosen(v_Request.Info)) { v_MessageHeader := v_Request.Info.msgHeader; v_MessageBody := v_Request.Info.messageBody; } else { FatalError(__FILE__, __LINE__, "Unknown request"); } if ((v_IMS_DATA_REQ.RoutingInfo.Protocol == tcp) and ispresent(v_MessageBody)) { /* @sic R5-169122 sic@ */ if (not match(v_MessageHeader, cr_MessageHeader_Dummy(cr_ContentLength()))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Request sent on tcp shall have a ContentLength when having a MessageBody"); } } if (ischosen(v_MessageBody.mimeMessageBody)) { fl_IMS_RequestWithMimeBody_CheckBoundary(v_MessageHeader, valueof(v_MessageBody.mimeMessageBody)); } p_IMS_DATA_REQ_ByRef := v_IMS_DATA_REQ; } }

altstep a_IMS_ReceiveResponse(template (value) MessageHeader p_RequestHeader, template (present) InternetProtocol_Type p_Protocol, template (present) IMS_Response_Type p_Response, out IMS_DATA_RSP p_IMS_DATA_RSP_ByRef) runs on IMS_PTC { var IMS_DATA_RSP v_IMS_DATA_RSP; var MessageHeader v_MessageHeader; var template (omit) integer v_PrevRSeq; [] IMS_Client.receive(car_IMS_DATA_RSP(cr_IMS_RoutingInfo(p_Protocol), p_Response)) -> value v_IMS_DATA_RSP { if ((v_IMS_DATA_RSP.RoutingInfo.Protocol == tcp) and ispresent(v_IMS_DATA_RSP.Response.messageBody)) { /* @sic R5-169122 sic@ */ if (not match(v_IMS_DATA_RSP.Response.msgHeader, cr_MessageHeader_Dummy(cr_ContentLength()))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Response sent on tcp shall have a ContentLength when having a MessageBody"); } } v_MessageHeader := v_IMS_DATA_RSP.Response.msgHeader; f_IMS_MessageHeader_Response_CheckVia(valueof(p_RequestHeader), v_MessageHeader); select (v_IMS_DATA_RSP.Response.statusLine.statusCode) { /* @sic R5-181190: store and check RSeq value for reliable responses sic@ */ case (180, 183) { /* @sic R5s190882: rSeq may be optional for MCX => 183 and 180 are handle in the same way sic@ */ fl_IMS_MessageHeader_180_183_Response_CommonChecks(v_MessageHeader); /* @sic R5s180202 sic@ */ if (f_MessageHeader_CheckRequire(v_MessageHeader, c_tag100rel)) { if (not ispresent(v_MessageHeader.rSeq.responseNum)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "reliable 180 response shall have RSeq"); /* 34.229-1 A.2.6 condition A3, A12 */ } v_PrevRSeq := f_IMS_PTC_ImsInfo_DialogGetRemoteRSeq(); if (isvalue(v_PrevRSeq) and (v_MessageHeader.rSeq.responseNum != (valueof(v_PrevRSeq) + 1))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "RSeq value needs to be previous RSeq value + 1"); /* 34.229-1 A.2.6 condition A3 */ } f_IMS_PTC_ImsInfo_DialogSetRemoteRSeq(v_MessageHeader.rSeq.responseNum); } } } if (ischosen(v_IMS_DATA_RSP.Response.messageBody.mimeMessageBody)) { fl_IMS_RequestWithMimeBody_CheckBoundary(v_MessageHeader, v_IMS_DATA_RSP.Response.messageBody.mimeMessageBody); } p_IMS_DATA_RSP_ByRef := v_IMS_DATA_RSP; } }

## Common function to decode and check Geolocation Info

Common function extracted from f_IMS_MessageBody_CheckSdpMessageAndGeolocation and used by f_ECall_InitialInvite_AnnexC47 too.

function f_IMS_DecodeAndCheckGeolocation(charstring p_XmlBody) runs on IMS_PTC { var PIDF_Presence_Type v_Presence := f_PIDF_Decode(p_XmlBody); var Geolocation_Type v_Geolocation; var PIDF_Status_ElemList_Type v_Status_ElemList; var XSD.String v_XsdString; if (not match (v_Presence, cr_PIDF_Presence_Any)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Presence (PIDF) does not match"); } // Check PDIF-LO see RFC 4119 cl. 2.2 // extends the 'status' element of PIDF with a complex element called 'geopriv' // There are two major subelements that are encapsulated within geopriv: one for location information, and one for usage rules if (not ispresent(v_Presence.tuple_list[0].status.elem_list)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Presence.tuple_list[0].status.elem_list not present"); // } else { v_Status_ElemList := v_Presence.tuple_list[0].status.elem_list; } v_XsdString := v_Status_ElemList[0]; // first of 1 or more geopriv elements @sic R5s180518 sic@ v_Geolocation := f_Geolocation_Decode(v_XsdString); // The PIDF-LO shall contain at least the following elements: (34.229-1 cl. A.2.1) // - One or more 'geopriv' elements, each containing: // - One 'location-info' element describing the location of the UE; and // - One 'usage-rules' element describing the limitations of the usage of the location info. if (not match (v_Geolocation, cr_Geolocation_Any)) { // 1st geopriv element is checked: well structured information is matched, not the contents f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Geolocation does not match"); } }

function f_IMS_MessageBody_CheckSdpMessageAndGeolocation(MessageBody p_MessageBody, template (present) SDP_Message p_SDP_MessageTemplate, boolean p_GeolocationInfoAvailable := false) runs on IMS_PTC return SDP_Message { var SdpBody v_SdpBody; var MIME_Part_List v_MimePartList; var Geolocation_Type v_Geolocation; var PIDF_Presence_Type v_Presence; var PIDF_Status_ElemList_Type v_Status_ElemList; var charstring v_XmlBody; var XSD.String v_XsdString; if (p_GeolocationInfoAvailable) { if (not ispresent(p_MessageBody.mimeMessageBody.mimePartList) and lengthof(p_MessageBody.mimeMessageBody.mimePartList) != 2) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid Invite body"); } v_MimePartList := p_MessageBody.mimeMessageBody.mimePartList; if (ischosen(v_MimePartList[0].body.sdpMessageBody)) { v_SdpBody := v_MimePartList[0].body.sdpMessageBody; v_XmlBody := v_MimePartList[1].body.xmlBody; } else { v_SdpBody := v_MimePartList[1].body.sdpMessageBody; v_XmlBody := v_MimePartList[0].body.xmlBody; } // Check PDIF f_IMS_DecodeAndCheckGeolocation(v_XmlBody); } else { v_SdpBody := p_MessageBody.sdpMessageBody; } return f_IMS_SIP_DecodeMatchAndCheckSDP(v_SdpBody, p_SDP_MessageTemplate); }

# Branches Executed

The test case was executed on EUTRA band 1

# Execution Log Files

## Qualcomm SA415M

The Qualcomm SA415M UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_21_1_LOG.html

- PICS/PIXIT parameter file:

TC_21_1_PIXIT.html

| MCC160 Comment | R5s210010: rejected. Replacement SS log provided in R5s210272, based on the MCC160Comments provided above. R5s210272: accepted. |
| --- | --- |

# References

| [1] | R5s210010 : Supporting information for addition of eCall over IMS test case 21.1 |
| --- | --- |