3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210011

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0660 | rev | - | Current version: | 15.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall over IMS test case 21.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS eCall test case 21.2 to the IMS/EUTRA ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS eCall test case 21.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	5

2.5	Change 5	7

2.6	Change 6	8

3	Branches Executed	10

4	Execution Log Files	10

4.1	Qualcomm SA415M	10

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 21.2 which is part of the IMS EUTRA test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	21.2

ATS Version:	iwd-TTCN3-B2019-06_D20wk38

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SA415M

Verification Status:	PASS

# Corrections required

In addition to the below changes, changes 2-6 of R5s210009 are required and applied.

| MCC160 Comment | See MCC160 comments inside R5s210009_MCC160Comments. |
| --- | --- |

## Change 1

| Function name | Testcase TC_21_2 |
| --- | --- |
| Reason for change | IMS2 PTC is not started |
| Summary of change | Create IMS2 PTC and start test case |
| TTCN module | IMS_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before change

testcase TC_21_2() runs on MTC_IMS system SYSTEM_IMS {

// @purpose

//   eCall over IMS / Manual initiation / Normal registration / Emergency registration / Success / 200 OK with ACK

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(300);

v_IMS1 := IMS_PTC.create alive;

f_MTC_IMS_CreateMapAndConnectPTCs_EUTRA(system, v_IMS1, v_IMS2);

v_IMS1.start(f_TC_21_2_IMS1());

t_GuardTimer.start;

f_MTC_MainLoop_IMS(t_GuardTimer);

}

After change

testcase TC_21_2() runs on MTC_IMS system SYSTEM_IMS {

// @purpose

//   eCall over IMS / Manual initiation / Normal registration / Emergency registration / Success / 200 OK with ACK

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(300);

v_IMS1 := IMS_PTC.create alive;

v_IMS2 := IMS_PTC.create alive;

f_MTC_IMS_CreateMapAndConnectPTCs_EUTRA(system, v_IMS1, v_IMS2);

v_IMS1.start(f_TC_21_2_IMS1());

v_IMS2.start(f_TC_21_2_IMS2());

t_GuardTimer.start;

f_MTC_MainLoop_IMS(t_GuardTimer);

}

# Branches Executed

The test case was executed on EUTRA band 1

# Execution Log Files

## Qualcomm SA415M

The Qualcomm SA415M UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_21_2_LOG.html

- PICS/PIXIT parameter file:

TC_21_2_PIXIT.html

| MCC160 Comment | R5s210012: rejected. Replacement SS log provided in R5s210273, based on the MCC160Comments provided above. R5s210273: accepted. |
| --- | --- |

# References

| [1] | R5s210012 : Supporting information for addition of eCall over IMS test case 21.2 |
| --- | --- |