3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210016

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1462 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT test case 8.1.4.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Expected active EPS bearer ID EPS bearer context status in TRACKING AREA UPDATE REQUEST at step 3 is currently fixed to default value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set expected active EPS bearer ID in EPS bearer context status based on the current DRB ID (DRB ID + 4) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_2_1_1_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | Expected active EPS bearer ID EPS bearer context status in TRACKING AREA UPDATE REQUEST at step 3 is currently fixed to default value. |
| Summary of change | Set expected active EPS bearer ID in EPS bearer context status based on the current DRB ID (DRB ID + 4) |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment | Alternative solution implemented subject to prose CR approval: removed the p_EPS_BearerContextStatus parameter as the default value is cr_EPS_BearerContextStatus_Any |

Before Change

function fl_TC_8_1_4_2_1_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var template (value) MobileIdentity v_MappedGUTI; var SRB_COMMON_IND v_ReceivedMsg; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(eutra_Cell1); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); var TrackingAreaCode v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); var template (present) TrackingAreaId v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); var EUTRA_SecurityParams_Type v_Auth_Params := f_EUTRA_Security_Get(); //@sic R5s201390 sic@ var ConfiguredPDNInfoList_Type v_ExistingPDNs; var template (value) EUTRA38_DRBInfoList_Type v_DRBIdList; var DRB_IdentityList_Type v_ListOfDRBs; var template (value) RadioBearerId_Type v_RbId_DRB; var integer i; var octetstring v_HOCmd := f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA(eutra_Cell1, f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1), f_EUTRA_CellInfo_GetRootSequenceIndex (eutra_Cell1)); v_DRBIdList := f_EUTRA38_MobileInfo_GetDRBInfoList(); // @sic R5s201390 sic@ for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { // Hellen v_ListOfDRBs[i] := valueof(v_DRBIdList[i].DRB_Id); } v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @sic R5s200509 sic@ v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (NR); v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit); // Configure C-RNTI based contention resolution in target cell f_EUTRA_SS_ConfigRachProcedure(eutra_Cell1, tsc_C_RNTI_Def2, cs_RachProcedureConfig_CRNTI_Dedicated(f_EUTRA_CellInfo_GetRAR_TA(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1))); // @sic R5-123712 sic@ f_EUTRA_CellInfo_SetC_RNTI ( eutra_Cell1, tsc_C_RNTI_Def2 ); // activate RRC security at SS target cell for SRB 1, 2 and 1 AM DRB v_Auth_Params := f_EUTRA38_MappedContextFromN1_InitNAS(eutra_Cell1, v_SecurityRx, true); v_Auth_Params.AS_Ciphering.ActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(eutra_Cell1, tsc_DrbList_Def); // @sic R5s200938 sic@ f_EUTRA_SS_RRC_EnableIntProt_CiphULandDL(eutra_Cell1, v_Auth_Params.AS_Integrity, v_Auth_Params.AS_Ciphering, cs_TimingInfo_Now); f_EUTRA_Security_Set(v_Auth_Params); //Configure UL grant configuration ("OnSR", periodic TA is NOT started) in target cell (cell 1) f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_ULGrantAllocation_Def_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); // Send EUTRA HO command to NR f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); // @siclog "Step 2" siclog@ SRB.receive( car_SRB1_RrcPdu_IND( eutra_Cell1, cr_508_RRCConnectionReconfigurationComplete( tsc_RRC_TI_Def ) ) ); /* SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the indicated security key and the indicated security algorithms */ f_EUTRA_PreliminaryPass( __FILE__, __LINE__, "Step 2" ); //Configure UL grant configuration f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def)); // @sic R5s191111 sic@ // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), // @sic R5-201638 sic@ v_SecurityRx.KSI, // @sic R5s200509 sic@ cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, // @sic R5s200509 sic@ -, tsc_NasKsi_MappedSecurityContext, // @sic R5-201215, R5s200509 sic@ cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -, -, cr_508_EPS_BearerContextStatusTAUReq ifpresent))))-> value v_ReceivedMsg; // @sic R5s200509, R5s200968, Rs5200760 sic@ // @siclog "Step 4-5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_ReceivedMsg.Signalling.Nas[0], -, noRrcConnectionRelease, -, true); // @sic R5s191111 change 9 sic@ // @siclog "Step 6" siclog@ v_ExistingPDNs := f_EUTRA38_MobileInfo_GetConfiguredPDNInfoList(); v_RbId_DRB := cs_RadioBearerId_DRB(valueof(v_DRBIdList[0].DRB_Id)); // @sic R5s201390 sic@ if (pc_IP_Ping) { // @sic R5-204452 sic@ f_EUTRA_CheckDataPath(eutra_Cell1, PDCP, defaultBearer, "Step 6", v_RbId_DRB, -, -, v_ExistingPDNs[0].PdnIndex); //@sic R5s201390 sic@ } }

After Change

function fl_TC_8_1_4_2_1_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var template (value) MobileIdentity v_MappedGUTI; var SRB_COMMON_IND v_ReceivedMsg; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(eutra_Cell1); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); var TrackingAreaCode v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); var template (present) TrackingAreaId v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); var EUTRA_SecurityParams_Type v_Auth_Params := f_EUTRA_Security_Get(); //@sic R5s201390 sic@ var ConfiguredPDNInfoList_Type v_ExistingPDNs; var template (value) EUTRA38_DRBInfoList_Type v_DRBIdList; var DRB_IdentityList_Type v_ListOfDRBs; var template (value) RadioBearerId_Type v_RbId_DRB; var integer i; var B3_Type v_EBI5_7; var octetstring v_HOCmd := f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA(eutra_Cell1, f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1), f_EUTRA_CellInfo_GetRootSequenceIndex (eutra_Cell1)); v_DRBIdList := f_EUTRA38_MobileInfo_GetDRBInfoList(); // @sic R5s201390 sic@ for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { // Hellen v_ListOfDRBs[i] := valueof(v_DRBIdList[i].DRB_Id); } select (v_ListOfDRBs[0]) { case (1) { v_EBI5_7 := '001'B;} case (2) { v_EBI5_7 := '010'B;} case (3) { v_EBI5_7 := '100'B;} } v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); // @sic R5s200509 sic@ v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (NR); v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit); // Configure C-RNTI based contention resolution in target cell f_EUTRA_SS_ConfigRachProcedure(eutra_Cell1, tsc_C_RNTI_Def2, cs_RachProcedureConfig_CRNTI_Dedicated(f_EUTRA_CellInfo_GetRAR_TA(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1))); // @sic R5-123712 sic@ f_EUTRA_CellInfo_SetC_RNTI ( eutra_Cell1, tsc_C_RNTI_Def2 ); // activate RRC security at SS target cell for SRB 1, 2 and 1 AM DRB v_Auth_Params := f_EUTRA38_MappedContextFromN1_InitNAS(eutra_Cell1, v_SecurityRx, true); v_Auth_Params.AS_Ciphering.ActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(eutra_Cell1, tsc_DrbList_Def); // @sic R5s200938 sic@ f_EUTRA_SS_RRC_EnableIntProt_CiphULandDL(eutra_Cell1, v_Auth_Params.AS_Integrity, v_Auth_Params.AS_Ciphering, cs_TimingInfo_Now); f_EUTRA_Security_Set(v_Auth_Params); //Configure UL grant configuration ("OnSR", periodic TA is NOT started) in target cell (cell 1) f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_ULGrantAllocation_Def_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); // Send EUTRA HO command to NR f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); // @siclog "Step 2" siclog@ SRB.receive( car_SRB1_RrcPdu_IND( eutra_Cell1, cr_508_RRCConnectionReconfigurationComplete( tsc_RRC_TI_Def ) ) ); /* SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the indicated security key and the indicated security algorithms */ f_EUTRA_PreliminaryPass( __FILE__, __LINE__, "Step 2" ); //Configure UL grant configuration f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def)); // @sic R5s191111 sic@ // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), // @sic R5-201638 sic@ v_SecurityRx.KSI, // @sic R5s200509 sic@ cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, // @sic R5s200509 sic@ -, tsc_NasKsi_MappedSecurityContext, // @sic R5-201215, R5s200509 sic@ cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -, -, cr_508_EPS_BearerContextStatusTAUReq(v_EBI5_7) ifpresent))))-> value v_ReceivedMsg; // @sic R5s200509, R5s200968, Rs5200760 sic@ // @siclog "Step 4-5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_ReceivedMsg.Signalling.Nas[0], -, noRrcConnectionRelease, -, true); // @sic R5s191111 change 9 sic@ // @siclog "Step 6" siclog@ v_ExistingPDNs := f_EUTRA38_MobileInfo_GetConfiguredPDNInfoList(); v_RbId_DRB := cs_RadioBearerId_DRB(valueof(v_DRBIdList[0].DRB_Id)); // @sic R5s201390 sic@ if (pc_IP_Ping) { // @sic R5-204452 sic@ f_EUTRA_CheckDataPath(eutra_Cell1, PDCP, defaultBearer, "Step 6", v_RbId_DRB, -, -, v_ExistingPDNs[0].PdnIndex); //@sic R5s201390 sic@ } }