3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210032

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1478 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR PDCP test case 7.1.3.4.1 in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For ENDC case RRC Reconfiguration Complete was received before interleave in step 10(step 24) and therefore is not required to received it here. Interleave statement must be replaced with alt statement as interleave not supported any condition inside []. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace ineterleave statement with alt to receive RRC Reconfiguration Complete message only for NR5GC case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_3_4_1_NR_Step10to23 |
| --- | --- |
| Reason for change | For ENDC case RRC Reconfiguration Complete was received before interleave in step 10 and therefore is not required to received it here. Interleave statement must be replaced with alt statement as interleave not supported any condition inside []. |
| Summary of change | Replace ineterleave statement with alt to receive RRC Reconfiguration Complete message only for NR5GC case. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted in principle. Implemented according to MCC160 proposal |

Before Change:

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2), cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2) }; //@sic R5-195929 sic@ var integer v_Grants:= 1; //@sic R5s201484 sic@ timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU if ( p_WaitforRRCConCmp) { v_Grants:= 2; } //@siclog "Step 10" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ //@sic R5s201484 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants); interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})){} } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); }; //@siclog "Step 12" siclog@ //The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) //and PDU Type =000, FMC field = 3 DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell2, p_NR_PDCP_State.DRB_Id, -, {cs_NR_PDCP_STATUS_REPORT(2)})); //@sic R5-196261 sic@ //@siclog "Step 13" siclog@ …

After Change:

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2), cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2) }; //@sic R5-195929 sic@ var integer v_Grants:= 1; //@sic R5s201484 sic@ timer t_Wait:= 5.0; var boolean v_RRCConCmp := false; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU if ( p_WaitforRRCConCmp) { v_Grants:= 2; } //@siclog "Step 10" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ //@sic R5s201484 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants); alt {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete [p_WaitforRRCConCmp]SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){ v_RRCConCmp := true; repeat; } //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})){ if ( (p_WaitforRRCConCmp) and (not v_RRCConCmp)){ repeat; } } } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); }; //@siclog "Step 12" siclog@ //The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) //and PDU Type =000, FMC field = 3 DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell2, p_NR_PDCP_State.DRB_Id, -, {cs_NR_PDCP_STATUS_REPORT(2)})); //@sic R5-196261 sic@ //@siclog "Step 13" siclog@

MCC160 proposed implementation

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC

{

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82;

var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82;

var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88;

var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 :=  {

cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2),

cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2)

}; //@sic R5-195929 sic@

var integer v_Grants:= 1; //@sic R5s201484 sic@

timer t_Wait:= 5.0;

// for ENDC RRCReconfigurationComplete is not received, just one UL Grant is assigned (112 bits)for STATUS PDU

// For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU

if ( p_WaitforRRCConCmp) {

v_Grants:=  2;

}

//@siclog "Step 10" siclog@

// The SS assigns  UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and

// 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@

//@sic R5s201484 sic@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20);

f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants);

if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@

interleave {//@sic R5s201484 sic@

//Receive RRCReconfigurationComplete

[]SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){}

//@siclog "Step 11" siclog@

//Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS:

//D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5?

[]DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2,

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@

-,

{cr_NR_PDCP_STATUS_REPORT(5)})){}

}

}

else{

//@siclog "Step 11" siclog@

//Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS:

//D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5?

DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2,

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@

-,

{cr_NR_PDCP_STATUS_REPORT(5)})){}

}

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@

f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); //@sic R5s200690 sic@

f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit);

};

//@siclog "Step 12" siclog@

//The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU)

//and PDU Type =000, FMC field = 3

DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell2,

p_NR_PDCP_State.DRB_Id,

-,

{cs_NR_PDCP_STATUS_REPORT(2)})); //@sic R5-196261 sic@

//@siclog "Step 13" siclog@

// The SS starts the UL default grant transmission.

f_NR_ULGrantConfiguration_Start(nr_Cell2);

//@siclog "Step 15" siclog@

p_NR_PDCP_State.RX_NEXT:= 2;//@sic R5-196261 sic@

//The UE sends the PDCP Data PDU

f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell2, v_IPData, cr_RlcBearerRouting_NR(nr_Cell2));//@sic R5s200063 sic@

f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell2, v_IPData, cr_RlcBearerRouting_NR(nr_Cell2));

f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell2, v_IPData, cr_RlcBearerRouting_NR(nr_Cell2));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

//@siclog "Step 16" siclog@

//The SS sends the PDCP Data PDU#5

p_NR_PDCP_State.TX_NEXT:= 5;

f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell2, v_IPData);

//@siclog "Step 17" siclog@

//The UE sends the PDCP Data PDU#5

f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell2, v_IPData, cr_RlcBearerRouting_NR(nr_Cell2));

//@siclog "Step 20" siclog@

//SS transmits a PDCP Data PDU containing one PDCP SDU#7

p_NR_PDCP_State.TX_NEXT:= 7;

p_NR_PDCP_State.RX_NEXT:= 7;

//The SS sends the PDCP Data PDU

f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell2, v_IPData);

//@siclog "Step 21" siclog@

//Check: Does the UE transmit a PDCP DATA PDU#7 on NR Cell 2?

t_Wait.start;

alt {

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell2, p_NR_PDCP_State.DRB_Id)) {

t_Wait.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21");

}

[] t_Wait.timeout{}

}

//@siclog "Step 21A" siclog@

f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@sic R5-190752 sic@

//@siclog "Step 22" siclog@

//The SS changes NR Cell 1 and NR Cell 2 parameters according to row "T2" in Table 7.1.3.4.1.3.1-1.

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

}

## Change 2

| Function name | f_TC_7_1_3_4_1_NR_Step25to30 |
| --- | --- |
| Reason for change | For ENDC case RRC Reconfiguration Complete was received before interleave in step 24 and therefore is not required to received it here. Interleave statement must be replaced with alt statement as interleave not supported any condition inside []. |
| Summary of change | Replace ineterleave statement with alt to receive RRC Reconfiguration Complete message only for NR5GC case. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted in principle. Implemented according to MCC160 proposal |

Before Change:

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var integer v_Grants:= 1; //@sic R5s201484 sic@ timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU if ( p_WaitforRRCConCmp) { v_Grants:= 2; } //@siclog "Step 24-25" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants); interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){} } if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell2, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell2)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell2, cs_TimingInfo_Now, omit); }; //@siclog "Step 27" siclog@

After Change:

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var integer v_Grants:= 1; //@sic R5s201484 sic@ timer t_Wait:= 5.0; var boolean v_RRCConCmp := false; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU if ( p_WaitforRRCConCmp) { v_Grants:= 2; } //@siclog "Step 24-25" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants); alt {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete [p_WaitforRRCConCmp]SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)){ v_RRCConCmp := true; repeat; } //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O { if ( (p_WaitforRRCConCmp) and (not v_RRCConCmp)){ repeat; } } } if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell2, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell2)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell2, cs_TimingInfo_Now, omit); }; //@siclog "Step 27" siclog@

MCC160 proposed implementation

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State,  boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC

{

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var integer v_Grants:= 1; //@sic R5s201484 sic@

timer t_Wait:= 5.0;

// for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU

// For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU

if ( p_WaitforRRCConCmp) {

v_Grants:=  2;

}

//@siclog "Step 24" siclog@

// The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report.

// 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20);

f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants);

if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@

//EXCEPTION: Steps 25 and 26 can occur in any order. (Note 7)

interleave {//@sic R5s201484 sic@

//@siclog "Step 25" siclog@

//The UE transmits a NR RRCReconfigurationComplete message.(Note 4)

[]SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)){}

//@siclog "Step 26" siclog@

//Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS:

//D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6?

[]DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1,

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@

-,

{cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){}

}

}

else{

//@siclog "Step 26" siclog@

//Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS:

//D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6?

DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1,

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@

-,

{cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){}

}

if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@

f_NR_SS_SRBs_DRBs_Release(nr_Cell2, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell2)); //@sic R5s200690 sic@

f_NR_SS_SRBs_DRBs_Config(nr_Cell2, cs_TimingInfo_Now, omit);

};

//@siclog "Step 27" siclog@

//The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6 on NR Cell 1.

DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell1,

p_NR_PDCP_State.DRB_Id,

-,

{cs_NR_PDCP_STATUS_REPORT(6)}));

//@siclog "Step 28" siclog@

// The SS starts the UL default grant transmission.

f_NR_ULGrantConfiguration_Start(nr_Cell1);

//@siclog "Step 28A" siclog@

//The SS sends the PDCP Data PDU#5

p_NR_PDCP_State.TX_NEXT:= 5;

f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData); //@sic R5-190752 sic@

//@siclog "Step 28B" siclog@

//Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS?

t_Wait.start; //@sic R5-190752 sic@

alt {

[] DRB.receive((car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1,

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell1)))) {

t_Wait.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28B");

}

[] t_Wait.timeout{}

}

//@siclog "Step 29" siclog@

//SS transmits a PDCP Data PDU containing one PDCP SDU#6

p_NR_PDCP_State.TX_NEXT:= 6;

p_NR_PDCP_State.RX_NEXT:= 6;

f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData);

//@siclog "Step 30" siclog@

//The UE sends the PDCP Data PDU#6

f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData, cr_RlcBearerRouting_NR(nr_Cell1));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 30");

//The UE sends the PDCP Data PDU#7

f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData, cr_RlcBearerRouting_NR(nr_Cell1));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 31");

}