3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210037

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0661 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Generic IMS functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the 20wk50 TTCN implementation of the function f_NR5GC_IPCAN_MO_SpeechCall, the establishment Cause is expected as mo_Data. This should be changed to mo_Voicecall. A Prose CR will be raised for this. - In the 20wk50 TTCN implementation of the function f_NR5GC_IPCAN_MT_SpeechCall, the service type is expected as ‘0001’B which represents Data. Since this is a MT situation, the service Type should be expected as ‘0010’B, which represents Mobile Terminated Services (24.501 cl 9.11.3.50). A Prose CR will be raised for this. - In the 20wk50 TTCN implementation of the function f_NR5GC_IPCAN_MO_IMS_Signalling, the establishment Cause is expected as mo_Data. However, this function is not just used for MO_IMS_Signalling, but also for IMS_MO_SMS. Thus, when this function is called for IMS_MO_SMS, the expected establishment cause should be mo_SMS. This function needs to be able to handle both, mo_Data and mo_SMS. This needs to be changed accordingly. A Prose CR will be raised for this. - In the 20wk50 TTCN implementation of the function f_NR5GC_IPCAN_MT_IMS_Signalling, the service type is expected as ‘0001’B which represents mo_Data. Since this is a MT situation, the service Type should be expected as ‘0010’B, which represents Mobile Terminated Services (24.501 cl 9.11.3.50). A Prose CR will be raised for this. - As a result of the changes mentioned above, the function f_NR5GC_SpeechCall_Steps2_8 needs to be modified to pass Service Type as a parameter. Also in the main body, the call to function f_NR5GC_RRCIdelToConnected_Steps2_4 also needs to be parameterised for Service Type, rather than hard coding it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the function f_NR5GC_IPCAN_MO_SpeechCall, instead of expecting establishment cause as mo_Data, changed it to mo_Voicecall. - In the function f_NR5GC_IPCAN_MT_SpeechCall, instead of expecting service type as ‘0001’B, changed it to ‘0010’B. - Instead of expecting establishment cause as only mo_Data, updated the function to expect mo_Data and mo_SMS. Please see below. - Instead of expecting establishment cause as only mo_Data, updated the function to expect mo_Data and mo_SMS. Please see below - function f_NR5GC_SpeechCall_Steps2_8 is modified to pass Service Type as a parameter. Also in the main body, the call to function f_NR5GC_RRCIdelToConnected_Steps2_4 now uses a parameter for Service Type instead of it being hard coded to ‘0001’B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS Testcases will fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IMS testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Corrections required	5

1.1	function f_NR5GC_IPCAN_MO_SpeechCall	5

1.2	function f_NR5GC_IPCAN_MT_SpeechCall	6

1.3	function f_NR5GC_IPCAN_MO_IMS_Signalling	7

1.4	function f_NR5GC_IPCAN_MT_IMS_Signalling	7

1.5	function f_NR5GC_SpeechCall_Steps2_8	8

# Corrections required

## function f_NR5GC_IPCAN_MO_SpeechCall

| Function name | f_NR5GC_IPCAN_MO_SpeechCall |
| --- | --- |
| Reason for change | In the 20wk50 TTCN implementation of the function f_NR5GC_IPCAN_MO_SpeechCall, the establishment Cause is expected as mo_Data. This should be changed to mo_Voicecall. A Prose CR will be raised for this. |
| Summary of change | In the function f_NR5GC_IPCAN_MO_SpeechCall, instead of expecting establishment cause as mo_Data, changed it to mo_Voicecall. |
| TTCN module | Common_IMS/PART3_IPCAN/IPCAN_NR5GC.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval |

Before Change:

function f_NR5GC_IPCAN_MO_SpeechCall(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type { var TriggerResult_Type v_Result := NORMAL; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mo_Data); v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port); // wait until IMS PTC has sent "100 Trying" and "183 Session Progress" (steps 3 and 4 of C.21) if (v_Result == NORMAL) { // => procedure shall not be aborted f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); } return v_Result; }

After Change:

function f_NR5GC_IPCAN_MO_SpeechCall(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type { var TriggerResult_Type v_Result := NORMAL; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mo_VoiceCall, '0001'B); //WA#784729 v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port); // wait until IMS PTC has sent "100 Trying" and "183 Session Progress" (steps 3 and 4 of C.21) if (v_Result == NORMAL) { // => procedure shall not be aborted f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); } return v_Result; }

## function f_NR5GC_IPCAN_MT_SpeechCall

| Function name | f_NR5GC_IPCAN_MT_SpeechCall |
| --- | --- |
| Reason for change | In the 20wk50 TTCN implementation of the function f_NR5GC_IPCAN_MT_SpeechCall, the service type is expected as ‘0001’B which represents Data. Since this is a MT situation, the service Type should be expected as ‘0010’B, which represents Mobile Terminated Services (24.501 cl 9.11.3.50). A Prose CR will be raised for this. |
| Summary of change | In the function f_NR5GC_IPCAN_MT_SpeechCall, instead of expecting service type as ‘0001’B, changed it to ‘0010’B. |
| TTCN module | Common_IMS/PART3_IPCAN/IPCAN_NR5GC.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval |

Before Change:

function f_NR5GC_IPCAN_MT_SpeechCall(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type { var TriggerResult_Type v_Result := NORMAL; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access); v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port); // wait until IMS PTC has sent "100 Trying" and "183 Session Progress" (steps 3 and 4 of C.21) if (v_Result == NORMAL) { // => procedure shall not be aborted f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); } return v_Result; }

After Change:

function f_NR5GC_IPCAN_MT_SpeechCall(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type { var TriggerResult_Type v_Result := NORMAL; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B); //WA#784729 _7.x_Service_type handling v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port); // wait until IMS PTC has sent "100 Trying" and "183 Session Progress" (steps 3 and 4 of C.21) if (v_Result == NORMAL) { // => procedure shall not be aborted f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); } return v_Result; }

## function f_NR5GC_IPCAN_MO_IMS_Signalling

| Function name | f_NR5GC_IPCAN_MO_IMS_Signalling |
| --- | --- |
| Reason for change | In the 20wk50 TTCN implementation of the function f_NR5GC_IPCAN_MO_IMS_Signalling, the establishment Cause is expected as mo_Data. However, this function is not just used for MO_IMS_Signalling, but also for IMS_MO_SMS. Thus, when this function is called for IMS_MO_SMS, the expected establishment cause should be mo_SMS. This function needs to be able to handle both, mo_Data and mo_SMS. This needs to be changed accordingly. A Prose CR will be raised for this. |
| Summary of change | Instead of expecting establishment cause as only mo_Data, updated the function to expect mo_Data and mo_SMS. Please see below |
| TTCN module | Common_IMS/PART3_IPCAN/IPCAN_NR5GC.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval |

Before Change:

function f_NR5GC_IPCAN_MO_IMS_Signalling(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_NR_CellId, '0001'B, mo_Data); // @sic R5s201327 sic@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); }

After Change:

function f_NR5GC_IPCAN_MO_IMS_Signalling(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_NR_CellId, '0001'B, (mo_Data, mo_SMS)/*WA#784729*/); // @sic R5s201327 sic@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); }

## function f_NR5GC_IPCAN_MT_IMS_Signalling

| Function name | f_NR5GC_IPCAN_MT_IMS_Signalling |
| --- | --- |
| Reason for change | In the 20wk50 TTCN implementation of the function f_NR5GC_IPCAN_MT_IMS_Signalling, the service type is expected as ‘0001’B which represents mo_Data. Since this is a MT situation, the service Type should be expected as ‘0010’B, which represents Mobile Terminated Services (24.501 cl 9.11.3.50). A Prose CR will be raised for this. |
| Summary of change | Instead of expecting establishment cause as only mo_Data, updated the function to expect mo_Data and mo_SMS. Please see below |
| TTCN module | Common_IMS/PART3_IPCAN/IPCAN_NR5GC.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval |

Before Change:

function f_NR5GC_IPCAN_MT_IMS_Signalling(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_NR_CellId, '0001'B, mt_Access); // @sic R5-204387 sic@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); f_IMS_IPCAN_SendCoOrdMsg(p_Port); // trigger IMS PTC to do IMS signalling after RRC connection establishment }

After Change:

function f_NR5GC_IPCAN_MT_IMS_Signalling(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_NR_CellId, '0010'B, mt_Access); // @sic R5-204387 sic@ //WA#784729 '0001'B --> '0010'B f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); f_IMS_IPCAN_SendCoOrdMsg(p_Port); // trigger IMS PTC to do IMS signalling after RRC connection establishment }

## function f_NR5GC_SpeechCall_Steps2_8

| Function name | f_NR5GC_SpeechCall_Steps2_8 |
| --- | --- |
| Reason for change | As a result of the changes mentioned above, the function f_NR5GC_SpeechCall_Steps2_8 needs to be modified to pass Service Type as a parameter. Also in the main body, the call to function f_NR5GC_RRCIdelToConnected_Steps2_4 also needs to be parameterised for Service Type, rather than hard coding it. |
| Summary of change | function f_NR5GC_SpeechCall_Steps2_8 is modified to pass Service Type as a parameter. Also in the main body, the call to function f_NR5GC_RRCIdelToConnected_Steps2_4 now uses a parameter for Service Type instead of it being hard coded to ‘0001’B. |
| TTCN module | Common/NR5GC/NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_SpeechCall_Steps2_8(NR_CellId_Type p_NR_CellId, EstablishmentCause p_EstCause) runs on NR5GC_PTC return template (omit) GSM_MobilityInfo_Type { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; // Steps 2-4 RRC connection Setup v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_NR_CellId, '0001'B, p_EstCause); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Voice Call Establishment: Service Request"); // Steps 5-8 Configure SRB2 & DRB f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); // Get the Info for the IMS PDU Session already created return f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); }

After Change:

function f_NR5GC_SpeechCall_Steps2_8(NR_CellId_Type p_NR_CellId, EstablishmentCause p_EstCause, template (present) B4_Type p_ServiceType := '0010'B /*WA#784729*/) runs on NR5GC_PTC return template (omit) GSM_MobilityInfo_Type { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; // Steps 2-4 RRC connection Setup v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_NR_CellId, p_ServiceType, p_EstCause); //WA#784729 f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Voice Call Establishment: Service Request"); // Steps 5-8 Configure SRB2 & DRB f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg); // Get the Info for the IMS PDU Session already created return f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); }