3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210043

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1485 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test cases 6.2.2.1 and 6.2.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Corrections for 2nd S1->N1 transition in these test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct paramters passed to f_NR5GC_IRAT_S1ToN1_IdleMode() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.NR5GC, 6.2.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_6_2_2_1_TestBody |
| --- | --- |
| Reason for change | AT step 4, this is not the 1st time transitioning from S1 to N1, so the 3rd parameter of f_NR5GC_IRAT_S1ToN1_IdleMode() should be set to false |
| Summary of change | Set the 3rd parameter of f_NR5GC_IRAT_S1ToN1_IdleMode() to false |
| TTCN module | Idle_CellSelection_NR5GC_NR |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_6_2_2_1_TestBody() runs on NR5GC_PTC { // @siclog "Step 1" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T1" in table 6.2.2.1.3.2-1/2 f_NR_SetCellPower (nr_Cell1, -115, -115); // FR2 FFS // @siclog "Step 2" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); // @siclog "Step 3" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T2" in table 6.2.2.1.3.2-1/2 f_NR_SetCellPower(nr_Cell1, -85, -85); // FR2 FFS // @siclog "Step 4" siclog@ // Generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 take place and UE is camped on NR Cell 1 // The SS transmits RRCRelease message on NR Cell 1 f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1); // @siclog "Step 5" siclog@ // The SS changes SIB1 of NR Cell 1 according to table 6.2.2.1.3.3-3 and notifies the UE of change of System Information on NR Cell 1 by sending Short Message on PDCCH using P-RNTI. // Table 6.2.2.1.3.3-3: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_cellReservedForOtherUse(nr_Cell1, true_); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 6" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect f_NR_WaitModificationPeriods(nr_Cell1); // @siclog "Step 7" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T3" in table 6.2.3.3.3.2-1/2 f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); // send coordination to move context to EUTRA // @siclog "Step 8" siclog@ f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); }

After Change

function fl_TC_6_2_2_1_TestBody() runs on NR5GC_PTC { // @siclog "Step 1" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T1" in table 6.2.2.1.3.2-1/2 f_NR_SetCellPower (nr_Cell1, -115, -115); // FR2 FFS // @siclog "Step 2" siclog@ // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); // @siclog "Step 3" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T2" in table 6.2.2.1.3.2-1/2 f_NR_SetCellPower(nr_Cell1, -85, -85); // FR2 FFS // @siclog "Step 4" siclog@ // Generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 take place and UE is camped on NR Cell 1 // The SS transmits RRCRelease message on NR Cell 1 f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, false); // @siclog "Step 5" siclog@ // The SS changes SIB1 of NR Cell 1 according to table 6.2.2.1.3.3-3 and notifies the UE of change of System Information on NR Cell 1 by sending Short Message on PDCCH using P-RNTI. // Table 6.2.2.1.3.3-3: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_cellReservedForOtherUse(nr_Cell1, true_); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 6" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect f_NR_WaitModificationPeriods(nr_Cell1); // @siclog "Step 7" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T3" in table 6.2.3.3.3.2-1/2 f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); // send coordination to move context to EUTRA // @siclog "Step 8" siclog@ f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); }

## Change 2

| Function name | fl_TC_6_2_3_3_TestBody |
| --- | --- |
| Reason for change | AT step 6, this is not the 1st time transitioning from S1 to N1, so the 3rd parameter of f_NR5GC_IRAT_S1ToN1_IdleMode() should be set to false |
| Summary of change | Set the 3rd parameter of f_NR5GC_IRAT_S1ToN1_IdleMode() to false |
| TTCN module | Idle_CellReSelection_NR5GC_NR |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_6_2_3_3_TestBody() runs on NR5GC_PTC { var integer v_ModificationPeriodFrames; var float v_ModificationPeriod_s; //@siclog "Step 1" siclog@ //The SS adjusts the NR and E-UTRAN Cell power levels according to row "T1" in table 6.2.3.3.3.2-1/2 f_NR_SetCellPower (nr_Cell1, -95, -95); // FR2 FFS // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA); //@siclog "Step 4" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T2" in table 6.2.3.3.3.2-1/2 f_NR_SetCellPower (nr_Cell1, -78, -78); // FR2 FFS // @siclog "Step 5 - Void " siclog@ // @siclog "Step 6" siclog@ // Generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 take place and UE is camped on NR Cell 1 // The SS transmits RRCRelease message on NR Cell 1 f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1); // @siclog "Step 8" siclog@ // The SS notifies the UE of change of System Information on NR Cell 1 by send Short Message on PDCCH using P-RNTI. // @siclog "Step 9" siclog@ // The SS changes Priority of E-UTRA cell 1 in the SIB5 of NR Cell 1, The ValueTag of SIB5 in the SIB1 is increased on NR Cell 1. // Table 6.2.3.3.3.3-5: SIB5 of NR Cell 1 f_NR5GC_CellInfo_SetSIB5_cellReselectionPriority(nr_Cell1, 0, 3); f_NR5GC_CellInfo_SetSIB5_threshXHigh(nr_Cell1, 0, 20); f_NR5GC_CellInfo_SetSIB5_threshXLow(nr_Cell1, 0, 10); // Table 6.2.3.3.3.3-6: SIB1 of NR Cell 1 f_NR_IncrementValueTag(nr_Cell1, sibType5); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 10" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod (nr_Cell1); v_ModificationPeriod_s := int2float( v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod_s); // @siclog "Step 11" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T3" in table 6.2.3.3.3.2-1/2 f_NR_SetCellPower (nr_Cell1, -95, -95); // FR2 FFS f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); // send coordination to move context to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); // @sic R5s191181 Ch.11 sic@ // @siclog "Step 12 - Void " siclog@ }

After Change

function fl_TC_6_2_3_3_TestBody() runs on NR5GC_PTC { var integer v_ModificationPeriodFrames; var float v_ModificationPeriod_s; //@siclog "Step 1" siclog@ //The SS adjusts the NR and E-UTRAN Cell power levels according to row "T1" in table 6.2.3.3.3.2-1/2 f_NR_SetCellPower (nr_Cell1, -95, -95); // FR2 FFS // Send both authentication and NR sysinfo parameters to EUTRA that will be used throughout the test f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA); //@siclog "Step 4" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T2" in table 6.2.3.3.3.2-1/2 f_NR_SetCellPower (nr_Cell1, -78, -78); // FR2 FFS // @siclog "Step 5 - Void " siclog@ // @siclog "Step 6" siclog@ // Generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 take place and UE is camped on NR Cell 1 // The SS transmits RRCRelease message on NR Cell 1 f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, -, false);//KS R5sxxx change_6233 // @siclog "Step 8" siclog@ // The SS notifies the UE of change of System Information on NR Cell 1 by send Short Message on PDCCH using P-RNTI. // @siclog "Step 9" siclog@ // The SS changes Priority of E-UTRA cell 1 in the SIB5 of NR Cell 1, The ValueTag of SIB5 in the SIB1 is increased on NR Cell 1. // Table 6.2.3.3.3.3-5: SIB5 of NR Cell 1 f_NR5GC_CellInfo_SetSIB5_cellReselectionPriority(nr_Cell1, 0, 3); f_NR5GC_CellInfo_SetSIB5_threshXHigh(nr_Cell1, 0, 20); f_NR5GC_CellInfo_SetSIB5_threshXLow(nr_Cell1, 0, 10); // Table 6.2.3.3.3.3-6: SIB1 of NR Cell 1 f_NR_IncrementValueTag(nr_Cell1, sibType5); f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS // @siclog "Step 10" siclog@ // Wait for 2.1* modification period to allow the new system information to take effect v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod (nr_Cell1); v_ModificationPeriod_s := int2float( v_ModificationPeriodFrames)/100.0; //Modification period in seconds f_Delay(2.1 * v_ModificationPeriod_s); // @siclog "Step 11" siclog@ // The SS adjusts the NR and E-UTRAN Cell power levels according to row "T3" in table 6.2.3.3.3.2-1/2 f_NR_SetCellPower (nr_Cell1, -95, -95); // FR2 FFS f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); // send coordination to move context to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); // @sic R5s191181 Ch.11 sic@ // @siclog "Step 12 - Void " siclog@ }