3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210052

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1494 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN-DC FR1 : Re-verification of NR RLC test case 7.1.2.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Rohde & Scharwz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To re-verify NR RLC test case 7.1.2.3.4 with iwd-20wk50 for EN-DC FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR RLC test case 7.1.2.3.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.4.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

3	Branches executed	7

4	Execution Log Files	7

4.1	Qualcomm SM8350	7

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.2.3.4 which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Parikshit Bhise
	parikshit.bhise@rogde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.2.3.4.ENDC

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and R&S 5G Protocol Conformance Test platform

UE used:	Qualcomm SM8350

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | function fl_TC_7_1_2_3_3and4() |
| --- | --- |
| Reason for change | Not needed to increment ‘i’ after ‘k’ loops ends since it is increment in “for” statement for the next interation of the “i” loop |
| Summary of change | Remove statement to increment value of ‘I’. |
| TTCN module | NR_TC_Common\7_1_2\ RLC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_7_1_2_3_3and4(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { var integer v_RLC_SDUsize; //@sic R5s190358 sic@ var B1_Type v_Poll; var integer i; var integer j; var integer v_SlotsNumber := 0; // @sic R5s201119 sic@ var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataPerSlotList; var template (value) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU; var template (present) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Expected; var integer v_VerdictFilter; // @sic R5-185083 sic@ var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Received; var NR_L2DataList_Type v_NR_L2DataList; var integer v_LenPDUList; var integer k; ....................... for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-202613, R5-206326 sic@ // Get next list of RLC PDUs v_NR_L2DataList := f_NR_RxAMD_SDU_GetList(p_RLC_Rec, p_NR_DRB_Id); v_LenPDUList := lengthof(v_NR_L2DataList.RlcPdu); for (k := 0; k < v_LenPDUList; k := k + 1) { // set i to account for k i := i + k; if (i == p_InnerLoop) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, too many RLC PDUs received");}; // extract the RLC PDU to be handled v_NR_RLC_AMD_PDU_Received := v_NR_L2DataList.RlcPdu[k].AMD; // build the RLC PDU expected (template) v_NR_RLC_AMD_PDU_Expected := f_NR_RLC_Get_AMD_FullSDU_RX(p_RLC_Rec, ?, v_NR_RLC_DataList[i]); // match the RLC PDU to be handled against the RLC PDU expected (template) if (not match (v_NR_RLC_AMD_PDU_Received, v_NR_RLC_AMD_PDU_Expected)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, mismatch of received and expected PDUs"); }; // extract the received Pollbit select union (v_NR_RLC_AMD_PDU_Received) { case (SN18Bit) { v_Poll := v_NR_RLC_AMD_PDU_Received.SN18Bit.Header.Poll; } case (SN12Bit) { v_Poll := v_NR_RLC_AMD_PDU_Received.SN12Bit.Header.Poll; } } if ((i == 0) and (j == 0)) { // SN = 0 f_NR_PreliminaryPass(__FILE__, __LINE__, " SN=0 "); } else { // SN = 1 .. if ((((j * p_InnerLoop) + i + 1) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, " SN = " & int2str((j * p_InnerLoop) + i) )}; // @sic R5-185083, R5-187106, R5s200315, R5s200957 sic@ } // EXCEPTION: Step 3a1 describes behaviour that depends on the contents of the AMD PDU transmitted at Step 2. // @siclog "Step 3a1" siclog@ // IF the UE has set the poll bit in the AMD PDU transmitted at Step 2 THEN the SS transmits a Status Report. if (v_Poll == '1'B) { f_NR_TxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); } } // end k loop i := i + 1; // next value of i to be handled after a list of k elements has been processed } // end inner loop } // end outer loop // @siclog "Step 4" siclog@ // SS transmits a RLC Data PDU containing one RLC SDU. RLC SN equals 0. // Regenerate RLC SDU so that PDCP SN is incremented (not reset to 0) @sic R5s190358 sic@ v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); ...............

After Change:

function fl_TC_7_1_2_3_3and4(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { var integer v_RLC_SDUsize; //@sic R5s190358 sic@ var B1_Type v_Poll; var integer i; var integer j; var integer v_SlotsNumber := 0; // @sic R5s201119 sic@ var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataPerSlotList; var template (value) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU; var template (present) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Expected; var integer v_VerdictFilter; // @sic R5-185083 sic@ var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Received; var NR_L2DataList_Type v_NR_L2DataList; var integer v_LenPDUList; var integer k; …………. for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-202613, R5-206326 sic@ // Get next list of RLC PDUs v_NR_L2DataList := f_NR_RxAMD_SDU_GetList(p_RLC_Rec, p_NR_DRB_Id); v_LenPDUList := lengthof(v_NR_L2DataList.RlcPdu); for (k := 0; k < v_LenPDUList; k := k + 1) { // set i to account for k i := i + k; if (i == p_InnerLoop) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, too many RLC PDUs received");}; // extract the RLC PDU to be handled v_NR_RLC_AMD_PDU_Received := v_NR_L2DataList.RlcPdu[k].AMD; // build the RLC PDU expected (template) v_NR_RLC_AMD_PDU_Expected := f_NR_RLC_Get_AMD_FullSDU_RX(p_RLC_Rec, ?, v_NR_RLC_DataList[i]); // match the RLC PDU to be handled against the RLC PDU expected (template) if (not match (v_NR_RLC_AMD_PDU_Received, v_NR_RLC_AMD_PDU_Expected)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, mismatch of received and expected PDUs"); }; // extract the received Pollbit select union (v_NR_RLC_AMD_PDU_Received) { case (SN18Bit) { v_Poll := v_NR_RLC_AMD_PDU_Received.SN18Bit.Header.Poll; } case (SN12Bit) { v_Poll := v_NR_RLC_AMD_PDU_Received.SN12Bit.Header.Poll; } } if ((i == 0) and (j == 0)) { // SN = 0 f_NR_PreliminaryPass(__FILE__, __LINE__, " SN=0 "); } else { // SN = 1 .. if ((((j * p_InnerLoop) + i + 1) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, " SN = " & int2str((j * p_InnerLoop) + i) )}; // @sic R5-185083, R5-187106, R5s200315, R5s200957 sic@ } // EXCEPTION: Step 3a1 describes behaviour that depends on the contents of the AMD PDU transmitted at Step 2. // @siclog "Step 3a1" siclog@ // IF the UE has set the poll bit in the AMD PDU transmitted at Step 2 THEN the SS transmits a Status Report. if (v_Poll == '1'B) { f_NR_TxSTATUS_PDU(p_RLC_Rec, p_NR_DRB_Id); } } // end k loop //REMOVED i := i + 1; // next value of i to be handled after a list of k elements has been processed } // end inner loop } // end outer loop // @siclog "Step 4" siclog@ // SS transmits a RLC Data PDU containing one RLC SDU. RLC SN equals 0. // Regenerate RLC SDU so that PDCP SN is incremented (not reset to 0) @sic R5s190358 sic@ v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); ………… }

## Change 2

| Function name | TC_7_1_2_3_4_ENDC() |
| --- | --- |
| Reason for change | In the current TTCN implementation, The guard timer of 5300s is not enough to run the complete loop of 8192 * 32 and needs to be increased to match the same amount of time as in the NR5GC version of the test case |
| Summary of change | Increased the guard timer to 12000s to match the NR5GC version of the test case |
| TTCN module | ATS_ENDC/ENDC_Testsuite.ttcn |
| MCC160 Comment | Not needed The guard time seems to be largely sufficient. |

Before Change:

testcase TC_7_1_2_3_4_ENDC() runs on MTC_ENDC system SYSTEM_ENDC { // @purpose // AM RLC / 18-bit SN / Correct use of sequence numbering var ENDC_NR_PTC v_ENDC_NR := null; var EUTRA_5GS_PTC v_EUTRA_5GS := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(5300); v_ENDC_NR := ENDC_NR_PTC.create alive; v_EUTRA_5GS := EUTRA_5GS_PTC.create alive; f_MTC_ConnectPTCs_ENDC(system, v_ENDC_NR, v_EUTRA_5GS, v_IMS1, v_IMS2); v_ENDC_NR.start(f_TC_7_1_2_3_4_ENDC_NR()); v_EUTRA_5GS.start(f_TC_7_1_2_3_4_ENDC_EUTRA()); t_GuardTimer.start; f_MTC_MainLoop_ENDC(t_GuardTimer); }

After Change:

testcase TC_7_1_2_3_4_ENDC() runs on MTC_ENDC system SYSTEM_ENDC { // @purpose // AM RLC / 18-bit SN / Correct use of sequence numbering var ENDC_NR_PTC v_ENDC_NR := null; var EUTRA_5GS_PTC v_EUTRA_5GS := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(12000); v_ENDC_NR := ENDC_NR_PTC.create alive; v_EUTRA_5GS := EUTRA_5GS_PTC.create alive; f_MTC_ConnectPTCs_ENDC(system, v_ENDC_NR, v_EUTRA_5GS, v_IMS1, v_IMS2); v_ENDC_NR.start(f_TC_7_1_2_3_4_ENDC_NR()); v_EUTRA_5GS.start(f_TC_7_1_2_3_4_ENDC_EUTRA()); t_GuardTimer.start; f_MTC_MainLoop_ENDC(t_GuardTimer); }

# Branches executed

The test case was executed on EN-DC band combination DC_2_n5

# Execution Log Files

## Qualcomm SM8350

The Qualcomm SM8350 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_7_1_2_3_4_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_7_1_2_3_4_PIXIT.html

The Qualcomm SM8350 UE also passed this test case on R&S 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log and PIXS/PIXIT parameter file:

R&S/ tc_7_1_2_3_4_ENDC_FR1.txt

# References

| [1] | R5s210053 : EN-DC FR1 : Supporting information for Re-verification of NR RLC test case 7.1.2.3.4 |
| --- | --- |