3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210064

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4576 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to eMTC test case 7.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the Table 7.1.3.9.3.2-2: Main behavior in 36.523-1-g60, after Step 5, there is no checkpoint on the SFN of ACK for RRCConnectionReconfiguration. At Step 6, checked the UE transmit on Cell 2, RRCConnectionReconfigurationComplete, that can confirm the UE receive RRCConnectionReconfiguration or not. As per R5s190055, the SIB Typ1 information needs to be included in the RRCConnectionReconfiguration, hence the length of the message is increased. This otherwise for Cat-M mode, during HO procedure, causes the failure to receive the RRCConnectionReconfiguration on the UE side. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Cat-M, set the expected SFN of ACK to any. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## Change 1

| Function name | f_TC_7_1_3_9_EUTRA() |
| --- | --- |
| Reason for change | According to the Table 7.1.3.9.3.2-2: Main behavior in 36.523-1-g60, after Step 5, there is no checkpoint on the SFN of ACK for RRCConnectionReconfiguration. At Step 6, checked the UE transmit on Cell 2, RRCConnectionReconfigurationComplete, that can confirm the UE receive RRCConnectionReconfiguration or not. As per R5s190055, the SIB Typ1 information needs to be included in the RRCConnectionReconfiguration, hence the length of the message is increased. This otherwise for Cat-M mode, during HO procedure, causes the failure to receive the RRCConnectionReconfiguration on the UE side. |
| Summary of change | For Cat-M, set the expected SFN of ACK to any. |
| TTCN module | MAC_713.tttcn |
| MCC160 Comment | Accepted the issue implemented differently. Removed timing check for any nack and introduced new flag v_Step3NackRcvd. |

Before Change

function f_TC_7_1_3_9_EUTRA ( ) runs on EUTRA_PTC

{ ……

//@siclog "Step 6" siclog@

// Check: does the UE transmit a RRCConnectionReconfigurationComplete message to confirm the reconfiguration of the radio resources?

t_Watchdog.start;

alt {

[] SRB.receive ( car_SRB1_RrcPdu_IND ( eutra_Cell2, cr_508_RRCConnectionReconfigurationComplete ( tsc_RRC_TI_Def ) ) ) -> value v_SRB_COMMON_IND

{

v_Timing := v_SRB_COMMON_IND.Common.TimingInfo.SubFrame; //store TimeStamp when RRCConnectionReconfiguration has been received

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

t_Watchdog.stop;

}

[]  SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack ))

{

repeat;

// any NACKS received

}

//@sic R5s170939 sic@

[]  SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo( v_Timing2.SFN.Number, v_SubFrameRRCReconfigAck ), ack )) // subframe 8 or 0 for ACK both for FDD and TDD configuration 1

//note: retransmissions not allowed in this test case

{

repeat;

// ACK  received for RRC connection reconfiguration

}

};

//@siclog "Step 7" siclog@

……

}

After change

function f_TC_7_1_3_9_EUTRA ( ) runs on EUTRA_PTC

{

……

//@siclog "Step 6" siclog@

// Check: does the UE transmit a RRCConnectionReconfigurationComplete message to confirm the reconfiguration of the radio resources?

t_Watchdog.start;

alt {

[] SRB.receive ( car_SRB1_RrcPdu_IND ( eutra_Cell2, cr_508_RRCConnectionReconfigurationComplete ( tsc_RRC_TI_Def ) ) ) -> value v_SRB_COMMON_IND

{

v_Timing := v_SRB_COMMON_IND.Common.TimingInfo.SubFrame; //store TimeStamp when RRCConnectionReconfiguration has been received

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

t_Watchdog.stop;

}

[]  SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack ))

{

repeat;

// any NACKS received

}

//@sic R5s170939 sic@

[pc_ue_CategoryDL_M1]  SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo( ?, v_SubFrameRRCReconfigAck ), ack )) // subframe 8 or 0 for ACK both for FDD and TDD configuration 1

//note: retransmissions not allowed in this test case

{

repeat;

// ACK  received for RRC connection reconfiguration

}

[not pc_ue_CategoryDL_M1]  SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo( v_Timing2.SFN.Number, v_SubFrameRRCReconfigAck ), ack )) // subframe 8 or 0 for ACK both for FDD and TDD configuration 1

//note: retransmissions not allowed in this test case

{

repeat;

// ACK  received for RRC connection reconfiguration

}

};

//@siclog "Step 7" siclog@

……

}

MCC 160 Proposed implementation

function f_TC_7_1_3_9_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.3.9 : MAC reset-DL … //var SubFrameNumber_Type v_SubFrameRRCReconfigAck := 8; //@sic R5s170939 sic@ var boolean v_Step3NackRcvd := false; … f_EUTRA_508RRC_ConnReconfigHO_IntraLTE (eutra_Cell1, eutra_Cell2, cs_TimingInfo_SubFrameOffset( v_Timing2.SFN.Number, 4, v_Timing2.HSFN.Number ), v_RRCConnectionReconfiguration_v890_IEs); // next sub frame suitable for both FDD and TDD @sic R5w160204r1 sic@ /* if (pc_ue_CategoryDL_M1) //@sic R5s170939 R5s180176 r5s180381 sic@ { v_SubFrameRRCReconfigAck := 0; v_Timing2.HSFN.Number:= (v_Timing2.HSFN.Number + ((v_Timing2.SFN.Number) + 1)/ 1024) mod 1024; v_Timing2.SFN.Number := ((v_Timing2.SFN.Number) + 1) mod 1024; } */ … t_Watchdog.start; alt { [] SRB.receive ( car_SRB1_RrcPdu_IND ( eutra_Cell2, cr_508_RRCConnectionReconfigurationComplete ( tsc_RRC_TI_Def ) ) ) -> value v_SRB_COMMON_IND { v_Timing := v_SRB_COMMON_IND.Common.TimingInfo.SubFrame; //store TimeStamp when RRCConnectionReconfiguration has been received f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack )) { v_Step3NackRcvd := true; repeat; // any NACKS received } //@sic R5s170939 sic@ [v_Step3NackRcvd] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, ack )) // //note: retransmissions not allowed in this test case { repeat; // ACK received for RRC connection reconfiguration in step 5 } }; … }; // end of 7.1.3.9