3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210076

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1510 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test case 8.1.4.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For the EUTRA->NR handover, NCC at EUTRA (source) side should be set increased by one (set to 2) - For S1-N1 handover, the UE has not established a PDU session at NR side before handover, so the UE may not include pduSessionStatus in Regustration Request. In this case v_FromLTEList is not initialised which can lead to a run-time error later |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set v_SecurityParams.NCC to 2 - Add logic to initialise v_FromLTEList using p_PduSessionsFromLTE in case v_GMM_MobilityInfo.SessionStatus is not present |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | For the EUTRA->NR handover, NCC at EUTRA (source) side should be set increased by one (set to 2) |
| Summary of change | Set v_SecurityParams.NCC to 2 |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment | Rejected. There is no justification for this change. If this is required, this must be specified in the prose |

Before Change

function fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var octetstring v_NR_HOCmd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); // @sic R5s201078 sic@ as per 33.401 7.2.8.4.3: // the source MME shall increase its locally kept NCC value by one and compute a fresh NH using A.4 v_SecurityParams.NH := f_EUTRA_NB_Authentication_S12(v_SecurityParams.KDF, v_SecurityParams.KASME, v_SecurityParams.NH); // @sic R5s201654 sic@ // 33.501 cl. 8.4.2 point 3: the target AMF shall also set the NCC to zero v_SecurityParams.NCC := 0; f_EUTRA_Security_Set (v_SecurityParams); //@siclog "Step 1" siclog@ // Send latest parameters to NR f_EUTRA38_Send_AuthParametersToNR(); f_EUTRA38_SendPduSessionInfoListToNR(); // Wait for NR to send back encoded RRC Reconfiguration message v_NR_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(NR, "HO Cmd"); SRB.send( cas_SRB1_RrcPdu_REQ( eutra_Cell1, cs_TimingInfo_Now, cs_MobilityFromEutraCommand_Handover( tsc_RRC_TI_Def, false, nr, v_NR_HOCmd, omit))); // UE now moved to NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); }

After Change

function fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var octetstring v_NR_HOCmd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); // @sic R5s201078 sic@ as per 33.401 7.2.8.4.3: // the source MME shall increase its locally kept NCC value by one and compute a fresh NH using A.4 v_SecurityParams.NH := f_EUTRA_NB_Authentication_S12(v_SecurityParams.KDF, v_SecurityParams.KASME, v_SecurityParams.NH); // @sic R5s201654 sic@ // 33.501 cl. 8.4.2 point 3: the target AMF shall also set the NCC to zero v_SecurityParams.NCC := 2; f_EUTRA_Security_Set (v_SecurityParams); //@siclog "Step 1" siclog@ // Send latest parameters to NR f_EUTRA38_Send_AuthParametersToNR(); f_EUTRA38_SendPduSessionInfoListToNR(); // Wait for NR to send back encoded RRC Reconfiguration message v_NR_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(NR, "HO Cmd"); SRB.send( cas_SRB1_RrcPdu_REQ( eutra_Cell1, cs_TimingInfo_Now, cs_MobilityFromEutraCommand_Handover( tsc_RRC_TI_Def, false, nr, v_NR_HOCmd, omit))); // UE now moved to NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); }

## Change 2

| Function name | f_NR5GC_IRAT_S1ToN1_SessionModification |
| --- | --- |
| Reason for change | For S1-N1 handover, the UE has not established a PDU session at NR side before handover, so the UE may not include pduSessionStatus in Regustration Request. In this case v_FromLTEList is not initialised which can lead to a run-time error later |
| Summary of change | Add logic to initialise v_FromLTEList using p_PduSessionsFromLTE in case v_GMM_MobilityInfo.SessionStatus is not present |
| TTCN module | NR5GC_IRAT_CommonFunctions |
| MCC160 Comment | Rejected. The PDU session status must be included if there are active EPS sessions. If there are no active PDU sessions, there won’t be any sessions to be modified: 24.501 8.2.6.9 states that the PDU session status will be included that are active within the UE 24.501 clause 6.1.4.1: d) for each default EPS bearer context in state BEARER CONTEXT ACTIVE the UE shall set the state of the mapped PDU session to PDU SESSION ACTIVE 24.301 clause 6.1.3.2.2 specifies BEARER CONTEXT ACTIVE state is entered following Activate Default EPS Bearer Context Accept 24.501 clauses 6.1.3.2.1 and 6.1.3.3.1 state that PDU session Modification Request can only be sent when the PDU session is active |

Before Change

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_NR_CellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE, boolean p_FailWhenK_LessN := true, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection)) runs on NR5GC_PTC return boolean { var template (value) PDUSessionInfoList_Type v_FromLTEList; var NG_NAS_UL_PduList_Type v_PduSessionModificationRequestList := {}; var IntegerList_Type v_PDU_SessionIdList := {}; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_NG_NAS_UL_Pdu; var NG_NAS_UL_Message_Type v_GSMMsg; var integer v_PDU_SessionId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template(value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_RadioBearerList; // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here) var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var boolean v_ReregisterIMS := false; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var NR_DRBInfoList_Type v_NewDRBInfoList; var integer N := p_ExpectedNumberOfNewPDUSessions; // number of PDU SESSION MODIFICATION REQUEST messages being expected var integer K := 0; // number of PDU SESSION MODIFICATION REQUEST messages already processed (i.e. the corresponding PDU SESSION MODIFICATION COMMAND and PDU SESSION MODIFICATION COMPLETE have been sent/received) var integer L := 0; // number of PDU SESSION MODIFICATION REQUEST messages being received var boolean v_WaitForComplete := false; var template (present) NG_UE_SM_Cap v_NG_UE_SM_Cap := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1 var template (present) IntegrityProtMaxDataRate v_IntegrityProtMaxDataRate := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1 var integer i; var SRBs_AlreadyConfigured v_SRBs := SRB1; var IntegerList_Type v_TransferredSessions; var PDU_PDN_DNN_Type v_PDU_Type := Internet_DNN; // assume this for now @sic R5-206296 sic@ timer t_Wait := 10.0; // @sic R5-204330 sic@ // Build the GSMMobilityInfo for all received PDUs, but don't store it (some may be established, rather than modified) if (ispresent(v_GMM_MobilityInfo.SessionStatus)) { // @sic R5-206290 sic@ v_TransferredSessions := f_NG_ConvertPDU_SessionStatus(valueof(v_GMM_MobilityInfo.SessionStatus)); for (i := 0; i < lengthof (p_PduSessionsFromLTE); i := i + 1) { if (f_IntegerList_Search(v_TransferredSessions, p_PduSessionsFromLTE[i].PDU_SessionId) != -1) { // don't store if not in transferred list if (p_PduSessionsFromLTE[i].IsIMS) { // @sic R5-206296 sic@ v_PDU_Type := IMS_DNN; } v_FromLTEList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionsFromLTE[i].PDU_SessionId, 1), -, v_PDU_Type); // @sic R5s200564 sic@ } } } ….. }

After Change

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_NR_CellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE, boolean p_FailWhenK_LessN := true, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection)) runs on NR5GC_PTC return boolean { var template (value) PDUSessionInfoList_Type v_FromLTEList; var NG_NAS_UL_PduList_Type v_PduSessionModificationRequestList := {}; var IntegerList_Type v_PDU_SessionIdList := {}; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_NG_NAS_UL_Pdu; var NG_NAS_UL_Message_Type v_GSMMsg; var integer v_PDU_SessionId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template(value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_RadioBearerList; // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here) var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var boolean v_ReregisterIMS := false; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var NR_DRBInfoList_Type v_NewDRBInfoList; var integer N := p_ExpectedNumberOfNewPDUSessions; // number of PDU SESSION MODIFICATION REQUEST messages being expected var integer K := 0; // number of PDU SESSION MODIFICATION REQUEST messages already processed (i.e. the corresponding PDU SESSION MODIFICATION COMMAND and PDU SESSION MODIFICATION COMPLETE have been sent/received) var integer L := 0; // number of PDU SESSION MODIFICATION REQUEST messages being received var boolean v_WaitForComplete := false; var template (present) NG_UE_SM_Cap v_NG_UE_SM_Cap := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1 var template (present) IntegrityProtMaxDataRate v_IntegrityProtMaxDataRate := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1 var integer i; var SRBs_AlreadyConfigured v_SRBs := SRB1; var IntegerList_Type v_TransferredSessions; var PDU_PDN_DNN_Type v_PDU_Type := Internet_DNN; // assume this for now @sic R5-206296 sic@ timer t_Wait := 10.0; // @sic R5-204330 sic@ // Build the GSMMobilityInfo for all received PDUs, but don't store it (some may be established, rather than modified) if (ispresent(v_GMM_MobilityInfo.SessionStatus)) { // @sic R5-206290 sic@ v_TransferredSessions := f_NG_ConvertPDU_SessionStatus(valueof(v_GMM_MobilityInfo.SessionStatus)); for (i := 0; i < lengthof (p_PduSessionsFromLTE); i := i + 1) { if (f_IntegerList_Search(v_TransferredSessions, p_PduSessionsFromLTE[i].PDU_SessionId) != -1) { // don't store if not in transferred list if (p_PduSessionsFromLTE[i].IsIMS) { // @sic R5-206296 sic@ v_PDU_Type := IMS_DNN; } v_FromLTEList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionsFromLTE[i].PDU_SessionId, 1), -, v_PDU_Type); // @sic R5s200564 sic@ } } } else { for (i := 0; i < N; i := i + 1) { if (p_PduSessionsFromLTE[i].IsIMS) { // @sic R5-206296 sic@ v_PDU_Type := IMS_DNN; } v_FromLTEList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionsFromLTE[i].PDU_SessionId, 1), -, v_PDU_Type); } } ….. }