3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210082

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1516 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR PDCP test case 7.1.3.5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 7-9, PSCell and PCell passed to function f_TC_7_1_3_5_3_NR_TestBody_Step7to9 should be nr_Cell1 instead of nr_Cell10 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changing PSCell and PCell to nr_Cell1 from nr_Cell10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_7_1_3_5_3_ENDC_NR_TestBody() |
| --- | --- |
| Reason for change | At step 7-9, PSCell and PCell passed to function f_TC_7_1_3_5_3_NR_TestBody_Step7to9() should be nr_Cell1 instead of nr_Cell10 |
| Summary of change | Changing PSCell and PCell to nr_Cell1 from nr_Cell10 |
| TTCN module | PDCP_ENDC_NR.ttcn |
| MCC160 Comment | Accepted |

Before change

function fl_TC_7_1_3_5_3_ENDC_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on ENDC_NR_PTC

{

var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get();

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (omit) PDCP_Config v_PDCP_Config;

var template (value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); // @sic R5s191042, R5-198180 sic@

var SubFrameTiming_Type v_Timing;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc;

f_TC_7_1_3_5_3_NR_TestBody_Step1to4A (p_NR_PDCP_State, nr_Cell1);

//@siclog "Step 5-6" siclog@

// The SS transmits a  RRCConnectionReconfiguration message containing NR RRCReconfiguration.

// The UE transmits a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete.

v_PDCP_Config := cs_NR_PDCP_Config_Split_moreThan1RLC;

v_RadioBearerConfig := cs_NR_RbRecoverPDCP(secondary, v_PDCP_Config);

//Prepare CellGroupConfig as per Table 7.1.3.5.3.3.3-2A: CellGroupConfig-PDCP

v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; // @sic R5s191042, R5-198180 sic@

v_SCGConfig.spCellConfig := omit;

//Reestablish DRB2 RLC in SS

v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1);

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}, cs_TimingInfo_NR(v_Timing));

v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5);   // add 5ms

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

tsc_NR_DRB2,

-,

cs_NR_PDCP_RbConfig_TransparentMode)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList, cs_TimingInfo_NR(v_Timing));

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig, -, -, -, noRACH_Check); //@sic draft prose CR 7.1.3.5.3: no HO -> no RACH sic@

f_TC_7_1_3_5_3_NR_TestBody_Step7to9 (p_NR_PDCP_State, nr_Cell10, nr_Cell10);

//Coordination message to trigger the EUTRA Postamble

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}

After change

function fl_TC_7_1_3_5_3_ENDC_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on ENDC_NR_PTC

{

var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get();

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (omit) PDCP_Config v_PDCP_Config;

var template (value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell1); // @sic R5s191042, R5-198180 sic@

var SubFrameTiming_Type v_Timing;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc;

f_TC_7_1_3_5_3_NR_TestBody_Step1to4A (p_NR_PDCP_State, nr_Cell1);

//@siclog "Step 5-6" siclog@

// The SS transmits a  RRCConnectionReconfiguration message containing NR RRCReconfiguration.

// The UE transmits a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete.

v_PDCP_Config := cs_NR_PDCP_Config_Split_moreThan1RLC;

v_RadioBearerConfig := cs_NR_RbRecoverPDCP(secondary, v_PDCP_Config);

//Prepare CellGroupConfig as per Table 7.1.3.5.3.3.3-2A: CellGroupConfig-PDCP

v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; // @sic R5s191042, R5-198180 sic@

v_SCGConfig.spCellConfig := omit;

//Reestablish DRB2 RLC in SS

v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1);

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}, cs_TimingInfo_NR(v_Timing));

v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing, 5);   // add 5ms

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

tsc_NR_DRB2,

-,

cs_NR_PDCP_RbConfig_TransparentMode)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList, cs_TimingInfo_NR(v_Timing));

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig, -, -, -, noRACH_Check); //@sic draft prose CR 7.1.3.5.3: no HO -> no RACH sic@

f_TC_7_1_3_5_3_NR_TestBody_Step7to9 (p_NR_PDCP_State, nr_Cell1, nr_Cell1);

//Coordination message to trigger the EUTRA Postamble

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

}