3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210084

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1518 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle extension procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In function f_NR5GC_RRC_Idle_Extension(), if there is an already existing PDU session, the triggering of a new PDU session (step 1a1) is currently skipped. The resuting signaling (steps 2 to 9a1) should also be skipped as they will not happen without the UT trigger. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If a PDU connection exists, skip step 2-9a1. Note : An associated prose CR on 38.508-1 will be raised at RAN5#90 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail some NR5GC test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.5.NR5GC, 8.1.1.3.4.NR5GC and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_RRC_Idle_Extension |
| --- | --- |
| Reason for change | If there is an already existing PDU session, the triggering of a new PDU session (step 1a1) is currently skipped. The resuting signaling (steps 2 to 9a1) should also be skipped as they will not happen without the UT trigger. |
| Summary of change | At step 1, if PDU connection exists, skip step 2-9a1. Note : An associated prose CR on 38.508-1 will be raised at RAN5#90 |
| TTCN module | Common\NR5GC\NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted subject to prose CR approval. Additional change implemented: removed tsc_RandomAPN from the AT command as this is not specified in the prose and does not conform to 38.508-1 table 4.8.4-1 |

Before Change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ f_UT_RequestPDUSession(UT, 1, tsc_RandomAPN); } } } // Steps 2 - 4 ... ... }

After Change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ f_UT_RequestPDUSession(UT, 1, tsc_RandomAPN); } else { return; //if PDU connection exists, skip step 2-9a1 } } } // Steps 2 - 4 ... ... }