3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210090

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1524 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.10.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Corrections for NR5GC MAC test case 7.1.1.10.1 - DataInactivityTimer is not configured in rrcSetup - SDAP is not configured at SS side, this needs to be configured in order for loopback mode B to operate - The UE will send SR for RLC status PDU before expiry of IP PDU delay (because AM RLC is used) - SDAP needs to be configured on SS for loopback mode B to function properly - v_TimingDL is set incorrectly at step 7, it should be based on the current system time. - Due to possible race conditions withing the SS, SYSTEM_IND for SR and DRB_COMMON_IND may reach TTCN in either order - SS resources are not reset UE moves to RRC_IDLE, leading to the next RRCSetupComplete being out of receive window for SRB1 at step 14 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - configure DataInactivityTimer in cellInfo before configuring cell - configure SDAP at SS. Also remove pasing of DRB input parameter to f_TC_7_1_1_10_1_NR_TestBody() as no longer needed - enable the system indications for for SchedulingRequests later to skip the SR for RLC status PDU, and disable it after step 4/10 - send SDAP SDU with IP data to UE at step 3/9. Also remove DRB ID input parameter. - Refresh the system time at the start of the loop - using interleave to receive SR and DRB data in any order - release local resource at SS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.10.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_10_1_NR5GC |
| --- | --- |
| Reason for change | - DataInactivityTimer is not configured in rrcSetup - SDAP is not configured at SS side, this needs to be configured in order for loopback mode B to operate |
| Summary of change | - configure DataInactivityTimer in cellInfo before configuring cell - configure SDAP at SS. Also remove pasing of DRB input parameter to f_TC_7_1_1_10_1_NR_TestBody() as no longer needed |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | - Accepted - Accepted Removal of v_DRBId not accepted to be consistent with other test cases |

Before Change

function f_TC_7_1_1_10_1_NR5GC() runs on NR5GC_PTC { //DataInactivityTimer expiry var DRB_Identity v_DRBId; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //Init Cell parameters f_NR5GC_Init(NR_1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeB_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)}; //Configure on SS the DRB No SDAP and RLC/MAC/PDCP normal mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeB_ON, -, '06'O, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); // @siclog "Step 1-14" siclog@ f_TC_7_1_1_10_1_NR_TestBody(v_DRBId ); f_NR_TestBody_Set(false); //Postamble f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change

function f_TC_7_1_1_10_1_NR5GC() runs on NR5GC_PTC { //DataInactivityTimer expiry var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //Init Cell parameters f_NR5GC_Init(NR_1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.dataInactivityTimer.setup := s10; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeB_ON); f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeB_ON, -, '06'O); f_NR_TestBody_Set(true); // @siclog "Step 1-14" siclog@ f_TC_7_1_1_10_1_NR_TestBody( ); f_NR_TestBody_Set(false); //Postamble f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 2

| Function name | f_TC_7_1_1_10_1_NR_TestBody |
| --- | --- |
| Reason for change | - The UE will send SR for RLC status PDU before expiry of IP PDU delay (because AM RLC is used) - SDAP needs to be configured on SS for loopback mode B to function properly - v_TimingDL is set incorrectly at step 7, it should be based on the current system time. - Due to possible race conditions withing the SS, SYSTEM_IND for SR and DRB_COMMON_IND may reach TTCN in either order - SS resources are not reset UE moves to RRC_IDLE, leading to the next RRCSetupComplete being out of receive window for SRB1 at step 14 |
| Summary of change | - enable the system indications for for SchedulingRequests later to skip the SR for RLC status PDU, and disable it after step 4/10 - send SDAP SDU with IP data to UE at step 3/9. Also remove DRB ID input parameter. - Refresh the system time at the start of the loop - using interleave to receive SR and DRB data in any order - release local resource at SS |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | - Not needed based on R5-206322 - SDAP SDU is already done. Retained DRB ID to be consistent with other test cases - Already done in latest TTTCN - The SR reception steps have been voided in Latest TTCN based on R5-206322 - Accepted |

Before Change

function f_TC_7_1_1_10_1_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR5GC_PTC { var integer j; var SubFrameTiming_Type v_TimingDL; timer t_Watchdog := 10.0; // Enable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,100); // Timing of DL for (j := 0; j < 2; j:= j+1) { // @siclog "Step 1 - 3, 7-9 " siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 6000); DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //cs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_NR(v_TimingDL), {crs_NR_PDCP_SDU_37B} )); t_Watchdog.start; // @siclog "Step 4, 10" siclog@ SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4, 10"); // @siclog "Step 5-6, 11-12" siclog@ DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, -, {crs_NR_PDCP_SDU_37B})); } //Disable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 13 " siclog@ f_Delay (10.0); //@siclog "Step 14" siclog@ //Check: does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.4 indicate that the UE is in RRC_IDLE? f_NR5GC_CheckRRCIdle(nr_Cell1, noRrcConnectionRelease); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); };

After Change

function f_TC_7_1_1_10_1_NR_TestBody() runs on NR5GC_PTC { var integer j; var SubFrameTiming_Type v_TimingDL; timer t_Watchdog := 10.0; var integer v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); //Get the QosFlow ID information from the DRB ID var octetstring v_IPData; for (j := 0; j < 2; j:= j+1) { // @siclog "Step 1 - 3, 7-9 " siclog@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,100); // Timing of DL v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 6000); v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE(), -, j+1 ); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData}, cs_TimingInfo_NR(v_TimingDL))); t_Watchdog.start; alt { [] t_Watchdog.timeout { // Enable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); } } interleave { [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4, 10"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); } [] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})) { } //@sic R5s201136 sic@ } } //Disable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 13 " siclog@ f_Delay (10.0); //@siclog "Step 14" siclog@ //Check: does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.4 indicate that the UE is in RRC_IDLE? f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); f_NR5GC_CheckRRCIdle(nr_Cell1, noRrcConnectionRelease); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); };