3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210110

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1538 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GSM test case 10.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Following the implementation of MCC comments for R5s201371 change 2, The test case execution can get stuck at step 14 since both expected messages are expected on the same port. If PDU SESSION ESTABLISHMENT REQUEST is received first, the alt step will continually repeat but the message will not be removed from the queue, so it will not be possible to then receive RRCReconfigurationComplete. - For the call to f_NR5GC_PDUSessionEstablishment() at step 20, the parameter p_ExpectedNumberOfNewPDUSessions should explicitly set as 1 to establish 1 PDU session only (regardless of value of pc_noOf_PDUsSameConnection) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the handling of RRCConnectionReconfigurationComplete and PDU SESSION ESTABLISHMENT REQUEST at step 14, and Update the default handler a_NR5GC_AdditionalPDUSessionEstablishmentReq (activated during f_NR5GC_PDUSessionEstablishment()), to absorb any RRCReconfigurationComplete messages that are recieved before or after PDU SESSION ESTABLISHMENT REQUEST - Pass “1” as p ExpectedNumberOfNewPDUSessions in call to to f_NR5GC_PDUSessionEstablishment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_10_1_1_1_Body() |
| --- | --- |
| Reason for change | - Following the implementation of MCC comments for R5s201371 change 2, The test case execution can get stuck at step 14 since both expected messages are expected on the same port. If PDU SESSION ESTABLISHMENT REQUEST is received first, the alt step will continually repeat but the message will not be removed from the queue, so it will not be possible to then receive RRCReconfigurationComplete. - For the call to f_NR5GC_PDUSessionEstablishment() at step 20, the parameter p_ExpectedNumberOfNewPDUSessions should explicitly set as 1 to establish 1 PDU session only (regardless of value of pc_noOf_PDUsSameConnection) |
| Summary of change | - Remove the handling of RRCConnectionReconfigurationComplete and PDU SESSION ESTABLISHMENT REQUEST at step 14 (see also change 2) - pass “1” as p ExpectedNumberOfNewPDUSessions in call to to f_NR5GC_PDUSessionEstablishment |
| TTCN module | GSM_TestCases_NR5GC |
| MCC160 Comment | 1.Accepted but implemented differemtly. 2. Accepted |

Before Change

function fl_TC_10_1_1_1_Body() runs on NR5GC_PTC { ……………….. //@siclog "Steps 13" siclog@ f_NR_RRCReconfigExistingDRBs (v_NGC_NASCellA, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus))}, -, -, -, -, -, false); // @sic R5s201371 sic@ alt { // @sic R5s201371 sic@ [] SRB.receive(car_NR_SRB_RrcPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)){} [] SRB.check(receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered)))) {repeat;} } //@siclog "Step 14 - 19a1" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, -, cs_SDAP_Config(-,-,{3}), // @sic R5s201371 sic@ -, -, -, v_DNN_1, // @sic R5s201371 sic@ cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(3,6), '00'O)}), // @sic R5s201412 sic@ -, -, cs_QoSFlowDescr ({cs_QoS_Flow ('000011'B, { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)})}), -,-,-,-,-,-, -,-,-,-,-,-,-,-,-, cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success))) , -, true); //@siclog "Step 20" siclog@ // SS releases the rrc connection f_NR_RRCRelease(v_NGC_NASCellA); }

After Change

function fl_TC_10_1_1_1_Body() runs on NR5GC_PTC { ……………. //@siclog "Steps 13" siclog@ f_NR_RRCReconfigExistingDRBs (v_NGC_NASCellA, {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus))}, -, -, -, -, -, false); // @sic R5s201371 sic@ /* REMOVED alt { // @sic R5s201371 sic@ [] SRB.receive(car_NR_SRB_RrcPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)){} [] SRB.check(receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered)))) {repeat;} }*/ //@siclog "Step 14 - 19a1" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, cs_SDAP_Config(-,-,{3}), // @sic R5s201371 sic@ -, -, -, v_DNN_1, // @sic R5s201371 sic@ cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(3,6), '00'O)}), // @sic R5s201412 sic@ -, -, cs_QoSFlowDescr ({cs_QoS_Flow ('000011'B, { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)})}), -,-,-,-,-,-, -,-,-,-,-,-,-,-,-, cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success))) , -, true); //@siclog "Step 20" siclog@ // SS releases the rrc connection f_NR_RRCRelease(v_NGC_NASCellA); }

## Change 2

| Function name | altstep a_NR5GC_AdditionalPDUSessionEstablishmentReq |
| --- | --- |
| Reason for change | Following the implementation of MCC comments for R5s201371 change 2, The test case execution can get stuck at step 14 since both expected messages are expected on the same port. If PDU SESSION ESTABLISHMENT REQUEST is received first, the alt step will continually repeat but the message will not be removed from the queue, so it will not be possible to then receive RRCReconfigurationComplete. |
| Summary of change | Update the default handler a_NR5GC_AdditionalPDUSessionEstablishmentReq (activated during f_NR5GC_PDUSessionEstablishment()), to absorb any RRCReconfigurationComplete messages that are recieved before or after PDU SESSION ESTABLISHMENT REQUEST Note : The RRCReconfigurationComplete message could come in any scenario where SS has sent SERVICE ACCEPT with RRCReconfiguration before entering f_NR5GC_PDUSessionEstablishment() Note : This change was originally proposed as change 2 of R5s201371 |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Rejected. In this situation of 10.1.1.1, at Step 13 the rrcReconfiguration message with piggyback Service accept is sent on SRB1, but also configures SRB2. Thus, the rrcReconfigurationComplete which is sent on SRB1 will always come before PDU Session Establishment REQ which will be received on SRB2. SRB1 is a higher priority that SRB2. |

Before change:

altstep a_NR5GC_AdditionalPDUSessionEstablishmentReq(NR_CellId_Type p_CellId) runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_CellId, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

f_NR5GC_MsgInDefault_StorePDUSessionEstReq (v_ReceivedAsp);

repeat;

}

}

After change:

altstep a_NR5GC_AdditionalPDUSessionEstablishmentReq(NR_CellId_Type p_CellId) runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_CellId, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

f_NR5GC_MsgInDefault_StorePDUSessionEstReq (v_ReceivedAsp);

repeat;

}

[] SRB.receive(car_NR_SRB_RrcPdu_IND(p_CellId,tsc_NR_RbId_SRB1 , cr_38508_RRCReconfigurationComplete))

{

repeat;

}

}

MCC160 Implementation:

## … ## //@siclog "Steps 13" siclog@ ## f_NR_RRCReconfigExistingDRBs (v_NGC_NASCellA, ## {cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, ## f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus))}, ## -, -, -, -, -, ## true); // @sic R5s201371 sic@ ## //@siclog "Step 14 - 19a1" siclog@ ## f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, ## 1,-, ## cs_SDAP_Config(-,-,{3}), // @sic R5s201371 sic@ ## -, -, -, ## v_DNN_1, // @sic R5s201371 sic@ ## cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(3,6), '00'O)}), // @sic R5s201412 sic@ ## -, -, ## cs_QoSFlowDescr ({cs_QoS_Flow ('000011'B, { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)})}), ## -,-,-,-,-,-, ## -,-,-,-,-,-,-,-,-, ## cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success))) , -, true); ## …