3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210111

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1539 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current implementation, at Step 7 when the SS transmits the EAP challenge REQ, the function f_NG_EAP_ChallengeReq (v_NAS_SecurityParams, v_PLMN,'01'O) is called. In this function, instead of passing v_NAS_SecurityParams directly, the function f_NG_EAP_AuthenticationInit(v_NAS_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)) should be passed. Otherwise, the function f_Get_EAP_OtherAuthenticationReqMsgf_NG_EAP_AuthenticationInit will also use the same value of p_SecurityParams, and this will cause the UE to reject the EAP challenge. In Step 8, the SS expects the response with p_ID set as ‘00’O, whereas the Authentication EQ in Step 7 used ‘01’O. This also needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At Step 7, instead of passing v_NAS_SecurityParams directly, the function f_NG_EAP_AuthenticationInit(v_NAS_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)) is passed. At Step 8, passed p_ID as ‘01’O. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	function fl_TC_9_1_1_2_TestBody	4

# Corrections required

## function fl_TC_9_1_1_2_TestBody

| Function name | fl_TC_9_1_1_2_TestBody |
| --- | --- |
| Reason for change | In the current implementation, at Step 7 when the SS transmits the EAP challenge REQ, the function f_NG_EAP_ChallengeReq (v_NAS_SecurityParams, v_PLMN,'01'O) is called. In this function, instead of passing v_NAS_SecurityParams directly, the function f_NG_EAP_AuthenticationInit(v_NAS_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)) should be passed. Otherwise, the function f_Get_EAP_OtherAuthenticationReqMsgf_NG_EAP_AuthenticationInit will also use the same value of p_SecurityParams, and this will cause the UE to reject the EAP challenge. In Step 8, the SS expects the response with p_ID set as ‘00’O, whereas the Authentication EQ in Step 7 used ‘01’O. This also needs to be corrected. |
| Summary of change | At Step 7, instead of passing v_NAS_SecurityParams directly, the function f_NG_EAP_AuthenticationInit(v_NAS_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)) is passed. At Step 8, passed p_ID as ‘01’O. Please see below. |
| TTCN module | NR5GC_IRAT/Common/ EUTRA38_NR5GCProcedures.ttcn |
| MCC160 Comment | 1. Accepted in principle but moved the call to f_NG_EAP_AuthenticationInit and replaced the call to f_Get_EAP_OtherAuthenticationReqMsg with the template cs_NG_AUTHENTICATION_REQUEST 2. Accepted |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 7" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_OtherAuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, f_NG_EAP_ChallengeReq (v_NAS_SecurityParams, v_PLMN, '01'O))))); // @sic R5s201483 sic@ //@siclog "Step 8" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAPAKA_AuthenticationResponse(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg)) { // @sic R5-205579 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { // @sic R5s-201483 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 7" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_EAP_OtherAuthenticationReqMsg(v_NAS_SecurityParams, v_PLMN, f_NG_EAP_ChallengeReq (f_NG_EAP_AuthenticationInit(v_NAS_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)),//WA#9_1_1_2 v_NAS_SecurityParams, v_PLMN, '01'O))))); // @sic R5s201483 sic@ //@siclog "Step 8" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (f_Check_EAPAKA_AuthenticationResponse(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, -, '01'O/*WA#9_1_1_2*/)) { // @sic R5-205579 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { // @sic R5s-201483 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } <<SKIPPED CODE>>

## MCC160 Implementation

//@siclog "Step 7" siclog@

v_NAS_SecurityParams := f_NG_EAP_AuthenticationInit(v_NAS_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)); // @sic R5s210111 sic@

v_EAPMsg := f_NG_EAP_ChallengeReq (v_NAS_SecurityParams, v_PLMN, '01'O);

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA,

tsc_NR_RbId_SRB1,

-,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_NG_AUTHENTICATION_REQUEST (v_NAS_SecurityParams.KSIamf,

cs_ABBA (v_NAS_SecurityParams.ABBA, omit),

-,

-,

cs_EAP_Message(v_EAPMsg))))); // @sic R5s201483 sic@

//@siclog "Step 8" siclog@

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (f_Check_EAPAKA_AuthenticationResponse(v_NAS_SecurityParams, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, -, '01'O)) {  // @sic R5-205579 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

} else { // @sic R5s201483 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8");

}