3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210123

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1545 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 9.1.5.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Cell I is configured as Suitable neighbour cell at step 11A and not turned off till the end of test case. When UE is powered on and user selects PLMN at step 17, as both Cell E and Cell I of same PLMN are available, UE may select Cell I instead of Cell E causing test case failure. Similar issue during preamble is fixed with R5-205866 in RAN5#89-e. From 38.304, as this is cell selection process, UE may perform it by leveraging stored information of frequencies, previously received measurement control information elements or from previously detected cells. Due to this UE can sometimes camp on Cell I instead of Cell E during preamble causing test case failure. #draft ProseCR would be raised for RAN5#90 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell I is configured as Non-suitable Off cell at step 15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

## 2.1 Change 1

| Function name | fl_TC_9_1_5_1_8_TestBody() |
| --- | --- |
| Reason for change | Cell I is configured as Suitable neighbour cell at step 11A and not turned off till the end of test case. When UE is powered on and user selects PLMN at step 17, as both Cell E and Cell I of same PLMN are available, UE may select Cell I instead of Cell E causing test case failure. Similar issue during preamble is fixed with R5-205866 in RAN5#89-e. From 38.304, as this is cell selection process, UE may perform it by leveraging stored information of frequencies, previously received measurement control information elements or from previously detected cells. Due to this UE can sometimes camp on Cell I instead of Cell E during preamble causing test case failure. #draft ProseCR would be raised for RAN5#90 meeting |
| Summary of change | Cell I is configured as Non-suitable Off cell at step 15. |
| TTCN module | Initial_Registration_NR5GC |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#90-e. |

Before change:

function fl_TC_9_1_5_1_8_TestBody() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellI := f_NR5GC_MapNASCell(ngc_CellI, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN);

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellE);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

timer t_Wait := 30.0;

//The UE is switched on

//@siclog "Step 1" siclog@

f_UT_SwitchOnUE(UT);

//Steps 2-9 of Table 4.5.2.2-2 in TS38.508-1 [4] are performed on NGC Cell E.

//@siclog "Step 2-9" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellE, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, v_NGC_NASCellE, v_ReceivedMsg, v_PLMN, Initial_Secure);

//The SS transmits a REGISTRATION REJECT with cause #73 (Serving network not authorized).

//@siclog "Step 10" siclog@

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellE,//@sic R5s200154 sic@

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_ServNetNotAuthorized)))));

//The SS releases RRC connection.

//@siclog "Step 11" siclog@

f_NR_RRCRelease(v_NGC_NASCellE);

//@siclog "Step 11A" siclog@

f_NR_SetCellPower (v_NGC_NASCellI, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1);//@sic R5-205866 sic@

//@siclog "Step 12" siclog@

//Check: Does the UE send in the next 30 sec a request for RRC connection establishment on Cell E or Cell I.

t_Wait.start;

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellE, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))

{

t_Wait.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received");

}

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellI, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))

{

t_Wait.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received");

}

[] t_Wait.timeout {}

}

//@siclog "Step 13" siclog@

//The SS configures-NGC Cell A as "Serving Cell"?

f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog "Step 14" siclog@

//Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2?

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);//@sic R5 200154 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

else {

f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 14");

}

//Steps 5-20 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed.

f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo,

v_NGC_NASCellA,

v_ReceivedMsg,

TESTMode_OFF);

//@siclog "Step 14A" siclog@//@sic R5-203413, R5s201113 sic@

// Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed.

f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_IDLE_1A);

//@siclog "Step 15" siclog@

//The SS configures-NGC Cell A as "Non-Suitable "Off" cell"?

f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-194879, R5-203413, R5s201113 sic@

//@siclog "Step 15A" siclog@//@sic R5-203413, R5s201113 sic@

// The UE is brought back to operation or the USIM is inserted. The UE is powered on or switched on.

f_UT_SwitchOnUE(UT);

//@siclog "Step 16" siclog@

//Set the UE in manual PLMN selection mode or request a PLMN search.

//@siclog "Step 17" siclog@

//The user selects the PLMN of NGC Cell E.

f_UT_ManualPLMN_Select (UT, v_PLMN);

//@siclog "Step 18" siclog@

//The UE performs Registration procedure on NGC Cell E as specified in TS 38.508-1 [4] subclause 4.5.2 with 'connected without release'.

f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellE);//@sic R5-200102, R5s200154 sic@

//@siclog "Step 19" siclog@

//Set the UE in Automatic PLMN selection mode.

f_UT_AutomaticPLMN_Select(UT);

//wait 300 seconds to let UE complete automatic PLMN search

f_Delay(300.0);

} //End of test body of TC_9_1_5_1_8

After change:

function fl_TC_9_1_5_1_8_TestBody() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellI := f_NR5GC_MapNASCell(ngc_CellI, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN);

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellE);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

timer t_Wait := 30.0;

//The UE is switched on

//@siclog "Step 1" siclog@

f_UT_SwitchOnUE(UT);

//Steps 2-9 of Table 4.5.2.2-2 in TS38.508-1 [4] are performed on NGC Cell E.

//@siclog "Step 2-9" siclog@

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellE, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_9_AKA (v_GMM_MobilityInfo, v_NGC_NASCellE, v_ReceivedMsg, v_PLMN, Initial_Secure);

//The SS transmits a REGISTRATION REJECT with cause #73 (Serving network not authorized).

//@siclog "Step 10" siclog@

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellE,//@sic R5s200154 sic@

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_ServNetNotAuthorized)))));

//The SS releases RRC connection.

//@siclog "Step 11" siclog@

f_NR_RRCRelease(v_NGC_NASCellE);

//@siclog "Step 11A" siclog@

f_NR_SetCellPower (v_NGC_NASCellI, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1);//@sic R5-205866 sic@

//@siclog "Step 12" siclog@

//Check: Does the UE send in the next 30 sec a request for RRC connection establishment on Cell E or Cell I.

t_Wait.start;

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellE, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))

{

t_Wait.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received");

}

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(v_NGC_NASCellI, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))

{

t_Wait.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 12: RRCSetupRequest received");

}

[] t_Wait.timeout {}

}

//@siclog "Step 13" siclog@

//The SS configures-NGC Cell A as "Serving Cell"?

f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog "Step 14" siclog@

//Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2?

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected);//@sic R5 200154 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

else {

f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 14");

}

//Steps 5-20 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed.

f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo,

v_NGC_NASCellA,

v_ReceivedMsg,

TESTMode_OFF);

//@siclog "Step 14A" siclog@//@sic R5-203413, R5s201113 sic@

// Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed.

f_NR_UE_DeRegisterOnSwitchOff (v_NGC_NASCellA, STATE_IDLE_1A);

//@siclog "Step 15" siclog@

//The SS configures-NGC Cell A and Cell I as "Non-Suitable "Off" cell"?

f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-194879, R5-203413, R5s201113 sic@

f_NR_SetCellPower (v_NGC_NASCellI, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

//@siclog "Step 15A" siclog@//@sic R5-203413, R5s201113 sic@

// The UE is brought back to operation or the USIM is inserted. The UE is powered on or switched on.

f_UT_SwitchOnUE(UT);

//@siclog "Step 16" siclog@

//Set the UE in manual PLMN selection mode or request a PLMN search.

//@siclog "Step 17" siclog@

//The user selects the PLMN of NGC Cell E.

f_UT_ManualPLMN_Select (UT, v_PLMN);

//@siclog "Step 18" siclog@

//The UE performs Registration procedure on NGC Cell E as specified in TS 38.508-1 [4] subclause 4.5.2 with 'connected without release'.

f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellE);//@sic R5-200102, R5s200154 sic@

//@siclog "Step 19" siclog@

//Set the UE in Automatic PLMN selection mode.

f_UT_AutomaticPLMN_Select(UT);

//wait 300 seconds to let UE complete automatic PLMN search

f_Delay(300.0);

}

} //End of test body of TC_9_1_5_1_8