3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210124

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4581 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS default handler |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IMS PTC variables are currently not being reset when requested from other PTC, this can cause issues processing subsequent IMS registrations (NOTIFY message is observed to be sent unencrypted) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add call to f_IMS_PTC_Reset() for p_IPCAN_IMS_Control.Reset |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 11.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_IMS_DefaultHandler_ChangeInfo |
| --- | --- |
| Reason for change | IMS PTC variables are currently not being reset when requested from other PTC, this can cause issues processing subsequent IMS registrations (NOTIFY message is observed to be sent unencrypted) |
| Summary of change | Add call to f_IMS_PTC_Reset() for p_IPCAN_IMS_Control.Reset |
| TTCN module | IMS_DefaultHandler.ttcn |
| MCC160 Comment | Accepted in principle. Used f_IMS_GlobalRelease() instead of f_IMS_PTC_Reset() |

Before change

function fl_IMS_DefaultHandler_ChangeInfo(IMS_DefaultHandlerInfo_Type p_DefaultHandlerInfo,

IMS_IpcanPortType_Type p_IpcanType,

IPCAN_IMS_Control_Type p_IPCAN_IMS_Control) runs on IMS_PTC return IMS_DefaultHandlerInfo_Type

{

var IMS_DefaultHandlerInfo_Type v_DefaultHandlerInfo := p_DefaultHandlerInfo;

v_DefaultHandlerInfo.IpcanType := p_IpcanType;  /* IPCAN_E or OtherIPCAN_E */

if (ischosen(p_IPCAN_IMS_Control.Reset)) {

v_DefaultHandlerInfo.State := WaitForRegistration;

}

else if (ischosen(p_IPCAN_IMS_Control.IpcanInd)) {

/* nothing else to be done */

}

else if (ischosen(p_IPCAN_IMS_Control.ReregistrationStart)) {

f_IMS_PTC_SetRanType(p_IPCAN_IMS_Control.ReregistrationStart);

v_DefaultHandlerInfo.State := WaitForReregistration;

}

else if (ischosen(p_IPCAN_IMS_Control.ReregistrationStop)) {

if (v_DefaultHandlerInfo.State == WaitForReregistration) {

v_DefaultHandlerInfo.State := WaitForDeregistration;

}

}

else if (ischosen(p_IPCAN_IMS_Control.NoRegistration)) {    // @sic R5-169150 sic@

v_DefaultHandlerInfo.State := NoRegistration;

}

return v_DefaultHandlerInfo;

}

After change

function fl_IMS_DefaultHandler_ChangeInfo(IMS_DefaultHandlerInfo_Type p_DefaultHandlerInfo,

IMS_IpcanPortType_Type p_IpcanType,

IPCAN_IMS_Control_Type p_IPCAN_IMS_Control) runs on IMS_PTC return IMS_DefaultHandlerInfo_Type

{

var IMS_DefaultHandlerInfo_Type v_DefaultHandlerInfo := p_DefaultHandlerInfo;

v_DefaultHandlerInfo.IpcanType := p_IpcanType;  /* IPCAN_E or OtherIPCAN_E */

if (ischosen(p_IPCAN_IMS_Control.Reset)) {

v_DefaultHandlerInfo.State := WaitForRegistration;

f_IMS_PTC_Reset();

}

else if (ischosen(p_IPCAN_IMS_Control.IpcanInd)) {

/* nothing else to be done */

}

else if (ischosen(p_IPCAN_IMS_Control.ReregistrationStart)) {

f_IMS_PTC_SetRanType(p_IPCAN_IMS_Control.ReregistrationStart);

v_DefaultHandlerInfo.State := WaitForReregistration;

}

else if (ischosen(p_IPCAN_IMS_Control.ReregistrationStop)) {

if (v_DefaultHandlerInfo.State == WaitForReregistration) {

v_DefaultHandlerInfo.State := WaitForDeregistration;

}

}

else if (ischosen(p_IPCAN_IMS_Control.NoRegistration)) {    // @sic R5-169150 sic@

v_DefaultHandlerInfo.State := NoRegistration;

}

return v_DefaultHandlerInfo;