3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210125

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4582 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EUTRA RRC test case 8.2.4.13a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following the change of CR R5s201655: “v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9 and v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9 are interchanged in the function f_EUTRA_RRCConnectionReconfiguration_MeasConfig()” This change causes the message to be rejected for bands < = 64 where dl_CarrierFreq_r9 is not used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Only exchange v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9 and v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9 in the case of primary and secondary bands > 64 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.13a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_RRCConnectionReconfiguration_MeasConfig() |
| --- | --- |
| Reason for change | Following the change of CR R5s201655: “v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9 and v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9 are interchanged in the function f_EUTRA_RRCConnectionReconfiguration_MeasConfig()” This change causes the message to be rejected for bands < = 64 where dl_CarrierFreq_r9 is not used. |
| Summary of change | Only exchange v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9 and v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9 in the case of primary and secondary bands > 64 |
| TTCN module | RRC_Handover.ttcn |
| MCC160 Comment | Accepted in principle. MCC160 Implementation see below |

Before change

……………….

f_EUTRA_508RRC_IntraLTE_HO_InterCell_Common_Steps13_14(eutra_Cell1); // @sic R5s151039 sic@

//@siclog "Step 7" siclog@

//Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 28?

f_EUTRA_508Check_ConnectedState (eutra_Cell28);

//@siclog "Step 8" siclog@

//The SS transmits an RRCConnectionReconfiguration message on Cell 28 to setup inter band measurement.

//@siclog "Step 9" siclog@

//The UE transmits an RRCConnectionReconfigurationComplete message on Cell 28.

if (px_ePrimaryFrequencyBand>64 and px_eSecondaryFrequencyBand>64)

{

f_EUTRA_RRCConnectionReconfiguration_MeasConfig (eutra_Cell28, //@sic R5s130649 sic@

cs_508_RRCConnectionReconfiguration_ConditionMEAS (v_RRC_TI,

cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8(  //@sic R5-133693, R5s201655 sic@

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell28.AllowedMeasBandwidth,

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9,

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9)))); // @sic R5-171995, R5s201655 sic@

}

else

{

f_EUTRA_RRCConnectionReconfiguration_MeasConfig (eutra_Cell28, //@sic R5s130649 sic@

cs_508_RRCConnectionReconfiguration_ConditionMEAS (v_RRC_TI,

cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8(  //@sic R5-133693 sic@

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell28.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9)))); // @sic R5-171995 sic@

}

//@siclog "Step 10" siclog@

//The SS changes Cell 1 and Cell 28 power levels according to the row "T2" in table 8.2.4.13a.3.2-1.

f_EUTRA_SetCellPowerList (v_CellPowerList_AtT2);

…………….

After change

…………….

f_EUTRA_508RRC_IntraLTE_HO_InterCell_Common_Steps13_14(eutra_Cell1); // @sic R5s151039 sic@

//@siclog "Step 7" siclog@

//Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 28?

f_EUTRA_508Check_ConnectedState (eutra_Cell28);

//@siclog "Step 8" siclog@

//The SS transmits an RRCConnectionReconfiguration message on Cell 28 to setup inter band measurement.

//@siclog "Step 9" siclog@

//The UE transmits an RRCConnectionReconfigurationComplete message on Cell 28.

if (px_ePrimaryFrequencyBand>64 and px_eSecondaryFrequencyBand>64)

{

f_EUTRA_RRCConnectionReconfiguration_MeasConfig (eutra_Cell28, //@sic R5s130649 sic@

cs_508_RRCConnectionReconfiguration_ConditionMEAS (v_RRC_TI,

cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8(  //@sic R5-133693, R5s201655 sic@

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell28.AllowedMeasBandwidth,

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9,

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9)))); // @sic R5-171995, R5s201655 sic@

}

else

{

f_EUTRA_RRCConnectionReconfiguration_MeasConfig (eutra_Cell28, //@sic R5s130649 sic@

cs_508_RRCConnectionReconfiguration_ConditionMEAS (v_RRC_TI,

cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8(  //@sic R5-133693 sic@

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell28.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9)))); // @sic R5-171995 sic@

}

//@siclog "Step 10" siclog@

//The SS changes Cell 1 and Cell 28 power levels according to the row "T2" in table 8.2.4.13a.3.2-1.

f_EUTRA_SetCellPowerList (v_CellPowerList_AtT2);

MCC160 Implementation

The issue stems from the fact the functions feeding the measured object lists are not using the parameters in the same order. This is solved by changing template cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8 to use the parameters in the same order as they have to be put into the measured object list.

cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8

template (value) MeasConfig cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8(ARFCN_ValueEUTRA p_EUTRA_DL_CarrierFreq1,

AllowedMeasBandwidth p_MeasurementBandwidth1,

ARFCN_ValueEUTRA p_EUTRA_DL_CarrierFreq2,

AllowedMeasBandwidth p_MeasurementBandwidth2,

template (omit) MeasGapConfig p_MeasGapConfig := omit,

template (omit) MeasObjectToAddModList_v9e0 p_MeasObjectToAddModList_v9e0 := omit) :=  //@sic R5-133693 sic@ @sic R5-171995 sic@

/* @status    APPROVED (LTE) */

cs_MeasConfig(omit,

cs_MeasObjectToAddModList_2Entries(tsc_IdMeasObject_f1,

cs_MeasObject_measObjectEUTRA_Common(p_EUTRA_DL_CarrierFreq1, p_MeasurementBandwidth1),

tsc_IdMeasObject_f6,

cs_MeasObject_measObjectEUTRA_Common(p_EUTRA_DL_CarrierFreq2, p_MeasurementBandwidth2)),

omit,

cs_ReportConfigToAddModList_1Entry(tsc_IdReportConfig_A3, cs_508_ReportConfigEUTRA_A3),

omit,

cs_MeasIdToAddModList_1Entry( 1, tsc_IdMeasObject_f6, tsc_IdReportConfig_A3 ),

-,

p_MeasGapConfig,

-,

-,

-,

p_MeasObjectToAddModList_v9e0); // @sic R5-171995 sic@

Step8

……………….

//@siclog "Step 8" siclog@

//The SS transmits an RRCConnectionReconfiguration message on Cell 28 to setup inter band measurement.

//@siclog "Step 9" siclog@

//The UE transmits an RRCConnectionReconfigurationComplete message on Cell 28.

f_EUTRA_RRCConnectionReconfiguration_MeasConfig (eutra_Cell28, //@sic R5s130649 sic@

cs_508_RRCConnectionReconfiguration_ConditionMEAS (v_RRC_TI,

cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8(  //@sic R5-133693, R5s201655 sic@

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell28.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9)))); // @sic R5-171995, R5s201655 sic@

…………….