3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210129

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1547 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_GetSSB_ToMeasure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current ttcn for MediumBitmap is not according to the contidions in 38.508 (Table 4.6.3-187: SSB-ToMeasure) and cause incorrect contidion for FR2 cases FR1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update TTCN for MediumBitmap according to the condition FR1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | FR2 measurement testcases ( 8.2.3.x.1.ENDC) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR_GetSSB_ToMeasure |
| --- | --- |
| Reason for change | The current ttcn for MediumBitmap is not according to the contidions in 38.508 (Table 4.6.3-187: SSB-ToMeasure) and cause incorrect contidion for FR2 cases FR1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| Summary of change | Update TTCN for MediumBitmap according to the condition FR1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| TTCN module | NR_SysInfo_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR_GetSSB_ToMeasure(FreqBandIndicatorNR p_FrequencyBandIndicator, SubcarrierSpacing p_SCS) return template (value) SSB_ToMeasure { // @sic R5-204395 R5-2066301 sic@ var template (value) SSB_ToMeasure v_SSB_ToMeasure; var boolean v_TDD := not f_NR_IsBandFDD((p_FrequencyBandIndicator)); var boolean v_FDD := f_NR_IsBandFDD(p_FrequencyBandIndicator); var boolean v_FR1 := f_NR_IsBandFR1(p_FrequencyBandIndicator); if (v_FR1 and (not(f_NR_IsBAndBelow2_3GHz(p_FrequencyBandIndicator)) and f_NR_IsBAndBelow3GHz(p_FrequencyBandIndicator) and (v_FDD or (v_TDD and p_SCS == kHz15))) or f_NR_IsBAndBelow2_3GHz(p_FrequencyBandIndicator)){ //FR1 AND (2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHz) v_SSB_ToMeasure := {shortBitmap := '1100'B}; } else if (v_FR1 and (not(f_NR_IsBAndBelow2_3GHz(p_FrequencyBandIndicator)) and f_NR_IsBAndBelow3GHz(p_FrequencyBandIndicator) and (v_TDD and p_SCS == kHz30)) or not(f_NR_IsBAndBelow3GHz(p_FrequencyBandIndicator))) { //FR1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) //@sic R5s201355 sic@ v_SSB_ToMeasure := {mediumBitmap := '11000000'B}; } else { // FR2 v_SSB_ToMeasure := {longBitmap := '1100000000000000000000000000000000000000000000000000000000000000'B}; } return v_SSB_ToMeasure; }

After Change:

function f_NR_GetSSB_ToMeasure(FreqBandIndicatorNR p_FrequencyBandIndicator, SubcarrierSpacing p_SCS) return template (value) SSB_ToMeasure { // @sic R5-204395 R5-2066301 sic@ var template (value) SSB_ToMeasure v_SSB_ToMeasure; var boolean v_TDD := not f_NR_IsBandFDD((p_FrequencyBandIndicator)); var boolean v_FDD := f_NR_IsBandFDD(p_FrequencyBandIndicator); var boolean v_FR1 := f_NR_IsBandFR1(p_FrequencyBandIndicator); if (v_FR1 and (not(f_NR_IsBAndBelow2_3GHz(p_FrequencyBandIndicator)) and f_NR_IsBAndBelow3GHz(p_FrequencyBandIndicator) and (v_FDD or (v_TDD and p_SCS == kHz15))) or f_NR_IsBAndBelow2_3GHz(p_FrequencyBandIndicator)){ //FR1 AND (2.3GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=2.3GHz) v_SSB_ToMeasure := {shortBitmap := '1100'B}; } else if (v_FR1 and ((not(f_NR_IsBAndBelow2_3GHz(p_FrequencyBandIndicator)) and f_NR_IsBAndBelow3GHz(p_FrequencyBandIndicator) and (v_TDD and p_SCS == kHz30)) or not(f_NR_IsBAndBelow3GHz(p_FrequencyBandIndicator)))) { //FR1 AND (2.3GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) //@sic R5s201355 sic@ v_SSB_ToMeasure := {mediumBitmap := '11000000'B}; } else { // FR2 v_SSB_ToMeasure := {longBitmap := '1100000000000000000000000000000000000000000000000000000000000000'B}; } return v_SSB_ToMeasure; }