3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210135

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1551 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 8.1.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Steps 15-16 of test case 8.1.4.1.2, current TTCN implementation is: - Receive RRC Reconfiguration Complete for Step 15 - Execute f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 - Receive Registration Request for Step 16 UE can send Step 16 Registration Request 20ms after Step 15 RRCReconfigurationComplete. However, function f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 takes a lot of time to finish. For instance, the function f_NR_InitialiseRachProcedure takes around 250ms to finish. Main cause of delay is the f_NR_38508_RRCSetup which takes around 200ms to finish. It is suggested to move execution of f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 after receiving Registration Request for Step 16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 after receiving Registration Request for Step 16. - Declare new function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10. - Declare new function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_1_4_1_2_TestBody |
| --- | --- |
| Reason for change | For Steps 15-16 of test case 8.1.4.1.2, current TTCN implementation is: - Receive RRC Reconfiguration Complete for Step 15 - Execute f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 - Receive Registration Request for Step 16 UE can send Step 16 Registration Request 20ms after Step 15 RRCReconfigurationComplete. However, function f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 takes a lot of time to finish. For instance, the function f_NR_InitialiseRachProcedure takes around 250ms to finish. Main cause of delay is the f_NR_38508_RRCSetup which takes around 200ms to finish. It is suggested to move execution of f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 after receiving Registration Request for Step 16. |
| Summary of change | Move f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 after receiving Registration Request for Step 16. |
| TTCN module | RRC_IntraNR_Handover_NR5GC |
| MCC160 Comment | Accepted as improvement, in HO procedure step 11 (Starting periodic TA) and step 12 (Informing SS about completion of HO) are important steps for the good functioning of the procedure. So we propose to split at step 13 (Re-configure RACH procedure). See MCC160 proposed implementation below |

Before change:

function fl_TC_8_1_4_1_2_TestBody() runs on NR5GC_PTC

{

…

//@siclog "Steps 14-15" siclog@

//The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3

//Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3?

v_SIB1 := f_NR_GetSIB1(nr_Cell3);

v_SIB1.cellAccessRelatedInfo.plmn_IdentityList[0].trackingAreaCode := int2bit(2,24);

f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@

//@sic R5s201303 sic@

//To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split

f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_14(nr_Cell1,

nr_Cell3,

-,

false, // RBConfig_KeyChange := false

-,

v_SIB1); // SIB1

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

//@siclog "Step 16" siclog@

//Check: Does UE transmit a REGISTRATION REQUEST message to update the registration of the actual tracking area?

//This message will be received in SRB2, as this SRB was established after the HO

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell3, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s200309 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility, ?, -,-,-,-,-,-, *)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

else {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

}

//@sic R5s201303 sic@

//Final clean up of the target cell

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps15_16(nr_Cell1);

//@siclog "Steps 17-18" siclog@

//SS responds with REGISTRATION ACCEPT message

//The UE send a REGISTRATION COMPLETE message

//@sic R5s200309 sic@

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3,

v_GMM_MobilityInfo,

-, -,

noRrcConnectionRelease,

-,

tsc_NR_RbId_SRB2); // On SRB2

//@siclog "Step 19" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-204450 sic@

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

f_NR5GC_CheckDataPath(nr_Cell3,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 19");

}// End of fl_TC_8_1_4_1_2_TestBody

After change:

function fl_TC_8_1_4_1_2_TestBody() runs on NR5GC_PTC

{

…

//@siclog "Steps 14-15" siclog@

//The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3

//Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3?

v_SIB1 := f_NR_GetSIB1(nr_Cell3);

v_SIB1.cellAccessRelatedInfo.plmn_IdentityList[0].trackingAreaCode := int2bit(2,24);

f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@

//@sic R5s201303 sic@

//To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split

f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10(nr_Cell1,

nr_Cell3,

-,

false, // RBConfig_KeyChange := false

-,

v_SIB1); // SIB1

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

//@siclog "Step 16" siclog@

//Check: Does UE transmit a REGISTRATION REQUEST message to update the registration of the actual tracking area?

//This message will be received in SRB2, as this SRB was established after the HO

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell3, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s200309 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility, ?, -,-,-,-,-,-, *)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

else {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

}

//@sic R5s201303 sic@

//Final clean up of the target cell

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14(nr_Cell1, nr_Cell3);

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps15_16(nr_Cell1);

//@siclog "Steps 17-18" siclog@

//SS responds with REGISTRATION ACCEPT message

//The UE send a REGISTRATION COMPLETE message

//@sic R5s200309 sic@

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3,

v_GMM_MobilityInfo,

-, -,

noRrcConnectionRelease,

-,

tsc_NR_RbId_SRB2); // On SRB2

//@siclog "Step 19" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-204450 sic@

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

f_NR5GC_CheckDataPath(nr_Cell3,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 19");

}// End of fl_TC_8_1_4_1_2_TestBody

MCC160 proposed implementation

- The change done in fl_TC_8_1_4_1_2_TestBody  is also done in fl_TC_9_1_6_1_2_TestBody and fl_TC_9_1_6_1_3_TestBody

function fl_TC_8_1_4_1_2_TestBody() runs on NR5GC_PTC

{

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0;

…

//@siclog "Step 13" siclog@

//SS adjusts the cell-specific reference signal level according to row "T3".

f_NR_SetCellPowerList (v_CellPowerList_AtT3);

//@siclog "Steps 14-15" siclog@

//The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3

//Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3?

v_SIB1 := f_NR_GetSIB1(nr_Cell3);

v_SIB1.cellAccessRelatedInfo.plmn_IdentityList[0].trackingAreaCode := int2bit(2,24);

f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@

//@sic R5s201303 R5s210135 sic@

//To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split

f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12(nr_Cell1,

nr_Cell3,

-,

false, // RBConfig_KeyChange := false

-,

v_SIB1); // SIB1

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

//@siclog "Step 16" siclog@

//Check: Does UE transmit a REGISTRATION REQUEST message to update the registration of the actual tracking area?

//This message will be received in SRB2, as this SRB was established after the HO

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell3, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s200309 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility, ?, -,-,-,-,-,-, *)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

else {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

}

//@sic R5s201303 R5s210135 sic@

//Final clean up of the target cell

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps13_16(nr_Cell1, nr_Cell3);

//@siclog "Steps 17-18" siclog@

//SS responds with REGISTRATION ACCEPT message

//The UE send a REGISTRATION COMPLETE message

//@sic R5s200309 sic@

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3,

v_GMM_MobilityInfo,

-, -,

noRrcConnectionRelease,

-,

tsc_NR_RbId_SRB2); // On SRB2

//@siclog "Step 19" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-204450 sic@

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

f_NR5GC_CheckDataPath(nr_Cell3,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 19");

}// End of fl_TC_8_1_4_1_2_TestBody

//----------------------------------------------------------------------------------------------------------------------------------

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_12(NR_CellId_Type                       p_SourceCellId,

NR_CellId_Type                       p_TargetCellId,

RNTI_Value                           p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                              p_RBConfig_KeyChange := true,

template (omit)  MeasConfig          p_MeasConfig  := omit,

template (omit) SIB1                 p_SIB1 := omit,

template (omit) SystemInformation    p_SysInfo := omit,

template (omit) MasterKeyUpdate      p_MasterKeyUpdate := omit,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit

) runs on NR5GC_PTC

{ //@sic R5s2008190 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId);

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA

var template (value) DL_DCCH_Message v_RRCReconfiguration;

var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList();

var NextHopChainingCount v_NCC;

if (isvalue(p_RACH_ConfigDedicated.uplink)){ //CFRA

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId);

v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters);

}

if (isvalue(p_MasterKeyUpdate)){

v_NCC := valueof(p_MasterKeyUpdate.nextHopChainingCount);

} else {

v_NCC := tsc_NR_38508_NextHopChainingCount;

}

v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

p_MeasConfig,

p_SIB1,

p_SysInfo,

p_MasterKeyUpdate,

p_RACH_ConfigDedicated

);

f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_12(p_SourceCellId,

p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

v_RRCReconfiguration,

v_RachProcedureConfig,

v_DRBInfoList,

-,

-,

-,

v_NCC,

p_RACH_ConfigDedicated );

}

//----------------------------------------------------------------------------------------------------------------------------------

function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_12(NR_CellId_Type                               p_SourceCellId,

NR_CellId_Type                               p_TargetCellId,

RNTI_Value                                   p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                                      p_RBConfig_KeyChange := true,

template (value) DL_DCCH_Message             p_RRCReconfiguration,

template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig,

template (value) NR_DRBInfoList_Type         p_DrbInfoList,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR,

template (present) UL_DCCH_Message           p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete,

NextHopChainingCount                         p_NCC := tsc_NR_38508_NextHopChainingCount,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit

) runs on NR_BASE_PTC

{ //@sic R5s200819 sic@

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps1_3(p_SourceCellId,

p_TargetCellId,

p_RBConfig_KeyChange,

p_DrbInfoList

);

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps4_9( p_SourceCellId,

p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

p_RRCReconfiguration,

p_RachProcedureConfig,

p_UL_GrantConfig_SourceCell,

p_UL_GrantConfig_TargetCell,

p_NCC,

p_RACH_ConfigDedicated);

// Step 10: Target Cell:    Receive RRCReconfigurationComplete

SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, p_RRCConnReconfigComplete));

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_12(p_SourceCellId);  //@sic R5s210135 sic@

}

//----------------------------------------------------------------------------------------------------------------------------------

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps13_16(NR_CellId_Type              p_SourceCellId,

NR_CellId_Type              p_TargetCellId,

template (omit)  MeasConfig  p_MeasConfig  := omit

) runs on NR_BASE_PTC

{  //@sic R5s210135 sic@

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps13_14( p_SourceCellId, p_TargetCellId, p_MeasConfig );

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps15_16(p_SourceCellId);

}

//----------------------------------------------------------------------------------------------------------------------------------

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps13_14(NR_CellId_Type                              p_SourceCellId,

NR_CellId_Type                              p_TargetCellId,

template (omit)  MeasConfig                 p_MeasConfig  := omit

)

runs on NR_BASE_PTC

{ //@sic R5s210135 sic@

var NR_MeasGapCtrl_Type    v_MeasGapCtrl;

// Step 13: Target Cell:    Re-configure RACH procedure as for initial access.

f_NR_SS_RachProcedureConfig_Def(p_TargetCellId);

// Step 14: Target Cell:    Configure measurement gap (if configured in the source cell or as provided in the RRCReconfiguration message)

//@sic R5-197222 R5s200309 change 2 sic@

if (isvalue(p_MeasConfig.measGapConfig)){ // MeasGap provided to UE in RRCReconfiguration

f_NR_SS_MeasGapCtrlConfig(p_TargetCellId, cs_NR_MeasGapCtrl_Config(p_MeasConfig.measGapConfig),cs_TimingInfo_Now);

} else {

v_MeasGapCtrl := f_NR_CellInfo_GetMeasGapCtrl(p_SourceCellId);

if (not(match(v_MeasGapCtrl, cs_NR_MeasGapCtrl_None))){  //Or MeasGap started in the source cell => to be continued in the target cell

f_NR_SS_MeasGapCtrlConfig(p_TargetCellId, v_MeasGapCtrl, cs_TimingInfo_Now);

}

}

}

//----------------------------------------------------------------------------------------------------------------------------------

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_12( NR_CellId_Type   p_TargetCellId

) runs on NR_BASE_PTC

{ //@sic R5-197222 R5s210135 sic@

// Step 11: Target Cell:    Start periodic TA

f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -);

// Step 12: Target Cell:    Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU).

f_NR_SS_PdcpHandoverCtrl(p_TargetCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetCellId));

}

//----------------------------------------------------------------------------------------------------------------------------------

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Common(NR_CellId_Type                               p_SourceCellId,

NR_CellId_Type                               p_TargetCellId,

RNTI_Value                                   p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                                      p_RBConfig_KeyChange := true,

template (value) DL_DCCH_Message             p_RRCReconfiguration,

template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig,

template (value) NR_DRBInfoList_Type         p_DrbInfoList,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR,

template (present) UL_DCCH_Message           p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete,

NextHopChainingCount                         p_NCC := tsc_NR_38508_NextHopChainingCount,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit,

template (omit)  MeasConfig                  p_MeasConfig  := omit

) runs on NR_BASE_PTC

{ //@sic R5-197222: added p_MeasConfig sic@

//@sic R5-198906: split functions sic@

//@sic R5s200819 sic@

f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_12(p_SourceCellId,

p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

p_RRCReconfiguration,

p_RachProcedureConfig,

p_DrbInfoList,

p_UL_GrantConfig_SourceCell,

p_UL_GrantConfig_TargetCell,

p_RRCConnReconfigComplete,

p_NCC,

p_RACH_ConfigDedicated

);

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps13_14(p_SourceCellId,

p_TargetCellId,

p_MeasConfig

);

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps15_16(p_SourceCellId);

}

## 2.2 Change 2

| Function name | New function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10 |
| --- | --- |
| Reason for change | As stated in Change 1, f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 will be moved after receiving Registration Request for Step 16. It is suggested to declare another function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10. |
| Summary of change | Declare new function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10. |
| TTCN module | NR5GC_Handover |
| MCC160 Comment | See change 1 |

New function:

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10(NR_CellId_Type                       p_SourceCellId,

NR_CellId_Type                       p_TargetCellId,

RNTI_Value                           p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                              p_RBConfig_KeyChange := true,

template (omit)  MeasConfig          p_MeasConfig  := omit,

template (omit) SIB1                 p_SIB1 := omit,

template (omit) SystemInformation    p_SysInfo := omit,

template (omit) MasterKeyUpdate      p_MasterKeyUpdate := omit,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit

) runs on NR5GC_PTC

{ //@sic R5s2008190 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId);

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA

var template (value) DL_DCCH_Message v_RRCReconfiguration;

var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList();

var NextHopChainingCount v_NCC;

if (isvalue(p_RACH_ConfigDedicated.uplink)){ //CFRA

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId);

v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters);

}

if (isvalue(p_MasterKeyUpdate)){

v_NCC := valueof(p_MasterKeyUpdate.nextHopChainingCount);

} else {

v_NCC := tsc_NR_38508_NextHopChainingCount;

}

v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

p_MeasConfig,

p_SIB1,

p_SysInfo,

p_MasterKeyUpdate,

p_RACH_ConfigDedicated

);

f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10(p_SourceCellId,

p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

v_RRCReconfiguration,

v_RachProcedureConfig,

v_DRBInfoList,

-,

-,

-,

v_NCC,

p_RACH_ConfigDedicated,

p_MeasConfig

);

}

## 2.3 Change 3

| Function name | New function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10 |
| --- | --- |
| Reason for change | As stated in Change 1, f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 will be moved after receiving Registration Request for Step 16. It is suggested to declare another function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10. |
| Summary of change | Declare new function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10. |
| TTCN module |  |
| MCC160 Comment | See change 1 |

New function:

function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10(NR_CellId_Type                               p_SourceCellId,

NR_CellId_Type                               p_TargetCellId,

RNTI_Value                                   p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                                      p_RBConfig_KeyChange := true,

template (value) DL_DCCH_Message             p_RRCReconfiguration,

template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig,

template (value) NR_DRBInfoList_Type         p_DrbInfoList,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR,

template (present) UL_DCCH_Message           p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete,

NextHopChainingCount                         p_NCC := tsc_NR_38508_NextHopChainingCount,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit,

template (omit)  MeasConfig                  p_MeasConfig  := omit

) runs on NR_BASE_PTC

{ //@sic R5s200819 sic@

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps1_3(p_SourceCellId,

p_TargetCellId,

p_RBConfig_KeyChange,

p_DrbInfoList

);

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps4_9( p_SourceCellId,

p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

p_RRCReconfiguration,

p_RachProcedureConfig,

p_UL_GrantConfig_SourceCell,

p_UL_GrantConfig_TargetCell,

p_NCC,

p_RACH_ConfigDedicated);

// Step 10: Target Cell:    Receive RRCReconfigurationComplete

SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, p_RRCConnReconfigComplete));

}