3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210136

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0667 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS call control test case 7.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS call control test case 7.11 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS call control test case 7.11 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches Executed	5

4	Execution Log Files	5

4.1	Qualcomm SM8350 + SDX60	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.11 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.11

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350 + SDX60

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_11_NR5GC_IMS1() |
| --- | --- |
| Reason for change | - After Receing the INVITE for MT call from the SS, the UE may send an optional ‘100 Trying’ provisional response before sending the 420 final response. - Upon receing the 420 response from the UE, the To-Tag for the dialog should be stored else a run-time error will occur when creating ACK |
| Summary of change | - Added function for optional Trying message handling. Note: An associated prose CR on 34.229-5 will be raised at RAN5#90 - Function called to store the dialog parameters for this MT call |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted in principle |

Before Change:

function f_TC_7_11_NR5GC_IMS1() runs on IMS_PTC { var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (present) MessageHeader v_MessageHeader_Response420; var template (value) MessageHeader v_MessageHeaderACK; var boolean v_PreconditionsRequired := true; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartSignalling(IPCAN_MT_IMS_Signalling); // @siclog "Step 1 - 8" siclog@ Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 // @siclog "Step 9" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, v_PreconditionsRequired); v_InviteRequestWithSdp.Invite.msgHeader.require := cs_Require(tsc_OptionTagList_precondition); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); // @siclog "Step 10" siclog@ 420 Bad Extension v_MessageHeader_Response420 := f_IMS_InviteResponse_4XX_MessageHeaderRX(v_InviteRequestWithSdp.Invite.msgHeader); v_MessageHeader_Response420.unsupported := cr_Unsupported(tsc_OptionTagList_precondition); f_IMS_ReceiveResponse(v_InviteRequestWithSdp.Invite.msgHeader, v_InviteRequestWithSdp.RoutingInfo.Protocol, cr_Response(c_statusLine420, v_MessageHeader_Response420)); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // @siclog "Step 11" siclog@ ACK !!!! PROSE ISSUE: not in the prose yet !!!! v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite, c_statusLine420); IMS_Client.send(cas_IMS_Ack_Request(v_InviteRequestWithSdp.RoutingInfo, cs_ACK_Request(v_InviteRequestWithSdp.Invite.requestLine.requestUri, v_MessageHeaderACK))); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_IMS_Signalling); }

After Change:

function f_TC_7_11_NR5GC_IMS1() runs on IMS_PTC { var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (present) MessageHeader v_MessageHeader_Response420; var template (value) MessageHeader v_MessageHeaderACK; var boolean v_PreconditionsRequired := true; var IMS_DATA_RSP v_IMS_DATA_RSP; var template (omit) charstring v_ReceivedToTag := omit; timer t_Dummy; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartSignalling(IPCAN_MT_IMS_Signalling); // @siclog "Step 1 - 8" siclog@ Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 // @siclog "Step 9" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, v_PreconditionsRequired); v_InviteRequestWithSdp.Invite.msgHeader.require := cs_Require(tsc_OptionTagList_precondition); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); // @siclog "Step 10" siclog@ 420 Bad Extension v_MessageHeader_Response420 := f_IMS_InviteResponse_4XX_MessageHeaderRX(v_InviteRequestWithSdp.Invite.msgHeader); v_MessageHeader_Response420.unsupported := cr_Unsupported(tsc_OptionTagList_precondition); v_IMS_DATA_RSP := f_IMS_ReceiveResponse_OptionalTrying_Timeout(t_Dummy, v_InviteRequestWithSdp.Invite, v_InviteRequestWithSdp.RoutingInfo.Protocol, cr_Response(c_statusLine420, v_MessageHeader_Response420)); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // @siclog "Step 11" siclog@ v_ReceivedToTag := f_MessageHeader_GetToTag(v_IMS_DATA_RSP.Response.msgHeader); f_IMS_PTC_ImsInfo_DialogSetRemoteTag(v_ReceivedToTag); v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite, c_statusLine420); IMS_Client.send(cas_IMS_Ack_Request(v_InviteRequestWithSdp.RoutingInfo, cs_ACK_Request(v_InviteRequestWithSdp.Invite.requestLine.requestUri, v_MessageHeaderACK))); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_IMS_Signalling); }

MCC160 Implementation

function f_TC_7_11_NR5GC_IMS1() runs on IMS_PTC { var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (present) MessageHeader v_MessageHeader_Response420; var template (value) MessageHeader v_MessageHeaderACK; var IMS_DATA_RSP v_IMS_DATA_RSP; var boolean v_PreconditionsRequired := true; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartSignalling(IPCAN_MT_IMS_Signalling); // @siclog "Step 1 - 8" siclog@ Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 // @siclog "Step 9" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, v_PreconditionsRequired); v_InviteRequestWithSdp.Invite.msgHeader.require := cs_Require(tsc_OptionTagList_precondition); IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); // @siclog "Step 10" siclog@ 420 Bad Extension v_MessageHeader_Response420 := f_IMS_InviteResponse_4XX_MessageHeaderRX(v_InviteRequestWithSdp.Invite.msgHeader); v_MessageHeader_Response420.unsupported := cr_Unsupported(tsc_OptionTagList_precondition); v_IMS_DATA_RSP := f_IMS_ReceiveResponse_OptionalTrying(v_InviteRequestWithSdp.Invite, v_InviteRequestWithSdp.RoutingInfo.Protocol, cr_Response(c_statusLine420, v_MessageHeader_Response420)); f_IMS_Dialog_CompleteCreationForMTCall(v_IMS_DATA_RSP.Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); // @siclog "Step 11" siclog@ ACK !!!! PROSE ISSUE: not in the prose yet !!!! v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite, c_statusLine420); IMS_Client.send(cas_IMS_Ack_Request(v_InviteRequestWithSdp.RoutingInfo, cs_ACK_Request(v_InviteRequestWithSdp.Invite.requestLine.requestUri, v_MessageHeaderACK))); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_IMS_Signalling); }

# Branches Executed

The test case was executed on NR band 1

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_7_11_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_7_11_PIXIT.xml

The Qualcomm SM8350 + SDX60 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_7_11_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_7_11_PIXIT.txt

# References

| [1] | R5s210137 : Supporting information for addition of IMS 5GS call control test case 7.11 |
| --- | --- |