3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210141

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3657 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_SS_ReceiveRegister |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of the function f_SS_ReceiveRegister the variable v_FacilityInvoke is assigned : v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.facilityul.facility_Str.components); This is incorrect. This assignment should be based on the earlier received REGISTER message, but the assignment happnes based on FACILITYul. The assignment should be : v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.register.facility_Str.components); // @sic R5s140989 sic@ This needs to be corrected. Same issue exists for function f_SS_ReceiveRegister_UE_Active. That also needs correction as explained above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of assigning v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.facilityul.facility_Str.components); assigned v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.register.facility_Str.components); // @sic R5s140989 sic@ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN is using incorrect assignment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.8.1 and other SS NITZ testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Same issue exists for function f_SS_ReceiveRegister_UE_Active. That also needs correction as explained above. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to function f_SS_ReceiveRegister	4

1.2	Correction to function f_SS_ReceiveRegister_UE_Active	6

# Corrections required

## Correction to function f_SS_ReceiveRegister

| Function name | f_SS_ReceiveRegister |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_SS_ReceiveRegister the variable v_FacilityInvoke is assigned : v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.facilityul.facility_Str.components); This is incorrect. This assignment should be based on the earlier received REGISTER message, but the assignment happnes based on FACILITYul. The assignment should be : v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.register.facility_Str.components); // @sic R5s140989 sic@ This needs to be corrected. Same issue exists for function f_SS_ReceiveRegister_UE_Active. That also needs correction as explained above. |
| Summary of change | Instead of assigning v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.facilityul.facility_Str.components); assigned v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.register.facility_Str.components); // @sic R5s140989 sic@ |
| TTCN module | Common/SS_CommonFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> // Receive REGISTER message U_Dc.receive(car_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, cr_U_RegisterMO(cr_FacilityIE_tlv, cr_SS_VersionIndAny)))-> value v_RRC_DataInd; //@sic R5s141079 sic@ v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.facilityul.facility_Str.components); // @sic R5s140989 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // Receive REGISTER message U_Dc.receive(car_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, cr_U_RegisterMO(cr_FacilityIE_tlv, cr_SS_VersionIndAny)))-> value v_RRC_DataInd; //@sic R5s141079 sic@ v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.register.facility_Str.components); // @sic R5s140989 sic@ <<SKIPPED CODE>>

## Correction to function f_SS_ReceiveRegister_UE_Active

| Function name | f_SS_ReceiveRegister_UE_Active |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_SS_ReceiveRegister_UE_Active the variable v_FacilityInvoke is assigned : v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.facilityul.facility_Str.components); This is incorrect. This assignment should be based on the earlier received REGISTER message, but the assignment happnes based on FACILITYul. The assignment should be : v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.register.facility_Str.components); // @sic R5s140989 sic@ This needs to be corrected. |
| Summary of change | Instead of assigning v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.facilityul.facility_Str.components); assigned v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.register.facility_Str.components); // @sic R5s140989 sic@ |
| TTCN module | Common/SS_CommonFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> // Receive REGISTER message U_Dc.receive(car_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, cr_U_RegisterMO(cr_FacilityIE_tlv, cr_SS_VersionIndAny))) -> value v_RRC_DataInd; //@sic R5s141081 sic@ v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.facilityul.facility_Str.components); // @sic R5s140989 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // Receive REGISTER message U_Dc.receive(car_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, cr_U_RegisterMO(cr_FacilityIE_tlv, cr_SS_VersionIndAny))) -> value v_RRC_DataInd; //@sic R5s141081 sic@ v_FacilityInvoke := f_SS_ComponentsDecode(v_RRC_DataInd.msg.register.facility_Str.components); // @sic R5s140989 sic@ <<SKIPPED CODE>>