.3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210148

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1558 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_PDUSessionEstablishment_InnerLoop |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some UEs may use dnnValue = “internet” during preamble and user should set pc_APN_ID_Internet = “internet” However, for test cases 9.1.7.2, 10.1.1.2, 10.1.3.2, 10.1.4.1, 10.1.6.1, 10.1.6.2, 11.3.9, they require a specific DNN such as “ABCC”, “APN-1”, and “ABCD” during the test body. Currently, it will lead to TTCN error: "DNN PICS do not match received DNN" because function f_GetPDN_DNNTypeFromAPN will result to v_PDNType = None if UE uses a specific DNN, i.e. “ABCC”, “APN-1”, or “ABCD”. It is suggested to add p_DNN parameter when calling f_NR5GC_PDUSessionEstablishment from individual test cases and use it to set value for variable v_DNN in function f_NR5GC_PDUSessionEstablishment_InnerLoop. Additionally, it is suggested to set PDUType to Internet_DNN when p_DNN is included when calling f_NR5GC_PDUSessionEstablishment. This change is needed particularly for test case 10.1.3.2 Step 26 due to TTCN will check DNN value for the last configured internet PDU session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add condition in f_NR5GC_PDUSessionEstablishment_InnerLoop to set v_DNN = p_DNN if p_DNN is passed into the call from the individual test cases. Set PDUType = Internet_DNN if p_DNN is included. Pass p_DNN when calling f_NR5GC_PDUSessionEstablishment from individual test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.2.NR5GC, 10.1.1.2.NR5GC, 10.1.3.2.NR5GC, 10.1.4.1.NR5GC, 10.1.6.1.NR5GC, 10.1.6.2.NR5GC, 11.3.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_NR5GC_PDUSessionEstablishment_InnerLoop |
| --- | --- |
| Reason for change | Some UEs may use dnnValue = “internet” during preamble and user should set pc_APN_ID_Internet = “internet” However, for test cases 9.1.7.2, 10.1.1.2, 10.1.3.2, 10.1.4.1, 10.1.6.1, 10.1.6.2, 11.3.9, they require a specific DNN such as “ABCC”, “APN-1”, and “ABCD” during the test body. Currently, it will lead to TTCN error: "DNN PICS do not match received DNN" because function f_GetPDN_DNNTypeFromAPN will result to v_PDNType = None if UE uses a specific DNN, i.e. “ABCC”, “APN-1”, or “ABCD”. It is suggested to add p_DNN parameter when calling f_NR5GC_PDUSessionEstablishment from individual test cases and use it to set value for variable v_DNN in function f_NR5GC_PDUSessionEstablishment_InnerLoop. Additionally, it is suggested to set PDUType to Internet_DNN when p_DNN is included when calling f_NR5GC_PDUSessionEstablishment. This change is needed particularly for test case 10.1.3.2 Step 26 due to TTCN will check DNN value for the last configured internet PDU session. #draft proseCR shall be raised in RAN5#90-e |
| Summary of change | Add condition in f_NR5GC_PDUSessionEstablishment_InnerLoop to set v_DNN = p_DNN if p_DNN is passed into the call from the individual test cases. Set PDUType = Internet_DNN if p_DNN is included. |
| TTCN module | NR5GC_NASSteps |
| MCC160 Comment | Accepted in principle, but created a new function to trigger a single PDU session as shown below. |

Before change:

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId,

NR_SRB_COMMON_IND p_ReceivedAsp,

SRBs_AlreadyConfigured p_SRB := SRB1_2,

template (omit) SDAP_Config p_SDAP_Config,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := ?,

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_BuildQosFlow := omit) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

// Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SSConfig;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList;

var SdapConfig_Type v_SdapConfig;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var template (value) SDAP_Config v_SDAP_Config;

var template (omit) PDN_Index_Type v_PDN_Index := omit;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@

var template (value) DNN v_DNN;

var PDU_PDN_DNN_Type v_PDUType;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

p_ReceivedAsp.Signalling.Nas[0].Pdu,

p_PDU_SessionId,

p_PTI,

p_IntegrityProtMaxDataRate,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_AdditionalInfo)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@

v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@

} else {

if (v_GSM_MobilityInfo.PDUType == IMS_DNN) {   // @sic R5-206296 sic@

v_SDAP_Config := cs_SDAP_Config(-, -, {2});

} else {

v_SDAP_Config := cs_SDAP_Config;

}

}

f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig,

v_CellGroupConfig,

v_SSConfig,

v_SDAP_ConfigurationList,

p_NR_CellId,

{v_GSM_MobilityInfo},

p_SRB,

v_SDAP_Config); // @sic R5s201532 sic@

// Only one PDU session input, so only 1st element relevant in output

v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig);

//It is assumed that by default only 1 QoS Flow is specified

v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]);

// If this is a default DRB, enable IP handling for this DRB

if (valueof(v_SDAP_Config.defaultDRB)) {

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo);

f_IP_Handling_StartPDN(IP, valueof(v_PDN_Index), cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s201094 sic@

}

if(p_PDUSessionAuthenticationRequired){ // @sic R5-198184 sic@

//Perform PDU Session Authentication if required

v_SecurityParams := f_NR5GC_Security_Get();

if (p_SRB == SRB1_2) { // @sic R5s201412 sic@

v_SRB := tsc_NR_RbId_SRB2;

}

f_PDUSessionAuthentication( p_NR_CellId, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, v_SRB); // @sic R5s201412 sic@

}

// @sic R5-204400 sic@

if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@

if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) {

v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo));

v_GSM_MobilityInfo.PdnIndex := valueof(v_PDN_Index);

}

}

v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN);   // @sic R5-205939 sic@

if (v_PDUType == None) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN");

} else {

if (isvalue(p_DNN)) {

v_DNN := p_DNN;

} else {

v_DNN := f_NG_GetDNN(v_PLMN, v_PDUType, v_GSM_MobilityInfo.DNN);

}

}

if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B)) { // @sic R5-202550 sic@

// According to 4.9.11 and 4.9.12, this is the only way to flag it's an emergency PDU

v_GSM_MobilityInfo.PDUType := Emergency_PDN;  // @sic R5-206296 sic@

}

// Send Accept

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionEstablishmentAccept (v_GSM_MobilityInfo,

v_GMM_MobilityInfo,

v_DNN,              // @sic R5-205939 sic@

p_SSC_Mode,

p_Session_AMBR,

p_PDU_SessionType,

p_Cause,

p_RQTimer,

p_S_NSSAI,

-,

p_EAP,

p_QoS_Rules,

p_QoSFlowDescr,

omit,

p_BackOff,

p_BuildQosFlow);

// Add this PDU Session to stored list

f_NR5GC_MobileInfo_AddPDUSession (v_GSM_MobilityInfo);

// Send RRC Reconfiguration message

f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId,

v_RadioBearerConfig,

v_SSConfig,

v_CellGroupConfig,

{ cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); // @sic R5s190629 sic@

return v_PDUType;  // @sic R5-206296 sic@

}

After change:

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId,

NR_SRB_COMMON_IND p_ReceivedAsp,

SRBs_AlreadyConfigured p_SRB := SRB1_2,

template (omit) SDAP_Config p_SDAP_Config,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := ?,

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_BuildQosFlow := omit) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

// Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SSConfig;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList;

var SdapConfig_Type v_SdapConfig;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var template (value) SDAP_Config v_SDAP_Config;

var template (omit) PDN_Index_Type v_PDN_Index := omit;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@

var template (value) DNN v_DNN;

var PDU_PDN_DNN_Type v_PDUType;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

p_ReceivedAsp.Signalling.Nas[0].Pdu,

p_PDU_SessionId,

p_PTI,

p_IntegrityProtMaxDataRate,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_AdditionalInfo)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

if (isvalue(p_DNN)){

v_GSM_MobilityInfo.PDUType := Internet_DNN;

}

if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@

v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@

} else {

if (v_GSM_MobilityInfo.PDUType == IMS_DNN) {   // @sic R5-206296 sic@

v_SDAP_Config := cs_SDAP_Config(-, -, {2});

} else {

v_SDAP_Config := cs_SDAP_Config;

}

}

f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig,

v_CellGroupConfig,

v_SSConfig,

v_SDAP_ConfigurationList,

p_NR_CellId,

{v_GSM_MobilityInfo},

p_SRB,

v_SDAP_Config); // @sic R5s201532 sic@

// Only one PDU session input, so only 1st element relevant in output

v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig);

//It is assumed that by default only 1 QoS Flow is specified

v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]);

// If this is a default DRB, enable IP handling for this DRB

if (valueof(v_SDAP_Config.defaultDRB)) {

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo);

f_IP_Handling_StartPDN(IP, valueof(v_PDN_Index), cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s201094 sic@

}

if(p_PDUSessionAuthenticationRequired){ // @sic R5-198184 sic@

//Perform PDU Session Authentication if required

v_SecurityParams := f_NR5GC_Security_Get();

if (p_SRB == SRB1_2) { // @sic R5s201412 sic@

v_SRB := tsc_NR_RbId_SRB2;

}

f_PDUSessionAuthentication( p_NR_CellId, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, v_SRB); // @sic R5s201412 sic@

}

// @sic R5-204400 sic@

if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@

if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) {

v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo));

v_GSM_MobilityInfo.PdnIndex := valueof(v_PDN_Index);

}

}

if (isvalue(p_DNN)) {

v_DNN := p_DNN;

} else {

v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN);   // @sic R5-205939 sic@

if (v_PDUType == None) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN");

} else {

if (isvalue(p_DNN)) {

v_DNN := p_DNN;

} else {

v_DNN := f_NG_GetDNN(v_PLMN, v_PDUType, v_GSM_MobilityInfo.DNN);

}

}

}

if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B)) { // @sic R5-202550 sic@

// According to 4.9.11 and 4.9.12, this is the only way to flag it's an emergency PDU

v_GSM_MobilityInfo.PDUType := Emergency_PDN;  // @sic R5-206296 sic@

}

// Send Accept

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionEstablishmentAccept (v_GSM_MobilityInfo,

v_GMM_MobilityInfo,

v_DNN,              // @sic R5-205939 sic@

p_SSC_Mode,

p_Session_AMBR,

p_PDU_SessionType,

p_Cause,

p_RQTimer,

p_S_NSSAI,

-,

p_EAP,

p_QoS_Rules,

p_QoSFlowDescr,

omit,

p_BackOff,

p_BuildQosFlow);

// Add this PDU Session to stored list

f_NR5GC_MobileInfo_AddPDUSession (v_GSM_MobilityInfo);

// Send RRC Reconfiguration message

f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId,

v_RadioBearerConfig,

v_SSConfig,

v_CellGroupConfig,

{ cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); // @sic R5s190629 sic@

return v_PDUType;  // @sic R5-206296 sic@

}

## MCC160 Implementation

function f_NR5GC_TriggerSinglePDUSession(NR_CellId_Type p_NR_CellId,

integer p_PDUSessionId,

PDU_PDN_DNN_Type p_DNNType := Internet_DNN,

template (omit) charstring p_DNN := "",

template (omit) S_NSSAI_Type p_S_NSSAI_InATCmd := omit,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI)

runs on NR5GC_PTC

{

var charstring v_S_NSSAI_ForATCmd := "";

var charstring v_DNN_ForATCmd := "";

var template (omit) DNN v_DNNRx := omit;

var template (value) DNN v_DNNTx;

var template (omit) QoS_Rules v_QoS_Rules;

var charstring v_QoSFlowToBuild;

// QoS Rules, QoS Flow and DNN in Accept message determined by p_DNNType

select (p_DNNType) {

case (Internet_DNN) {

v_DNN_ForATCmd := pc_APN_ID_Internet;

v_QoS_Rules := f_BuildDefaultQoSRules ("1");

v_QoSFlowToBuild := "1";

}

case (IMS_DNN) {

v_DNN_ForATCmd := pc_APN_ID_IMS;

v_QoS_Rules := f_BuildDefaultQoSRules ("2");

v_QoSFlowToBuild := "2";

}

case else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU_PDN_DNN_Type not specified");

}

}

v_DNNTx := cs_DNN(-, f_DomainName_Encode(v_DNN_ForATCmd));

// Determine if a different value should be used in the AT command and UL

if (isvalue(p_DNN)) { // omit => DNN NOT included in AT cmd

if (valueof(p_DNN) != "") {  // specific value of DNN

v_DNN_ForATCmd := valueof(p_DNN);

v_DNNTx := cs_DNN(-, f_DomainName_Encode(v_DNN_ForATCmd));

}

// DNN will be present and equal to value in AT command

v_DNNRx := cs_DNN(-, f_DomainName_Encode(v_DNN_ForATCmd));

} else {  // omit => DNN NOT included in AT cmd or in UL Req

v_DNN_ForATCmd := "";

v_DNNRx := omit;

}

if (isvalue(p_S_NSSAI_InATCmd)){

v_S_NSSAI_ForATCmd := f_ConvertS_NSSAI_ForATCommand(p_S_NSSAI_InATCmd);

}

f_UT_RequestPDUSession(UT, p_PDUSessionId, v_DNN_ForATCmd, -, -, v_S_NSSAI_ForATCmd); //AT command to trigger a PDU session

f_NR5GC_PDUSessionEstablishment (p_NR_CellId,

1,                 // Only triggering this one session

-, -,

p_ExpectedS_NSSAI, // Value in UL Req message

v_DNNRx,           // Value in UL Req message

-,

v_DNNTx,           // Value in DL Accept message

v_QoS_Rules,       // Value determined by PICS

-,

p_S_NSSAI,         // Value in DL Accept message

-, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -,

v_QoSFlowToBuild);       // Value determined by PICS

}

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId,

NR_SRB_COMMON_IND p_ReceivedAsp,

SRBs_AlreadyConfigured p_SRB := SRB1_2,

template (omit) SDAP_Config p_SDAP_Config,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := ?,

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_BuildQosFlow := omit) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

// Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SSConfig;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList;

var SdapConfig_Type v_SdapConfig;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var template (value) SDAP_Config v_SDAP_Config;

var template (omit) PDN_Index_Type v_PDN_Index := omit;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@

var template (value) DNN v_DNN;

var PDU_PDN_DNN_Type v_PDUType;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

p_ReceivedAsp.Signalling.Nas[0].Pdu,

p_PDU_SessionId,

p_PTI,

p_IntegrityProtMaxDataRate,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_AdditionalInfo)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@

v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@

} else {

if (v_GSM_MobilityInfo.PDUType == IMS_DNN) {   // @sic R5-206296 sic@

v_SDAP_Config := cs_SDAP_Config(-, -, {2});

} else {

v_SDAP_Config := cs_SDAP_Config;

}

}

f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig,

v_CellGroupConfig,

v_SSConfig,

v_SDAP_ConfigurationList,

p_NR_CellId,

{v_GSM_MobilityInfo},

p_SRB,

v_SDAP_Config); // @sic R5s201532 sic@

// Only one PDU session input, so only 1st element relevant in output

v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig);

//It is assumed that by default only 1 QoS Flow is specified

v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]);

// If this is a default DRB, enable IP handling for this DRB

if (valueof(v_SDAP_Config.defaultDRB)) {

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo);

f_IP_Handling_StartPDN(IP, valueof(v_PDN_Index), cs_DrbInfo_NR(p_NR_CellId, v_QosFlowId)); // @sic R5s201094 sic@

}

if(p_PDUSessionAuthenticationRequired){ // @sic R5-198184 sic@

//Perform PDU Session Authentication if required

v_SecurityParams := f_NR5GC_Security_Get();

if (p_SRB == SRB1_2) { // @sic R5s201412 sic@

v_SRB := tsc_NR_RbId_SRB2;

}

f_PDUSessionAuthentication( p_NR_CellId, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, v_SRB); // @sic R5s201412 sic@

}

// @sic R5-204400 sic@

if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@

if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) {

v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo));

v_GSM_MobilityInfo.PdnIndex := valueof(v_PDN_Index);

}

}

if (isvalue(p_DNN)) {

v_DNN := p_DNN;

} else {

v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN);   // @sic R5-205939 sic@

if (v_PDUType == None) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN");

} else {

v_DNN := f_NG_GetDNN(v_PLMN, v_PDUType, v_GSM_MobilityInfo.DNN);

}

}

if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B)) { // @sic R5-202550 sic@

// According to 4.9.11 and 4.9.12, this is the only way to flag it's an emergency PDU

v_GSM_MobilityInfo.PDUType := Emergency_PDN;  // @sic R5-206296 sic@

}

// Send Accept

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionEstablishmentAccept (v_GSM_MobilityInfo,

v_GMM_MobilityInfo,

v_DNN,              // @sic R5-205939 sic@

p_SSC_Mode,

p_Session_AMBR,

p_PDU_SessionType,

p_Cause,

p_RQTimer,

p_S_NSSAI,

-,

p_EAP,

p_QoS_Rules,

p_QoSFlowDescr,

omit,

p_BackOff,

p_BuildQosFlow);

// Add this PDU Session to stored list

f_NR5GC_MobileInfo_AddPDUSession (v_GSM_MobilityInfo);

// Send RRC Reconfiguration message

f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId,

v_RadioBearerConfig,

v_SSConfig,

v_CellGroupConfig,

{ cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); // @sic R5s190629 sic@

return v_PDUType;  // @sic R5-206296 sic@

}

## 2.2 Change 2

| Function name | fl_TC_10_1_6_1_Body |
| --- | --- |
| Reason for change | As stated in Change 1, pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| Summary of change | Pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| TTCN module | GSM_TestCases_NR5GC |
| MCC160 Comment | Accepted in principle, subject to prose CR approval, but instead called new function, f_NR5GC_TriggerSinglePDUSession, as shown below |

Before change:

function fl_TC_10_1_6_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var PDUSessionInfoList_Type v_PDUSessionInfo;

var integer v_SessionId := f_NR5GC_MobileInfo_GetNoOfSessions()+1;   // @sic R5s201362 sic@

var O1_Type v_PDUSessionIdToRelease;  // @sic R5s201362 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var NR_RadioBearerId_Type v_SRB;

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var charstring v_NSSAI;

var ProcedureTransactionIdentifier v_PTI; // @sic R5s201063 sic@

timer t_Wait := 3.0; // @sic R5-195981 sic@

v_SRB := tsc_NR_RbId_SRB2;

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

//@siclog "Steps 1" siclog@

f_UT_RequestPDUSession(UT, v_SessionId, tsc_RandomAPN, -, -, v_NSSAI); // @sic Rs201230 sic@

//@siclog "Steps 2" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1);

//@siclog "Steps 3" siclog@

//Cause the UE to transmit a PDU SESSION RELEASE REQ

f_UT_DeactivateBearer(UT, v_SessionId); // @sic R5s201362 sic@

…

}

After change:

import from NAS_AuxiliaryDefsAndFunctions all;

…

function fl_TC_10_1_6_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var PDUSessionInfoList_Type v_PDUSessionInfo;

var integer v_SessionId := f_NR5GC_MobileInfo_GetNoOfSessions()+1;   // @sic R5s201362 sic@

var O1_Type v_PDUSessionIdToRelease;  // @sic R5s201362 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var NR_RadioBearerId_Type v_SRB;

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var charstring v_NSSAI;

var ProcedureTransactionIdentifier v_PTI; // @sic R5s201063 sic@

var template DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_RandomAPN));

timer t_Wait := 3.0; // @sic R5-195981 sic@

v_SRB := tsc_NR_RbId_SRB2;

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

//@siclog "Steps 1" siclog@

f_UT_RequestPDUSession(UT, v_SessionId, tsc_RandomAPN, -, -, v_NSSAI); // @sic Rs201230 sic@

//@siclog "Steps 2" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, -, v_DNN, -, v_DNN);

//@siclog "Steps 3" siclog@

//Cause the UE to transmit a PDU SESSION RELEASE REQ

f_UT_DeactivateBearer(UT, v_SessionId); // @sic R5s201362 sic@

…

}

## MCC160 Implementation

function fl_TC_10_1_6_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var PDUSessionInfoList_Type v_PDUSessionInfo;

var integer v_SessionId := f_NR5GC_MobileInfo_GetNoOfSessions()+1;   // @sic R5s201362 sic@

var O1_Type v_PDUSessionIdToRelease;  // @sic R5s201362 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var NR_RadioBearerId_Type v_SRB;

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var ProcedureTransactionIdentifier v_PTI; // @sic R5s201063 sic@

timer t_Wait := 3.0; // @sic R5-195981 sic@

v_SRB := tsc_NR_RbId_SRB2;

//@siclog "Steps 1" siclog@

f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, v_SessionId, -, tsc_RandomAPN, cs_S_NSSAI_SST1eMBB); // @sic Rs201230 sic@

//@siclog "Steps 3" siclog@

//Cause the UE to transmit a PDU SESSION RELEASE REQ

f_UT_DeactivateBearer(UT, v_SessionId); // @sic R5s201362 sic@

v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList();

v_PDUSessionIdToRelease := v_PDUSessionInfo[f_NR5GC_MobileInfo_GetNoOfSessions()-1].SessionId; // @sic R5s201550 sic@

## 2.3 Change 3

| Function name | fl_TC_10_1_6_2_Body |
| --- | --- |
| Reason for change | As stated in Change 1, pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| Summary of change | Pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| TTCN module | GSM_TestCases_NR5GC |
| MCC160 Comment | Accepted in principle but see change 2 for implementation |

Before change:

function fl_TC_10_1_6_2_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

//var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

//var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var PDUSessionInfoList_Type v_PDUSessionInfo;

var integer v_SessionId := f_NR5GC_MobileInfo_GetNoOfSessions()+1;  // @sic R5s201362 sic@

var O1_Type v_PDUSessionIdToRelease;  // @sic R5s201362 sic@

//@siclog "Steps 1" siclog@

f_UT_RequestPDUSession(UT, v_SessionId, tsc_RandomAPN, -, -, v_NSSAI); // @sic R5s201230 sic@

//@siclog "Steps 2" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1);

v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); // @sic R5s190717 sic@

v_PDUSessionIdToRelease := v_PDUSessionInfo[f_NR5GC_MobileInfo_GetNoOfSessions()-1].SessionId; // @sic R5s201550 sic@

//@siclog "Steps 3" siclog@

//Cause the UE to transmit a PDU SESSION RELEASE REQ

f_UT_DeactivateBearer(UT, v_SessionId);  // @sic R5s201362 sic@

…

}

After change:

function fl_TC_10_1_6_2_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

//var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

//var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var PDUSessionInfoList_Type v_PDUSessionInfo;

var integer v_SessionId := f_NR5GC_MobileInfo_GetNoOfSessions()+1;  // @sic R5s201362 sic@

var O1_Type v_PDUSessionIdToRelease;  // @sic R5s201362 sic@

var template DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_RandomAPN));

//@siclog "Steps 1" siclog@

f_UT_RequestPDUSession(UT, v_SessionId, tsc_RandomAPN, -, -, v_NSSAI); // @sic R5s201230 sic@

//@siclog "Steps 2" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, -, v_DNN, -, v_DNN);

v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); // @sic R5s190717 sic@

v_PDUSessionIdToRelease := v_PDUSessionInfo[f_NR5GC_MobileInfo_GetNoOfSessions()-1].SessionId; // @sic R5s201550 sic@

//@siclog "Steps 3" siclog@

//Cause the UE to transmit a PDU SESSION RELEASE REQ

f_UT_DeactivateBearer(UT, v_SessionId);  // @sic R5s201362 sic@

…

}

## 2.4 Change 4

| Function name | fl_TC_10_1_1_2_Body |
| --- | --- |
| Reason for change | As stated in Change 1, pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| Summary of change | Pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| TTCN module | GSM_TestCases_NR5GC |
| MCC160 Comment | Accepted in principle but see change 2 for implementation |

Before change:

function fl_TC_10_1_1_2_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA));

var PDUSessionInfoList_Type v_PDUSessionInfo;

var O1_Type v_PDUSessionIdToRelease;

var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get();

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

v_GSM_MobilityInfo := v_PDUSessionInfoList[0];

//@siclog "Steps 0A - 0B" siclog@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN); // @sic R5-203785, R5s201230 sic@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); // @sic R5-203785 sic@

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

v_GSM_MobilityInfo := v_PDUSessionInfoList[1];

//@siclog "Steps 1 - 2" siclog@

//Send PDU Session Authentication Command and receive PDU Session Authentication Complete

f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); // @sic R5s20956 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

…

}

After change:

function fl_TC_10_1_1_2_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA));

var PDUSessionInfoList_Type v_PDUSessionInfo;

var O1_Type v_PDUSessionIdToRelease;

var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get();

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

var template DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_RandomAPN));

v_GSM_MobilityInfo := v_PDUSessionInfoList[0];

//@siclog "Steps 0A - 0B" siclog@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN); // @sic R5-203785, R5s201230 sic@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, -, v_DNN, -, v_DNN); // @sic R5-203785 sic@

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

v_GSM_MobilityInfo := v_PDUSessionInfoList[1];

//@siclog "Steps 1 - 2" siclog@

//Send PDU Session Authentication Command and receive PDU Session Authentication Complete

f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); // @sic R5s20956 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2");

…

}

## 2.5 Change 5

| Function name | f_TC_10_1_4_1_NR5GC |
| --- | --- |
| Reason for change | As stated in Change 1, pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| Summary of change | Pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| TTCN module | GSM_TestCases_NR5GC |
| MCC160 Comment | Accepted in principle: f_Check_NG_PDUSessionEstablishmentReq modified to save NSSAI value New function to get NSSAI value added: f_Get_NSSAIForPDUSessionId Preamble modified. PDU session triggered in case there is no one after preamble: called to function f_NR5GC_TriggerSinglePDUSession |

Before change:

function f_TC_10_1_4_1_NR5GC() runs on NR5GC_PTC

{

//UE-requested PDU session establishment / Abnormal / T3580

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var PLMN_Identity v_ASN_PLMN;

var NAS_PlmnId v_PLMN;

var NG_NAS_MSG_Indication_Type v_RegistrationRequest;

var charstring v_NSSAI;

f_NR5GC_Init_NAS(NR_1);

//Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellA);

//Preamble

//The UE is in 5GS state 3N-A

// @sic R5s201028 sic@

v_RegistrationRequest := f_NR5GC_Preamble_Steps1_4(v_NGC_NASCellA);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_RegistrationRequest.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA);

v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, v_NGC_NASCellA, v_RegistrationRequest, v_PLMN, TESTMode_OFF, Initial_NoSecurity); // @sic R5-202564 sic@

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session @sic R5s201230 sic@

// @sic R5s201304 sic@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, tsc_NR_RbId_SRB1);

f_NR_RRCRelease(v_NGC_NASCellA);

f_NR5GC_RRC_ConnectedState3N_Def (v_NGC_NASCellA, TESTMode_OFF); //STATE_CONNECTED_3A

//Main test body

fl_TC_10_1_4_1_Body();

// Finish

f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A);

}

After change:

function f_TC_10_1_4_1_NR5GC() runs on NR5GC_PTC

{

//UE-requested PDU session establishment / Abnormal / T3580

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var PLMN_Identity v_ASN_PLMN;

var NAS_PlmnId v_PLMN;

var NG_NAS_MSG_Indication_Type v_RegistrationRequest;

var charstring v_NSSAI;

var template DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_RandomAPN));

f_NR5GC_Init_NAS(NR_1);

//Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellA);

//Preamble

//The UE is in 5GS state 3N-A

// @sic R5s201028 sic@

v_RegistrationRequest := f_NR5GC_Preamble_Steps1_4(v_NGC_NASCellA);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_RegistrationRequest.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA);

v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, v_NGC_NASCellA, v_RegistrationRequest, v_PLMN, TESTMode_OFF, Initial_NoSecurity); // @sic R5-202564 sic@

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session @sic R5s201230 sic@

// @sic R5s201304 sic@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, tsc_NR_RbId_SRB1, -, -, v_DNN, -, v_DNN);

f_NR_RRCRelease(v_NGC_NASCellA);

f_NR5GC_RRC_ConnectedState3N_Def (v_NGC_NASCellA, TESTMode_OFF); //STATE_CONNECTED_3A

//Main test body

fl_TC_10_1_4_1_Body();

// Finish

f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A);

}

## MCC160 Implementation

type record GSM_MobilityInfo_Type {                                /* structured type to be stored & passed in/out of GSM Msgs

@status    APPROVED (IMS, NR5GC, NR5GC_IRAT, POS) */

O1_Type                         SessionId,  // @sic R5s200564 sic@

ProcedureTransactionIdentifier  PTI,

PDU_PDN_DNN_Type                PDUType,    // @sic R5-206296 sic@

B3_Type                         SessionType optional,

DNN                             DNN         optional,

AlwaysOnPDUSessionInd           AlwaysOn    optional,

ExtdProtocolConfigOptions       PCO         optional,

integer                         EPS_Bearer  optional,   // @sic R5s201094 sic@

PDN_Index_Type                  PdnIndex    optional,   // @sic R5s201094 sic@

S_NSSAI_Type                    NSSAI       optional

};

function f_Check_NG_PDUSessionEstablishmentReq(out GSM_MobilityInfo_Type p_GSM_MobilityInfo,

NG_NAS_UL_Pdu_Type p_ReceivedMsgs,

template (present) O1_Type p_PDU_SessionId

...

p_GSM_MobilityInfo.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; // @sic R5s00564 sic@

p_GSM_MobilityInfo.PTI := v_GSMMsg.pdu_Session_Establishment_Request.procedureTransactionIdentifier;

p_GSM_MobilityInfo.SessionType := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionType.typeValue;

p_GSM_MobilityInfo.EPS_Bearer := omit; // these will be set in the Accept message @sic R5-204400 sic@

p_GSM_MobilityInfo.PdnIndex := omit;   // these will be set in the Accept message @sic R5-204400 sic@

if (ispresent (p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn)) {

p_GSM_MobilityInfo.DNN := p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn;

} else {

p_GSM_MobilityInfo.DNN := omit;

}

p_GSM_MobilityInfo.PDUType := f_GetPDNTypeFromDNN(p_GSM_MobilityInfo.DNN);  // @sic R5-206296 sic@

if (ispresent (v_GSMMsg.pdu_Session_Establishment_Request.alwaysOnPDUSessionReq)) {

p_GSM_MobilityInfo.AlwaysOn := v_GSMMsg.pdu_Session_Establishment_Request.alwaysOnPDUSessionReq;

} else {

p_GSM_MobilityInfo.AlwaysOn := omit;

}

if (ispresent (v_GSMMsg.pdu_Session_Establishment_Request.extdProtocolConfigurationOptions)) {

p_GSM_MobilityInfo.PCO := v_GSMMsg.pdu_Session_Establishment_Request.extdProtocolConfigurationOptions;

} else {

p_GSM_MobilityInfo.PCO := omit;

}

if (ispresent (p_ReceivedMsgs.Msg.ul_Nas_Transport.s_NSSAI)) { // @sic R5s2101448 sic@

p_GSM_MobilityInfo.NSSAI := p_ReceivedMsgs.Msg.ul_Nas_Transport.s_NSSAI;

} else {

p_GSM_MobilityInfo.NSSAI := omit;

}

return v_Result;

}

/*

* @desc      Get the NSSAI for PDU Session id

* @param     p_PDUSessionInfo

* @param     p_PDUSessionId

* @return    template S_NSSAI_Type

* @status

*/

function f_Get_NSSAIForPDUSessionId(PDUSessionInfoList_Type p_PDUSessionInfo, O1_Type p_PDUSessionId) return template (omit) S_NSSAI_Type

{

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var integer i;

for (i :=0; i < lengthof(p_PDUSessionInfo); i := i+1) {

v_GSM_MobilityInfo := p_PDUSessionInfo[i];

if (match (v_GSM_MobilityInfo.SessionId, p_PDUSessionId)) {  // @sic R5s200564 sic@

return v_GSM_MobilityInfo.NSSAI;

}

}

return omit;

}

// ==========================TC 10.1.4.1 =====================================

/*

* @desc      REFERENCE TS 38.523-1 clause 10.1.4.1

* @status    APPROVED (NR5GC)

*/

function f_TC_10_1_4_1_NR5GC() runs on NR5GC_PTC

{

//UE-requested PDU session establishment / Abnormal / T3580

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

f_NR5GC_Init_NAS(NR_1);

//Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellA);

// @sic R5s210148 sic@

//Preamble

//The UE is in 5GS state 3N-A

f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A);

if (f_NR5GC_MobileInfo_GetNoOfSessions() == 0){

f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, 1, -, tsc_RandomAPN, cs_S_NSSAI_SST1eMBB);

}

//Main test body

fl_TC_10_1_4_1_Body();

// Finish

f_NR_Postamble(v_NGC_NASCellA, STATE_CONNECTED_3A);

}

//===========================================================================================================================

/*

* @desc      test body of TC_10_1_4_1

* @status    APPROVED (NR5GC)

*/

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var PDUSessionInfoList_Type v_PDUSessionInfoList :=f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ?

var O1_Type v_PDUSessionIdToRelease;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend;

var template (omit) DNN v_RequestedDNN;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (omit) NR_DRBInfo_Type v_DRBInfo;

var template (value) CellGroupConfig v_CellGroupConfig;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_Count := 0;

var integer v_DRBId;

var float v_T3580Val := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 16.0);

var charstring v_NSSAI;

var template (omit)S_NSSAI_Type v_RequestedNSSAI; // @sic R5s210148 sic@

timer t_Wait;

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease);

v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); // @sic R5s210148 sic@

// @sic R5-206357 sic@

//@siclog "Steps 1" siclog@

// The generic test procedure in TS 38.508-1 clause 4.9.21 for PDU Session Release is performed withThe SS transmits an PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release PDU session X.

v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); // 5GSM cause "Reactivation requested"

//Get DRB Info for DRB associated with last PDU session

v_DRBId := f_NR_GetDefaultDRB_ForLastPDUSession();

v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId);

// Build RLC-BearerConfig for SRB2 and DRB

//v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB (v_NGC_NASCellA, {v_DRBId}, true);

v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, {v_DRBInfo.RLC_Config.logicalChannelIdentity});

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, {v_DRBId});

f_NR_RRCReconfigNewDRBWithSSConfig(v_NGC_NASCellA,

v_RadioBearerConfig,

{v_DRBInfo.SS_Config}, // re-establish DRB in SS with exactly the same config as earlier

v_CellGroupConfig,

{ cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend) },

tsc_NR_RbId_SRB1, -, false);

//The UE transmits a RRCReconfigurationComplete

SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete));

//The UE transmits a PDU SESSION RELEASE COMPLETE message?

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect PDU Session Release Complete");

}

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1");

//@siclog "Steps 2 - 15 void" siclog@

//@siclog "Steps 16" siclog@

//@sic R5-202664 sic@

//The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2).

t_Wait.start (1.0);

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

t_Wait.stop;

}

[]t_Wait.timeout

{

// @sic R5s201028 sic@

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI);

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session @sic R5s201230 sic@

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

}

}

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

//@siclog "Steps 17-25" siclog@

for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) {

//@siclog "Steps 17-19-21-23" siclog@

//The SS waits 16 seconds (T3580).

f_Delay(v_T3580Val);

//@siclog "Steps 18-20-22-24" siclog@

//Check: Does the UE re-transmit the PDU SESSION ESTABLISHMENT REQUEST message sent in step 16?

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

}

//@siclog "Step 25" siclog@

//The SS waits 16 seconds (5th expiry of T3580).

f_Delay(v_T3580Val);

//@siclog "Step 26" siclog@

t_Wait.start(20.0);

alt{

[] SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu,-, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN))// @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

}

[]t_Wait.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26");

}

}

## 2.6 Change 6

| Function name | fl_TC_10_1_3_2_Body |
| --- | --- |
| Reason for change | As stated in Change 1, pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| Summary of change | Pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| TTCN module | GSM_TestCases_NR5GC |
| MCC160 Comment | Accepted in principle but see change 2 for implementation |

Before change:

function fl_TC_10_1_3_2_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var O1_Type v_PDUSessionIdToRelease;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

var template (omit) DNN v_DNN;

var integer v_InternetPduSessionID;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; // @sic R5-206355 sic@

timer t_T3396 := 120.0; // @sic R5-201629 sic@

timer t_T3584 := 120.0; // @sic R5-201629 sic@

timer t_Wait := 5.0;

v_InternetPduSessionID := f_NR5GC_MobileInfo_GetNoOfSessions()+1;

//@siclog "Steps 1" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 2" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, * , omit);

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

//@siclog "Steps 3-4" siclog@

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources

cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00010'B)); //T3396 = 2 minutes // @sic R5-201629 sic@

//@siclog "Steps 5" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 6" siclog@

t_T3396.start; // @sic R5s200585 sic@

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered)))

{

t_T3396.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

[]t_T3396.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6");

}

}

//@siclog "Steps 7" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 8-9" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, * , omit); // @sic R5s200585 sic@ // @sic R5s201659 sic@

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

//@siclog "Steps 10-11" siclog@

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources

cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00000'B)); //T3396 = 0 minutes // @sic R5-201629 sic@

//@siclog "Steps 12" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 13-14" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, * , omit); // @sic R5s200585 sic@ // @sic R5s201659 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13");

//@siclog "Steps 15-16" siclog@

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID(); // @sic R5s200585 sic@

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources

cs_GPRS_Timer2_3_IEI('37'O, '111'B, '00000'B)); //T3396 = Deactivated

//@siclog "Steps 17" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 18" siclog@

t_Wait.start;

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered)))

{

t_Wait.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18");

}

[]t_Wait.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18");

}

}

//@siclog "Step 19" siclog@

f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_CONNECTED_3A);

//@siclog "Step 20" siclog@

f_UT_SwitchOnUE(UT); //The UE is Switched ON

//@siclog "Step 21" siclog@

//Steps from Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed.

f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, noRrcConnectionRelease); // @sic R5s200585 sic@

//@siclog "Steps 22" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 23-24" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, * , omit); // @sic R5s200585 sic@ // @sic R5s201659 sic@

v_InternetPduSessionID := v_InternetPduSessionID +1;

//@siclog "Steps 25" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, tsc_APN1_String, -, -, v_NSSAI); //AT command to trigger a PDU session with S-NSSAI and DNN // @sic R5s201659 sic@

//@siclog "Steps 26" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, cs_S_NSSAI_SST1eMBB_WithIEI); // @sic R5s200585 sic@ // @sic R5s201659 sic@

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

//Check that DNN is present in the PDU session established at Step 26

v_DNN := f_Get_DNNForPDUSessionId(f_NR5GC_MobileInfo_GetSessionInfoList(), v_PDUSessionIdToRelease); //Store it for further use

if (not isvalue(v_DNN)){

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DNN being sent in Step 26");

}

//@siclog "Steps 27-28" siclog@

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '01000011'B), //5GSM cause Insufficient resources for specific slice and DNN

cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00010'B)); //T3584 = 2 minutes // @sic R5-201629 sic@

…

}

After change:

function fl_TC_10_1_3_2_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var O1_Type v_PDUSessionIdToRelease;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

var template (omit) DNN v_DNN;

var integer v_InternetPduSessionID;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; // @sic R5-206355 sic@

var template DNN v_DNN_1 := cs_DNN(-, f_DomainName_Encode(tsc_APN1_String));

timer t_T3396 := 120.0; // @sic R5-201629 sic@

timer t_T3584 := 120.0; // @sic R5-201629 sic@

timer t_Wait := 5.0;

v_InternetPduSessionID := f_NR5GC_MobileInfo_GetNoOfSessions()+1;

//@siclog "Steps 1" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 2" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, * , omit);

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

//@siclog "Steps 3-4" siclog@

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources

cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00010'B)); //T3396 = 2 minutes // @sic R5-201629 sic@

//@siclog "Steps 5" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 6" siclog@

t_T3396.start; // @sic R5s200585 sic@

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered)))

{

t_T3396.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6");

}

[]t_T3396.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6");

}

}

//@siclog "Steps 7" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 8-9" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, * , omit); // @sic R5s200585 sic@ // @sic R5s201659 sic@

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

//@siclog "Steps 10-11" siclog@

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources

cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00000'B)); //T3396 = 0 minutes // @sic R5-201629 sic@

//@siclog "Steps 12" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 13-14" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, * , omit); // @sic R5s200585 sic@ // @sic R5s201659 sic@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13");

//@siclog "Steps 15-16" siclog@

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID(); // @sic R5s200585 sic@

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources

cs_GPRS_Timer2_3_IEI('37'O, '111'B, '00000'B)); //T3396 = Deactivated

//@siclog "Steps 17" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 18" siclog@

t_Wait.start;

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered)))

{

t_Wait.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 18");

}

[]t_Wait.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18");

}

}

//@siclog "Step 19" siclog@

f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_CONNECTED_3A);

//@siclog "Step 20" siclog@

f_UT_SwitchOnUE(UT); //The UE is Switched ON

//@siclog "Step 21" siclog@

//Steps from Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed.

f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, noRrcConnectionRelease); // @sic R5s200585 sic@

//@siclog "Steps 22" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, -, -, -, v_NSSAI); //AT command to trigger a PDU session WITHOUT DNN (APN)

//@siclog "Steps 23-24" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, * , omit); // @sic R5s200585 sic@ // @sic R5s201659 sic@

v_InternetPduSessionID := v_InternetPduSessionID +1;

//@siclog "Steps 25" siclog@

f_UT_RequestPDUSession(UT, v_InternetPduSessionID, tsc_APN1_String, -, -, v_NSSAI); //AT command to trigger a PDU session with S-NSSAI and DNN // @sic R5s201659 sic@

//@siclog "Steps 26" siclog@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, cs_S_NSSAI_SST1eMBB_WithIEI, v_DNN_1, -, v_DNN_1); // @sic R5s200585 sic@ // @sic R5s201659 sic@

v_PDUSessionIdToRelease := f_GetLastConfiguredInternetPDUSessionID();

//Check that DNN is present in the PDU session established at Step 26

v_DNN := f_Get_DNNForPDUSessionId(f_NR5GC_MobileInfo_GetSessionInfoList(), v_PDUSessionIdToRelease); //Store it for further use

if (not isvalue(v_DNN)){

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DNN being sent in Step 26");

}

//@siclog "Steps 27-28" siclog@

f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5s201131 sic@

v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '01000011'B), //5GSM cause Insufficient resources for specific slice and DNN

cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00010'B)); //T3584 = 2 minutes // @sic R5-201629 sic@

…

}

## 2.7 Change 7

| Function name | fl_TC_11_3_9_TestBody |
| --- | --- |
| Reason for change | As stated in Change 1, pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| Summary of change | Pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| TTCN module | UAC_NR5GC |
| MCC160 Comment | Accepted |

Before change:

function fl_TC_11_3_9_TestBody() runs on NR5GC_PTC

{

const charstring const_DNNValue := "ABCD";

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (value) NR_CellPowerList_Type v_CellPowerList_T1;

var template DNN v_DNN;

var boolean v_ReceiveRRCSetupRequest;

timer t_Wait := 20.0;

v_DNN := cs_DNN(-, 'ABCD'O);

//@siclog "Steps 3" siclog@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, const_DNNValue);

//@siclog "Steps 4" siclog@

// Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is the same value in UL NAS TRANSPORT message in preamble within 20s?

t_Wait.start;

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, -, v_DNN))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4");

}

}

[]t_Wait.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

}

}

// @siclog "Step 5" siclog@

//The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.9.3.2-1/2.

v_CellPowerList_T1 := { //@sic R5-205388 sic@

cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)

};

f_NR_SetCellPowerList(v_CellPowerList_T1);

// @siclog "Step 6" siclog@

//The mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 12 and release RRC connection.

f_NR5GC_MobilityRegistration(nr_Cell12);

//@siclog "Steps 7" siclog@

//Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command.

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, const_DNNValue);

//@siclog "Step 8" siclog@

//Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data within 20s?

t_Wait.start;

v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData);

if (v_ReceiveRRCSetupRequest) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8");

}

//@siclog "Steps 9 - 14" siclog@

//Steps 3-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed.

f_NR_RRCSetup_Def (nr_Cell12);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *),

tsc_SHT_IntegrityProtected);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0010'B)) {  // @sic R5s200654 subject to prose CR approval sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg);

//@siclog "Steps 15 - 16" siclog@

//The UE transmits a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is set to the same value with the DNN value of Operator-defined access category definitions defined in Table 11.3.9.3.3-1

//The SS transmits PDU SESSION ESTABLISHMENT ACCEPT message.

f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1, -, -, - , v_DNN);

}

After change:

import from NAS_AuxiliaryDefsAndFunctions all;

…

function fl_TC_11_3_9_TestBody() runs on NR5GC_PTC

{

const charstring const_DNNValue := "ABCD";

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (value) NR_CellPowerList_Type v_CellPowerList_T1;

var template DNN v_DNN;

var boolean v_ReceiveRRCSetupRequest;

timer t_Wait := 20.0;

v_DNN := cs_DNN(-, f_DomainName_Encode(const_DNNValue));

//@siclog "Steps 3" siclog@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, const_DNNValue);

//@siclog "Steps 4" siclog@

// Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is the same value in UL NAS TRANSPORT message in preamble within 20s?

t_Wait.start;

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, -, v_DNN))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4");

}

}

[]t_Wait.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

}

}

// @siclog "Step 5" siclog@

//The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.9.3.2-1/2.

v_CellPowerList_T1 := { //@sic R5-205388 sic@

cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)

};

f_NR_SetCellPowerList(v_CellPowerList_T1);

// @siclog "Step 6" siclog@

//The mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 12 and release RRC connection.

f_NR5GC_MobilityRegistration(nr_Cell12);

//@siclog "Steps 7" siclog@

//Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command.

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, const_DNNValue);

//@siclog "Step 8" siclog@

//Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data within 20s?

t_Wait.start;

v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData);

if (v_ReceiveRRCSetupRequest) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8");

}

//@siclog "Steps 9 - 14" siclog@

//Steps 3-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed.

f_NR_RRCSetup_Def (nr_Cell12);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *),

tsc_SHT_IntegrityProtected);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0010'B)) {  // @sic R5s200654 subject to prose CR approval sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg);

//@siclog "Steps 15 - 16" siclog@

//The UE transmits a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is set to the same value with the DNN value of Operator-defined access category definitions defined in Table 11.3.9.3.3-1

//The SS transmits PDU SESSION ESTABLISHMENT ACCEPT message.

f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1, -, -, - , v_DNN, -, v_DNN);

}

## 2.8 Change 8

| Function name | fl_TC_9_1_7_2_Body |
| --- | --- |
| Reason for change | As stated in Change 1, pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| Summary of change | Pass p_DNN when calling f_NR5GC_PDUSessionEstablishment. |
| TTCN module | Service_Request_NR5GC |
| MCC160 Comment | Accepted in principle but see change 2 for implementation |

Before change:

function fl_TC_9_1_7_2_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

var template (omit) QosFlow_Identification_Type v_QosFlowId;

var DRB_ToReleaseList v_DRBToReleaseList;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data

var template (value) PDU_SessionStatus v_PDUSessionStatus;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_PSI;

var boolean v_Result := false;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; //@sic R5s200716 sic@

var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5s200716 sic@

var NR_RLC_BearerToReleaseList_Type v_RlcToReleaseList; //@sic R5s200716 sic@

var template (value) NR_RadioBearerList_Type v_RadioBearerDrbList; //@sic R5s201492 sic@

var integer k := 0; //@sic R5s201492 sic@

var IntegerList_Type v_DrbList; //@sic R5s201492 sic@

//@siclog "Steps 1A - 1C" siclog@ //@sic R5-201088 sic@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); // @sic R5s201230 sic@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1);

//@siclog "Steps 1D - 1E" siclog@ //@sic R5-202661 sic@

f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '01'O); //@sic R5s200716 sic@

f_IP_Routing_ChangeToRawmode(IP); //@sic R5s200716 sic@

…

}

After change:

import from NAS_AuxiliaryDefsAndFunctions all;

…

function fl_TC_9_1_7_2_Body() runs on NR5GC_PTC

{

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB);

var template (omit) QosFlow_Identification_Type v_QosFlowId;

var DRB_ToReleaseList v_DRBToReleaseList;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data

var template (value) PDU_SessionStatus v_PDUSessionStatus;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_PSI;

var boolean v_Result := false;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; //@sic R5s200716 sic@

var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5s200716 sic@

var NR_RLC_BearerToReleaseList_Type v_RlcToReleaseList; //@sic R5s200716 sic@

var template (value) NR_RadioBearerList_Type v_RadioBearerDrbList; //@sic R5s201492 sic@

var integer k := 0; //@sic R5s201492 sic@

var IntegerList_Type v_DrbList; //@sic R5s201492 sic@

var template DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_RandomAPN));

//@siclog "Steps 1A - 1C" siclog@ //@sic R5-201088 sic@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); // @sic R5s201230 sic@

f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, -, v_DNN, -, v_DNN);

//@siclog "Steps 1D - 1E" siclog@ //@sic R5-202661 sic@

f_NR5GC_CloseUE_TestLoopModeB (v_NGC_NASCellA, '01'O); //@sic R5s200716 sic@

f_IP_Routing_ChangeToRawmode(IP); //@sic R5s200716 sic@

…

}