3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210158

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4586 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE eMTC testcase 23.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, at Step 17a1 and 18a1 SS sends MODIFY EPS BEARER CONTEXT REQUEST, with EPS bearer ID set as tsc_EpsDefaultBearerId . However, the TTCN expects EPS bearer ID as tsc_EpsDedicatedBearerId in the MODIFY EPS BEARER CONTEXT ACCEPT message at step17a2 and 18a2. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced tsc_EpsDedicatedBearerId with tsc_EpsDefaultBearerId for UE MODIFY EPS BEARER CONTEXT ACCEPT message at step17a2 and 18a2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation FAIL a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 23.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Corrections to update BearerId test cases 23.1.1	3

1. Correction to ' f_TC_23_1_1_EUTRA() '	3

# Corrections to update BearerId test cases 23.1.1

## 1. Correction to ' f_TC_23_1_1_EUTRA() '

| Function name | f_TC_23_1_1_EUTRA() () |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 17a1 and 18a1 SS sends MODIFY EPS BEARER CONTEXT REQUEST, with EPS bearer ID set as tsc_EpsDefaultBearerId . However, the TTCN expects EPS bearer ID as tsc_EpsDedicatedBearerId in the MODIFY EPS BEARER CONTEXT ACCEPT message at step17a2 and 18a2. This needs to be corrected. |
| Summary of change | Replaced tsc_EpsDedicatedBearerId with tsc_EpsDefaultBearerId for UE MODIFY EPS BEARER CONTEXT ACCEPT message at step17a2 and 18a2. |
| TTCN module | LTE_A_PRO\23\ CIoT_Optimization_CP.ttcn |
| MCC160 Comment | Accepted |

Before Change:

/* * @desc REFERENCE TS 36.523-1 clause 23.1.1 * @status APPROVED (LTE_A_PRO) */ function f_TC_23_1_1_EUTRA() runs on EUTRA_PTC {/* CIoT / Control Plane MO and MT IP and non-IP Data Transfer / Serving PLMN Rate Control / APN Rate Control */ var template (value) octetstring v_DataStep3_11 := 'F0F0F0'O; var template (value) octetstring v_DataStep17a5 := f_GetN_OctetsFromPRBS(1, 10); var EUTRA_SecurityParams_Type v_AuthParams; var ProcedureTransactionIdentifier v_PTI := tsc_PTI_Unassigned; var integer v_Count := 0; var integer v_PCOCount := 0; var integer v_DataLengthReceived := 0; … //Steps 17a1 to 17a8 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports APN control if (pc_APN_RateControl) { //@siclog "Step 17a1" siclog@ // The SS sends a MODIFY EPS BEARER CONTEXT REQUEST message of to modify APN, MTU and PLMN rate controls. v_APN_RateControl := cs_APNRateControlParams ('001'B, int2oct(1, 3)); v_ProtocolContainerAPN := cs_ProtocolContainer ('0016'O, bit2oct(encvalue(v_APN_RateControl))); v_NewPco := f_NAS_ExtdProtocolConfigOptionsTX ({v_ProtocolContainerAPN}); SRB.send(cas_SRB1_NasPdu_REQ(eutra_Cell1, //@sic R5s180379 sic@ cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ModifyEPSBearerCxtReq_AllParams(tsc_EpsDefaultBearerId, v_PTI, omit, omit, omit, omit, omit, f_Get_508_PFI(), omit, omit, omit, omit, v_NewPco, omit, // @sic R5s180271 Baseline Moving 2018 Phase 1 sic@ omit)))); //@siclog "Step 17a2" siclog@ //Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, //@sic R5s180379 sic@ cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_508_ModifyEPSBearerCxtAccept(tsc_EpsDedicatedBearerId)))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17a2"); //@siclog "Steps 17a3-17a4" siclog@ f_EUTRA_CloseUE_TestLoopModeGH(eutra_Cell1, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS, 2, 0); //@siclog "Step 17a5" siclog@ //SS transmits an ESM DATA TRANSPORT message containing 320 Octets of downlink user data. SRB.send(cas_SRB_NasPdu_REQ(eutra_Cell1, tsc_SRB1, //@sic R5s180379 sic@ cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, v_PTI, cs_UserDataContainer(v_DataStep17a5))))); v_ReleaseAssistanceInd := cr_ReleaseAssistanceInd ('00'B) ifpresent; for (i:=0; i==1; i:=i+1) { //Steps 17a6-17a8 are repeated 2 times. //@siclog "Step 17a6" siclog@ //SS starts timer 1 min APN Rate control t_PLMNRateControl.start( f_EUTRA_SetTimerToleranceMax (eutra_Cell1, nasTimer, 60.0)); //@siclog "Step 17a7" siclog@ //Check: Does the UE send an ESM DATA TRANSPORT message respecting the APN set in step 17a1: Max size of message 256 Octets? //@sic R5s180379 sic@ SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(tsc_EpsDefaultBearerId, cr_UserDataContainer(int2oct(lengthof(v_DataStep17a5),2),v_DataStep17a5), v_ReleaseAssistanceInd)))); //@siclog "Step 17a8" siclog@ //Wait until timer 1 min APN Rate control expires. t_PLMNRateControl.timeout; v_ReleaseAssistanceInd := cr_ReleaseAssistanceInd (('00'B, '01'B)) ifpresent; } } //Steps 18a1-18a6 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports MTU parameters if (pc_NonIP_Link_MTU_Parameter or pc_IPv4_Link_MTU_Parameter) { if (pc_IPv4_Link_MTU_Parameter) { v_ProtocolContainerIPv4 := cs_ProtocolContainer ('0010'O, int2oct(128, 2)); v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4; v_PCOCount := v_PCOCount + 1 ; } if (pc_NonIP_Link_MTU_Parameter) { v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4; v_PCOCount := v_PCOCount + 1 ; } if (pc_APN_RateControl) { v_ProtocolContainerAPN := cs_ProtocolContainer ('0016'O, '00'O & 'FFFFFF'O); v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4; } v_NewPco := f_NAS_ExtdProtocolConfigOptionsTX (v_ProtocolContainerList); //@siclog "Step 18a1" siclog@ //The SS sends a MODIFY EPS BEARER CONTEXT REQUEST message to modify APN, MTU and PLMN rate controls. SRB.send(cas_SRB_NasPdu_REQ(eutra_Cell1, tsc_SRB1, //@sic R5s180379 sic@ cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ModifyEPSBearerCxtReq_AllParams(tsc_EpsDefaultBearerId, v_PTI, omit, omit, omit, omit, omit, f_Get_508_PFI(), omit, omit, omit, omit, v_NewPco, omit, // @sic R5s180271 Baseline Moving 2018 Phase 1 sic@ omit)))); //@siclog "Step 18a2" siclog@ //Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? //@sic R5s180379 sic@ SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_508_ModifyEPSBearerCxtAccept(tsc_EpsDedicatedBearerId)))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18a2"); … }

After Change:

/* * @desc REFERENCE TS 36.523-1 clause 23.1.1 * @status APPROVED (LTE_A_PRO) */ function f_TC_23_1_1_EUTRA() runs on EUTRA_PTC {/* CIoT / Control Plane MO and MT IP and non-IP Data Transfer / Serving PLMN Rate Control / APN Rate Control */ var template (value) octetstring v_DataStep3_11 := 'F0F0F0'O; var template (value) octetstring v_DataStep17a5 := f_GetN_OctetsFromPRBS(1, 10); var EUTRA_SecurityParams_Type v_AuthParams; var ProcedureTransactionIdentifier v_PTI := tsc_PTI_Unassigned; var integer v_Count := 0; var integer v_PCOCount := 0; var integer v_DataLengthReceived := 0; … //Steps 17a1 to 17a8 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports APN control if (pc_APN_RateControl) { //@siclog "Step 17a1" siclog@ // The SS sends a MODIFY EPS BEARER CONTEXT REQUEST message of to modify APN, MTU and PLMN rate controls. v_APN_RateControl := cs_APNRateControlParams ('001'B, int2oct(1, 3)); v_ProtocolContainerAPN := cs_ProtocolContainer ('0016'O, bit2oct(encvalue(v_APN_RateControl))); v_NewPco := f_NAS_ExtdProtocolConfigOptionsTX ({v_ProtocolContainerAPN}); SRB.send(cas_SRB1_NasPdu_REQ(eutra_Cell1, //@sic R5s180379 sic@ cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ModifyEPSBearerCxtReq_AllParams(tsc_EpsDefaultBearerId, v_PTI, omit, omit, omit, omit, omit, f_Get_508_PFI(), omit, omit, omit, omit, v_NewPco, omit, // @sic R5s180271 Baseline Moving 2018 Phase 1 sic@ omit)))); //@siclog "Step 17a2" siclog@ //Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, //@sic R5s180379 sic@ cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_508_ModifyEPSBearerCxtAccept(tsc_EpsDefaultBearerId)))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 17a2"); //@siclog "Steps 17a3-17a4" siclog@ f_EUTRA_CloseUE_TestLoopModeGH(eutra_Cell1, tsc_UE_TestLoopMode_TypeG, tsc_TestLoopModeGH_UplinkMode_NAS, 2, 0); //@siclog "Step 17a5" siclog@ //SS transmits an ESM DATA TRANSPORT message containing 320 Octets of downlink user data. SRB.send(cas_SRB_NasPdu_REQ(eutra_Cell1, tsc_SRB1, //@sic R5s180379 sic@ cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, v_PTI, cs_UserDataContainer(v_DataStep17a5))))); v_ReleaseAssistanceInd := cr_ReleaseAssistanceInd ('00'B) ifpresent; for (i:=0; i==1; i:=i+1) { //Steps 17a6-17a8 are repeated 2 times. //@siclog "Step 17a6" siclog@ //SS starts timer 1 min APN Rate control t_PLMNRateControl.start( f_EUTRA_SetTimerToleranceMax (eutra_Cell1, nasTimer, 60.0)); //@siclog "Step 17a7" siclog@ //Check: Does the UE send an ESM DATA TRANSPORT message respecting the APN set in step 17a1: Max size of message 256 Octets? //@sic R5s180379 sic@ SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_ESM_DATA_TRANSPORT(tsc_EpsDefaultBearerId, cr_UserDataContainer(int2oct(lengthof(v_DataStep17a5),2),v_DataStep17a5), v_ReleaseAssistanceInd)))); //@siclog "Step 17a8" siclog@ //Wait until timer 1 min APN Rate control expires. t_PLMNRateControl.timeout; v_ReleaseAssistanceInd := cr_ReleaseAssistanceInd (('00'B, '01'B)) ifpresent; } } //Steps 18a1-18a6 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports MTU parameters if (pc_NonIP_Link_MTU_Parameter or pc_IPv4_Link_MTU_Parameter) { if (pc_IPv4_Link_MTU_Parameter) { v_ProtocolContainerIPv4 := cs_ProtocolContainer ('0010'O, int2oct(128, 2)); v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4; v_PCOCount := v_PCOCount + 1 ; } if (pc_NonIP_Link_MTU_Parameter) { v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4; v_PCOCount := v_PCOCount + 1 ; } if (pc_APN_RateControl) { v_ProtocolContainerAPN := cs_ProtocolContainer ('0016'O, '00'O & 'FFFFFF'O); v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4; } v_NewPco := f_NAS_ExtdProtocolConfigOptionsTX (v_ProtocolContainerList); //@siclog "Step 18a1" siclog@ //The SS sends a MODIFY EPS BEARER CONTEXT REQUEST message to modify APN, MTU and PLMN rate controls. SRB.send(cas_SRB_NasPdu_REQ(eutra_Cell1, tsc_SRB1, //@sic R5s180379 sic@ cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ModifyEPSBearerCxtReq_AllParams(tsc_EpsDefaultBearerId, v_PTI, omit, omit, omit, omit, omit, f_Get_508_PFI(), omit, omit, omit, omit, v_NewPco, omit, // @sic R5s180271 Baseline Moving 2018 Phase 1 sic@ omit)))); //@siclog "Step 18a2" siclog@ //Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? //@sic R5s180379 sic@ SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_508_ModifyEPSBearerCxtAccept(tsc_EpsDefaultBearerId)))); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18a2"); … }