3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210159

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1565 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, following assignment is done: v_PDUSessionStatus.iei := '26'O; //@sic R5s200587 sic@ v_PDUSessionStatus.psi0_7 := '00000000'B; //@sic R5s200587 sic@ After this , the variable v_PDUSessionStatus is sent in Service ACCEPT message at Steps 10 -11. This causes a runtime error as the IEs v_PDUSessionStatus.psi8_15 and v_PDUSessionStatus.spare are un-initialized before the sending of Service ACCEPT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Assigned the following after Step 7 : v_PDUSessionStatus := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase stalls with a run-time error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function fl_TC_9_1_7_2_Body()	3

# Corrections required

## function fl_TC_9_1_7_2_Body()

| Function name | fl_TC_9_1_7_2_Body() |
| --- | --- |
| Reason for change | In the current TTCN implementation, following assignment is done: v_PDUSessionStatus.iei := '26'O; //@sic R5s200587 sic@ v_PDUSessionStatus.psi0_7 := '00000000'B; //@sic R5s200587 sic@ After this , the variable v_PDUSessionStatus is sent in Service ACCEPT message at Steps 10 -11. This causes a runtime error as the IEs v_PDUSessionStatus.psi8_15 and v_PDUSessionStatus.spare are un-initialized before the sending of Service ACCEPT. |
| Summary of change | Assigned the following after Step 7 : v_PDUSessionStatus := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus; |
| TTCN module | ATS_NR5GC/9_1_7/Service_Request_NR5GC.ttcn |
| MCC160 Comment | Alternative solution implemented: use new template cs_PDU_SessionReactivationResult |

Before Change:

<<SKIPPED CODE>> //@siclog "Steps 7" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, //@sic R5s200587 sic@ cr_NG_SERVICE_REQUEST('0001'B, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), ?, cr_ULDataStatus(-,-), //@sic R5-201088 sic@ *,*,*)))) -> value v_ReceivedAsp; if(v_PSI <8){ v_Result := f_Bitstring_BitIsSet(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); //@sic R5s200587 sic@ } else { v_Result := f_Bitstring_BitIsSet(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus.psi8_15,15-v_PSI); // @sic R5s200757 sic@ //@sic R5s200587 sic@ } if(not v_Result) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } v_PDUSessionStatus.iei := '26'O; //@sic R5s200587 sic@ v_PDUSessionStatus.psi0_7 := '00000000'B; //@sic R5s200587 sic@ //The SS transmits an IDENTITY REQUEST requesting 5G-GUTI in the IE identity type //@siclog "Step 8" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_IDENTITY_REQUEST(tsc_IdType_NG_GUTI)))); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Steps 7" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, //@sic R5s200587 sic@ cr_NG_SERVICE_REQUEST('0001'B, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), ?, cr_ULDataStatus(-,-), //@sic R5-201088 sic@ *,*,*)))) -> value v_ReceivedAsp; if(v_PSI <8){ v_Result := f_Bitstring_BitIsSet(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); //@sic R5s200587 sic@ } else { v_Result := f_Bitstring_BitIsSet(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus.psi8_15,15-v_PSI); // @sic R5s200757 sic@ //@sic R5s200587 sic@ } if(not v_Result) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } v_PDUSessionStatus := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus;//WA#9_1_7_2 v_PDUSessionStatus.iei := '26'O; //@sic R5s200587 sic@ v_PDUSessionStatus.psi0_7 := '00000000'B; //@sic R5s200587 sic@ //The SS transmits an IDENTITY REQUEST requesting 5G-GUTI in the IE identity type //@siclog "Step 8" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_IDENTITY_REQUEST(tsc_IdType_NG_GUTI)))); <<SKIPPED CODE>>

MCC160 Implementation:

template (value) PDU_SessionStatus cs_PDU_SessionReactivationResult (template (value) B8_Type p_PSI0_7:= '00000000'B, template (value) B8_Type p_PSI8_15 := '00000000'B) :=

{                         /* 24.501 cl. 9.11.3.44 */

iei       := '26'O, /* present in case of TV or TLV; omit in case of V */

iel       := '02'O, /* present in case of LV or TLV; omit in case of V */

psi0_7    := p_PSI0_7,                         /* PSI(0)- PSI(7) */

psi8_15   := p_PSI8_15,                        /* PSI(8)- PSI(15)*/

spare     := omit

};

<<SKIPPED CODE>>

//@siclog "Steps 7" siclog@

SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, //@sic R5s200587 sic@

cr_NG_SERVICE_REQUEST('0001'B,

cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext),

?,

cr_ULDataStatus(-,-), //@sic R5-201088 sic@

*,*,*)))) -> value v_ReceivedAsp;

if(v_PSI <8){

v_Result := f_Bitstring_BitIsSet(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); //@sic R5s200587 sic@

}

else

{

v_Result := f_Bitstring_BitIsSet(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.service_Request.ulDataStatus.psi8_15,15-v_PSI); // @sic R5s200757 sic@ //@sic R5s200587 sic@

}

if(not v_Result)

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed");

}

v_PDUSessionStatus := cs_PDU_SessionReactivationResult('00000000'B); //@sic R5s200587, R5s210159 sic@

//The SS transmits an IDENTITY REQUEST requesting 5G-GUTI in the IE identity type

//@siclog "Step 8" siclog@

SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_NG_IDENTITY_REQUEST(tsc_IdType_NG_GUTI))));

<<SKIPPED CODE>>