3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210162

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1569 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.4.1.5 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-15 NR5GC test case 8.1.4.1.5 to the NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 8.1.4.1.5 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

1.1 Verification Test Summary	3

2 Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

3 Branches executed	7

4 Execution Log Files	8

4.1 HiSilicon Balong 5000	8

5 References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.4.1.5 which is part of the NR5GC_IWD_20wk50 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Ting Gao
	gaoting@starpointcomm.com

Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.1.4.1.5

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Starpoint SP9500 Test system，Keysight S8704A Protocol Conformance Toolset solution

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_4_1_5_NR5GC |
| --- | --- |
| Reason for change | As per TP3, Re-establishment needs to take place on the source cell but in the test procedure, it is happening on target cell. |
| Summary of change | End test case on NRcell1. |
| TTCN module | RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement |

Before change

function f_TC_8_1_4_1_5_NR5GC() runs on NR5GC_PTC

{ // Intra NR handover / Failure / Re-establishment successful

f_NR5GC_Init(NR_2);

//Set Table 8.1.4.1.5.3.3-1: SIB1

f_NR_CellInfo_SetSIB1_PreambleTransMax(nr_Cell1, n50);

f_NR_CellInfo_SetSIB1_PreambleTransMax(nr_Cell2, n50);

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell2);

//The UE is in 5GS state 3N-A

f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A);

f_NR_TestBody_Set(true);

fl_TC_8_1_4_1_5_TestBody();

f_NR_TestBody_Set(false);

f_NR_Postamble(nr_Cell2, STATE_IDLE_1A);

} //End of f_TC_8_1_4_1_5_NR5GC  }

After change

function f_TC_8_1_4_1_5_NR5GC() runs on NR5GC_PTC

{ // Intra NR handover / Failure / Re-establishment successful

f_NR5GC_Init(NR_2);

//Set Table 8.1.4.1.5.3.3-1: SIB1

f_NR_CellInfo_SetSIB1_PreambleTransMax(nr_Cell1, n50);

f_NR_CellInfo_SetSIB1_PreambleTransMax(nr_Cell2, n50);

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell2);

//The UE is in 5GS state 3N-A

f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A);

f_NR_TestBody_Set(true);

fl_TC_8_1_4_1_5_TestBody();

f_NR_TestBody_Set(false);

f_NR_Postamble(nr_Cell1, STATE_IDLE_1A);

} //End of f_TC_8_1_4_1_5_NR5GC

## Change 2

| Function name | fl_TC_8_1_4_1_5_TestBody |
| --- | --- |
| Reason for change | - As per TP3, Re-establishment needs to take place on the source cell but in the test procedure, it is happening on target cell - As per TP3, nextHopChainingcount needs to be same as with the currently active KgNB in Table 8.1.4.1.5.3.3-6 |
| Summary of change | 1 Modify the test procedure after step8 according to the prose cr. 2 Use NCC=2 in step 11. |
| TTCN module | RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | 1. Accepted conditional to prose CR agreement 2. Accepted conditional to prose CR agreement |

Before change

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC

{

var template (value) DL_DCCH_Message v_RRCReconfigurationHO;

var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19.

var float v_T304Max := f_NR_SetTimerToleranceMax(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19.

var template(value) TimingInfo_Type v_TimingInfo;

var SubFrameTiming_Type v_SubframeTiming;

//Power level as per Table 8.1.4.1.5.3.2-1

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)};

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)};

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT3 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)};

timer t_T304Min;

// Reconfigure nr_Cell2 to no respond to any RA Preamble

// Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond.

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse);

//@siclog "Step 1" siclog@

// The SS changes the power level setting according to the row "T1".

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 2" siclog@

// The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2.

// Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used

v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2);

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO));

t_T304Min.start(v_T304Min);

//@siclog "Step 3" siclog@

//The SS changes the power level setting according to the row "T2".

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

//@siclog "Steps 4-6B" siclog@

//Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1?

//@sic R5-206369, R5-206336 sic@

f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, -, "Step 4");

//@siclog "Step 7" siclog@

//The SS changes the power level setting according to the row "T1".

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 8" siclog@

//The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2.

v_SubframeTiming := f_NR_GetNextSendOccasion_DL(nr_Cell1);

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1,  cs_TimingInfo_NR(v_SubframeTiming), v_RRCReconfigurationHO));

//Re-configure RACH procedure as for initial access in nr_Cell2

v_TimingInfo := f_NR_TimingOtherCell(nr_Cell1, cs_TimingInfo_NR(v_SubframeTiming), nr_Cell2);

v_SubframeTiming := f_SubFrameTiming_AddMilliSeconds(valueof(v_TimingInfo.SubFrame), float2int(v_T304Max * 1000.0)); //Add T304max to make sure T304 timeout

f_NR_SS_RachProcedureConfig_Def(nr_Cell2,  cs_TimingInfo_NR(v_SubframeTiming));

//EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.5.3.2-4 should take place.

//Nothing to do nr_Cell2 already configured to not respond to any RA Preamble

//@siclog "Step 9" siclog@

//The SS changes the power level setting according to the row "T3".

f_NR_SetCellPowerList(v_CellPowerList_AtT3);

//@siclog "Step 10-12B" siclog@

// Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 2?

//@sic R5-206369, R5-206336 sic@

f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell2, -, 0, -, -, "Step 12");

//@siclog "Step 13" siclog@

//The SS transmits an RRCRelease message on NR Cell 2.

f_NR_RRCRelease(nr_Cell2);

} // End of fl_TC_8_1_4_1_5_TestBody

After change

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC

{

var template (value) DL_DCCH_Message v_RRCReconfigurationHO;

var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19.

var float v_T304Max := f_NR_SetTimerToleranceMax(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19.

var template(value) TimingInfo_Type v_TimingInfo;

var SubFrameTiming_Type v_SubframeTiming;

//Power level as per Table 8.1.4.1.5.3.2-1

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)};

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)};

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT3 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),

cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)};

timer t_T304Min;

// Reconfigure nr_Cell2 to no respond to any RA Preamble

// Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond.

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse);

//@siclog "Step 1" siclog@

// The SS changes the power level setting according to the row "T1".

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 2" siclog@

// The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2.

// Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used

v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2);

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO));

t_T304Min.start(v_T304Min);

//@siclog "Step 3" siclog@

//The SS changes the power level setting according to the row "T2".

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

//@siclog "Steps 4-6B" siclog@

//Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1?

//@sic R5-206369, R5-206336 sic@

f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, -, "Step 4");

//@siclog "Step 7" siclog@

//The SS changes the power level setting according to the row "T1".

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 8" siclog@

//The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2.

v_SubframeTiming := f_NR_GetNextSendOccasion_DL(nr_Cell1);

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1,  cs_TimingInfo_NR(v_SubframeTiming), v_RRCReconfigurationHO));

//Re-configure RACH procedure as for initial access in nr_Cell2

//EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.5.3.2-4 should take place.

//Nothing to do nr_Cell2 already configured to not respond to any RA Preamble

//@siclog "Step 9" siclog@

//The SS changes the power level setting according to the row "T3".

f_NR_SetCellPowerList(v_CellPowerList_AtT2);//prose cr

//@siclog "Step 10-12B" siclog@

// Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 2?

//@sic R5-206369, R5-206336 sic@

f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2, -, -, "Step 12");

//@siclog "Step 13" siclog@

//The SS transmits an RRCRelease message on NR Cell 2.

f_NR_RRCRelease(nr_Cell1);

} // End of fl_TC_8_1_4_1_5_TestBody

# Branches executed

The test case was executed on NR band n41.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- \ TC_8_1_4_1_5_NR5GC_PASS.spm

- \ NR5GC_Testsuite.sppar

The HiSilicon Balong 5000 passed this test case on the Keysight S8704A Protocol Conformance Toolset solution. The documentation below is enclosed as evidence of the successful test case run [1]:

- \ TC_8_1_4_1_5_LOG.html

- \ TC_8_1_4_1_5_PIXIT.xml

# References

| [1] | R5s210163: Supporting information for agreement of NR5GC test case 8.1.4.1.5 in FR1 |
| --- | --- |