3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210187

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1577 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC testcase 8.2.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the latest TS 38.523-1-g60, the securityConfig IE in step 5 shall be set as “Not present”, but in current TTCN implementation, the IE has been configured, which will cause an abnormal, since this has been configured in step 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed the securityConfig IE in step 5 to omit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a normal UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

- Corrections required

  - Change 1

| Function name | fl_TC_8_2_2_1_1_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | According to the latest TS 38.523-1-g60, the securityConfig IE in step 5 shall be set as “Not present”, but in current TTCN implementation, the IE has been configured, which will cause an abnormal, since this has been configured in step 1. |
| Summary of change | - Changed the securityConfig IE in step 5 to omit. |
| TTCN module | RRCReconfig_ENDC_NR |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_8_2_2_1_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) NR_RLC_BearerToReleaseList_Type v_Rlc_BearerToReleaseList := {3}; var template (omit) octetstring v_EncodedSCG_Config := omit; var SubFrameTiming_Type v_Timing; //@siclog "Steps 1 - 2" siclog@ //Set the NR ENDC Configuration for SRB3 v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3)},-, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); //@sic R5s200419, R5s201115, R5-203397, R5s201464 sic@ v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, {cs_38508_RLC_BearerConfig_SRB(tsc_SRB3)}); //Configure SRB3 in SS f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(f_NR_LogicalChannelId_SRB(tsc_SRB3), tsc_NR_SRB3_LogicalChannelPriority) ,omit)});//@sic R5s200419, R5s201218 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig); // Now send octetstrings to EUTRA f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA); //@siclog "Step 3" siclog@ //The SS transmits an RRCReconfiguration message to reconfigure NR MAC sent on SRB3/NR Cell 1. v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(-, -, cs_NR_BSR_ConfigDef(sf5, sf40))); v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig)); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config))); //Reconfigure SS with mac_CellGroupConfig f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_SCGConfig.mac_CellGroupConfig, omit))); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCReconfigurationComplete message on SRB3/NR Cell 1? SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Steps 5-6" siclog@ //@sic R5-190465, R5-199030, R5-201230 sic@ //The SS transmits an RRCReconfiguration message to reconfigure NR PDCP sent on SRB3/NR Cell 1. //Check: Does the UE transmit an RRCReconfigurationComplete message on SRB3/NR Cell 1? v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3, cs_PDCP_Config_SRB(ms20))},//@sic R5s200419 sic@ -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig))); SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Steps 7 - 8" siclog@ v_RadioBearerConfig := cs_NR_RadioBearer_ReleaseSRB3; v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, v_Rlc_BearerToReleaseList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig); // Now send octetstrings to EUTRA //Release SRB3 in SS f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB3)}); }

After Change:

function fl_TC_8_2_2_1_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) NR_RLC_BearerToReleaseList_Type v_Rlc_BearerToReleaseList := {3}; var template (omit) octetstring v_EncodedSCG_Config := omit; var SubFrameTiming_Type v_Timing; //@siclog "Steps 1 - 2" siclog@ //Set the NR ENDC Configuration for SRB3 v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3)},-, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); //@sic R5s200419, R5s201115, R5-203397, R5s201464 sic@ v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, {cs_38508_RLC_BearerConfig_SRB(tsc_SRB3)}); //Configure SRB3 in SS f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(f_NR_LogicalChannelId_SRB(tsc_SRB3), tsc_NR_SRB3_LogicalChannelPriority) ,omit)});//@sic R5s200419, R5s201218 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig); // Now send octetstrings to EUTRA f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA); //@siclog "Step 3" siclog@ //The SS transmits an RRCReconfiguration message to reconfigure NR MAC sent on SRB3/NR Cell 1. v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(-, -, cs_NR_BSR_ConfigDef(sf5, sf40))); v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig)); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config))); //Reconfigure SS with mac_CellGroupConfig f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_SCGConfig.mac_CellGroupConfig, omit))); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCReconfigurationComplete message on SRB3/NR Cell 1? SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Steps 5-6" siclog@ //@sic R5-190465, R5-199030, R5-201230 sic@ //The SS transmits an RRCReconfiguration message to reconfigure NR PDCP sent on SRB3/NR Cell 1. //Check: Does the UE transmit an RRCReconfigurationComplete message on SRB3/NR Cell 1? v_RadioBearerConfig := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3, cs_PDCP_Config_SRB(ms20))},//@sic R5s200419 sic@ -, -, omit); SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig))); SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Steps 7 - 8" siclog@ v_RadioBearerConfig := cs_NR_RadioBearer_ReleaseSRB3; v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, v_Rlc_BearerToReleaseList); f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig, v_SCGConfig); // Now send octetstrings to EUTRA //Release SRB3 in SS f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB3)});