3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210237

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4587 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Guard timers for NBIOT testcases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The guard timers for the testcases listed below are insufficient when ran with a device which has multiple bands enabled. This causes these testcases to timeout in middle of execution |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extended the guard timers of the testcases listed below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase fail because of guard timer timeout |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.22, 22.5.1, 22.2.2, 22.5.5, 22.2.1, 22.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	function TC_22_4_22	4

1.2	function TC_22_5_1	5

1.3	function TC_22_2_2	6

1.4	function TC_22_5_5	6

1.5	function TC_22_2_1	7

1.6	function TC_22_4_1	8

# Corrections required

## function TC_22_4_22

| Function name | TC_22_4_22 |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the guard timer is set to 300s, which is insufficient when the testcase is run with a device which has multiple bands enabled. The guard timer times out in the middle of testcase thus causing testcase failure. |
| Summary of change | Increased the guard timer from 300s 600s. |
| TTCN module | ATS_NBIOT/NBIOT_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_22_4_22() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / Radio link failure / T301 expiry / T311 expiry / Dedicated RLF timer (UP/S1-U) var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(300); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_4_22_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

After Change:

testcase TC_22_4_22() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / Radio link failure / T301 expiry / T311 expiry / Dedicated RLF timer (UP/S1-U) var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(600); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_4_22_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

## function TC_22_5_1

| Function name | TC_22_5_1 |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the guard timer is set to 1200s, which is insufficient when the testcase is run with a device which has multiple bands enabled. The guard timer times out in the middle of testcase thus causing testcase failure. |
| Summary of change | Increased the guard timer from 1200s 2400s. |
| TTCN module | ATS_NBIOT/NBIOT_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_22_5_1() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / Authentication not accepted by the network, GUTI used / Authentication not accepted by the UE, SQN failure / Authentication not accepted by the UE, non-EPS authentication unacceptable / Network failing the authentication check var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(1200); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_5_1_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

After Change:

testcase TC_22_5_1() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / Authentication not accepted by the network, GUTI used / Authentication not accepted by the UE, SQN failure / Authentication not accepted by the UE, non-EPS authentication unacceptable / Network failing the authentication check var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(2400); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_5_1_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

## function TC_22_2_2

| Function name | TC_22_2_2 |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the guard timer is set to 900s, which is insufficient when the testcase is run with a device which has multiple bands enabled. The guard timer times out in the middle of testcase thus causing testcase failure. |
| Summary of change | Increased the guard timer from 900s 1800s. |
| TTCN module | ATS_NBIOT/NBIOT_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_22_2_2() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Manual mode var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(900); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_2_2_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

After Change:

testcase TC_22_2_2() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Manual mode var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(1800); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_2_2_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

## function TC_22_5_5

| Function name | TC_22_5_5 |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the guard timer is set to 3000s, which is insufficient when the testcase is run with a device which has multiple bands enabled. The guard timer times out in the middle of testcase thus causing testcase failure. |
| Summary of change | Increased the guard timer from 3000s 6000s. |
| TTCN module | ATS_NBIOT/NBIOT_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_22_5_5() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / Attach Procedure / Success / List of equivalent PLMNs in the ATTACH ACCEPT message / Attach / Rejected / PLMN not allowed var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(3000); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_5_5_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

After Change:

testcase TC_22_5_5() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / Attach Procedure / Success / List of equivalent PLMNs in the ATTACH ACCEPT message / Attach / Rejected / PLMN not allowed var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(6000); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_5_5_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

## function TC_22_2_1

| Function name | TC_22_2_1 |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the guard timer is set to 15300s, which is insufficient when the testcase is run with a device which has multiple bands enabled. The guard timer times out in the middle of testcase thus causing testcase failure. |
| Summary of change | Increased the guard timer from 15300s 25300s. |
| TTCN module | ATS_NBIOT/NBIOT_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_22_2_1() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(15300); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_2_1_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

After Change:

testcase TC_22_2_1() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(25300); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_2_1_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

## function TC_22_4_1

| Function name | TC_22_4_1 |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the guard timer is set to 21300s, which is insufficient when the testcase is run with a device which has multiple bands enabled. The guard timer times out in the middle of testcase thus causing testcase failure. |
| Summary of change | Increased the guard timer from 21300s 30300s. |
| TTCN module | ATS_NBIOT/NBIOT_Testsuite.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_22_4_1() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / Notification of BCCH modification in idle mode / eDRX cycle longer than the modification period var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(21300); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_4_1_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }

After Change:

testcase TC_22_4_1() runs on MTC_NBIOT system SYSTEM_NBIOT { // @purpose // NB-IoT / Notification of BCCH modification in idle mode / eDRX cycle longer than the modification period var NBIOT_PTC v_NBIOT := null; var EUTRA_PTC v_EUTRA := null; timer t_GuardTimer := int2float(30300); v_NBIOT := NBIOT_PTC.create alive; f_MTC_ConnectPTCs_NBIOT(system, v_NBIOT, v_EUTRA); v_NBIOT.start(f_TC_22_4_1_NBIOT()); t_GuardTimer.start; f_MTC_MainLoop_NBIOT(t_GuardTimer); }