3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210238

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1583 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC test case 10.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For test case 10.2.2.1.ENDC, Step 4 requires 12Gbps maximum bit rate while Step 8 requires 16Gbps maximum bit rate. In current TTCN, AT command for both Step 4 and Step 8 is set to AT+CGEQOS=6,1,128,128,384,384. It is suggested to extend f_ConvertEPSQoS_ForATCommand to support Extended EPS QoS. AT commands for Step 4 and Step 8 will become: Step 4 AT+CGEQOS=6,1,128,128,12000000,384 Step 8 AT+CGEQOS=6,1,128,128,16000000,384 - In current TTCN, Step 1 uses packet filter identifier = 3 while Step 8 uses packet filter identifier = 1. As test requirement only need to modify maximum bit rate, packet filter identifier in Steps 1 and 8 should be the same. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extend f_ConvertEPSQoS_ForATCommand to support Extended EPS QoS. - Set packet filer IEI to ‘2’H in Step 8 which will be converted to packet filter identifier = 3 in AT+CGTFT command. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_ConvertEPSQoS_ForATCommand |
| --- | --- |
| Reason for change | For test case 10.2.2.1.ENDC, Step 4 requires 12Gbps maximum bit rate while Step 8 requires 16Gbps maximum bit rate. In current TTCN, AT command for both Step 4 and Step 8 is set to AT+CGEQOS=6,1,128,128,384,384. It is suggested to extend f_ConvertEPSQoS_ForATCommand to support Extended EPS QoS. AT commands for Step 4 and Step 8 will become: Step 4 AT+CGEQOS=6,1,128,128,12000000,384 Step 8 AT+CGEQOS=6,1,128,128,16000000,384 |
| Summary of change | Extend f_ConvertEPSQoS_ForATCommand to support Extended EPS QoS. |
| TTCN module | EPS_NAS_Templates |
| MCC160 Comment | Accepted in principle but implemented a new, complete solution, shown below |

Before change:

function f_ConvertEPSQoS_ForATCommand(template (value) EPS_QualityOfService p_QoS) return charstring

{

var EPS_QualityOfService v_QoS := valueof(p_QoS);

var charstring v_QoSstring := "";

v_QoSstring := v_QoSstring &","& int2str(bit2int(v_QoS.qci));

v_QoSstring := v_QoSstring &","& int2str(f_QoS_MaximumBitRate(v_QoS.guaranteedBitRateDl)); // @sic R5s100557 sic@

v_QoSstring := v_QoSstring &","& int2str(f_QoS_MaximumBitRate(v_QoS.guaranteedBitRateUl)); // @sic R5s100557 sic@

v_QoSstring := v_QoSstring &","& int2str(f_QoS_MaximumBitRate(v_QoS.maxBitRateDl)); // @sic R5s100557 sic@

v_QoSstring := v_QoSstring &","& int2str(f_QoS_MaximumBitRate(v_QoS.maxBitRateUl)); // @sic R5s100557 sic@

// Extended rates are not dealt with at the moment (all set to 0 in current 36.508)

return v_QoSstring;

}

After change:

function f_ConvertEPSQoS_ForATCommand(template (value) EPS_QualityOfService p_QoS,

template (omit) Extd_EPS_QOS p_Extd_EPS_QOS := omit) return charstring

{

var EPS_QualityOfService v_QoS := valueof(p_QoS);

var charstring v_QoSstring := "";

v_QoSstring := v_QoSstring &","& int2str(bit2int(v_QoS.qci));

v_QoSstring := v_QoSstring &","& int2str(f_QoS_MaximumBitRate(v_QoS.guaranteedBitRateDl)); // @sic R5s100557 sic@

v_QoSstring := v_QoSstring &","& int2str(f_QoS_MaximumBitRate(v_QoS.guaranteedBitRateUl)); // @sic R5s100557 sic@

if(not isvalue(p_Extd_EPS_QOS)){

v_QoSstring := v_QoSstring &","& int2str(f_QoS_MaximumBitRate(v_QoS.maxBitRateDl)); // @sic R5s100557 sic@

} else {

v_QoSstring := v_QoSstring &","& int2str(bit2int(p_Extd_EPS_QOS.maxBitRateDl_2)) &"000000"; // @sic R5s100557 sic@

}

v_QoSstring := v_QoSstring &","& int2str(f_QoS_MaximumBitRate(v_QoS.maxBitRateUl)); // @sic R5s100557 sic@

// Extended rates are not dealt with at the moment (all set to 0 in current 36.508)

return v_QoSstring;

}

MCC160 Implementation

function f_ConvertEPSQoS_ForATCommand(template (value) EPS_QualityOfService p_QoS,

template (omit) Extd_EPS_QOS p_Extd := omit) return charstring

{

var EPS_QualityOfService v_QoS := valueof(p_QoS);

var charstring v_QoSstring := "";

var integer v_GuaranteeDL;

var integer v_GuaranteeUL;

var integer v_MaxDL;

var integer v_MaxUL;

var Extd_EPS_QOS v_Extd_QOS;

var B16_Type v_ExtdBitstring;

// @sic R5s210238 sic@

v_GuaranteeDL := fl_QoS_MaximumBitRateExtd_2(fl_QoS_MaximumBitRateExtd(f_QoS_MaximumBitRate(v_QoS.guaranteedBitRateDl), v_QoS.guaranteedBitRateDlExt), v_QoS.guaranteedBitRateDL_Ext2); // @sic R5s100557 sic@

v_GuaranteeUL := fl_QoS_MaximumBitRateExtd_2(fl_QoS_MaximumBitRateExtd(f_QoS_MaximumBitRate(v_QoS.guaranteedBitRateUl), v_QoS.guaranteedBitRateUlExt), v_QoS.guaranteedBitRateUL_Ext2); // @sic R5s100557 sic@

v_MaxDL := fl_QoS_MaximumBitRateExtd_2(fl_QoS_MaximumBitRateExtd(f_QoS_MaximumBitRate(v_QoS.maxBitRateDl), v_QoS.maxBitRateDlExt), v_QoS.maxBitRateDL_Ext2); // @sic R5s100557 sic@

v_MaxUL := fl_QoS_MaximumBitRateExtd_2(fl_QoS_MaximumBitRateExtd(f_QoS_MaximumBitRate(v_QoS.maxBitRateUl), v_QoS.maxBitRateUlExt), v_QoS.maxBitRateUL_Ext2); // @sic R5s100557 sic@

if (isvalue(p_Extd)) { // @sic R5s210238 sic@

v_Extd_QOS := valueof(p_Extd);

v_ExtdBitstring := v_Extd_QOS.maxBitRateUl_1 & v_Extd_QOS.maxBitRateUl_2;

if (bit2int(v_ExtdBitstring) > 0) {

v_MaxUL := fl_Extd_QoS_MaximumBitRate(v_Extd_QOS.unitMaxBitRate, v_ExtdBitstring);

}

v_ExtdBitstring := v_Extd_QOS.maxBitRateDl_1 & v_Extd_QOS.maxBitRateDl_2;

if (bit2int(v_ExtdBitstring) > 0) {

v_MaxDL := fl_Extd_QoS_MaximumBitRate(v_Extd_QOS.unitMaxBitRate, v_ExtdBitstring);

}

v_ExtdBitstring := v_Extd_QOS.guaranteedBitRateUl_1 & v_Extd_QOS.guaranteedBitRateUl_2;

if (bit2int(v_ExtdBitstring) > 0) {

v_GuaranteeUL := fl_Extd_QoS_MaximumBitRate(v_Extd_QOS.unitGuaranteedBitRate, v_ExtdBitstring);

}

v_ExtdBitstring := v_Extd_QOS.guaranteedBitRateDl_1 & v_Extd_QOS.guaranteedBitRateDl_2;

if (bit2int(v_ExtdBitstring) > 0) {

v_GuaranteeDL := fl_Extd_QoS_MaximumBitRate(v_Extd_QOS.unitGuaranteedBitRate, v_ExtdBitstring);

}

}

v_QoSstring := v_QoSstring &","& int2str(bit2int(v_QoS.qci));

v_QoSstring := v_QoSstring &","& int2str(v_GuaranteeDL); // @sic R5s100557, R5s210238 sic@

v_QoSstring := v_QoSstring &","& int2str(v_GuaranteeUL); // @sic R5s100557, R5s210238 sic@

v_QoSstring := v_QoSstring &","& int2str(v_MaxDL); // @sic R5s100557, R5s210238 sic@

v_QoSstring := v_QoSstring &","& int2str(v_MaxUL); // @sic R5s100557, R5s210238 sic@

return v_QoSstring;

}

/*

* @desc      To convert the maximum bit rate (extended) for the QoS

* @param     p_CurrentBitRate

* @param     p_Val

* @return    integer

*/

function fl_QoS_MaximumBitRateExtd(integer p_CurrentBitRate, template (omit) B8_Type p_Val) return integer

{

var integer v_Val;

if (isvalue(p_Val)) {

v_Val := bit2int(valueof(p_Val));

if ((v_Val >= bit2int('00000001'B)) and (v_Val <= bit2int('01001010'B))) {

return (8600 + (v_Val * 100));

}

if ((v_Val >= bit2int('00000001'B)) and (v_Val <= bit2int('01001010'B))) {

return (8600 + (v_Val * 100));

}

if ((v_Val >= bit2int('01001011'B)) and (v_Val <= bit2int('10111010'B))) {

return (16 + ((v_Val - 74) * 1000));

}

if ((v_Val >= bit2int('10111011'B)) and (v_Val <= bit2int('11111010'B))) {

return (128 + ((v_Val - 186) * 2000));

}

}

// if any other value (including 0) just return previously calculated value

return p_CurrentBitRate;

}

/*

* @desc      To convert the maximum bit rate (extended-2) for the QoS

* @param     p_CurrentBitRate

* @param     p_Val

* @return    integer

*/

function fl_QoS_MaximumBitRateExtd_2(integer p_CurrentBitRate, B8_Type p_Val) return integer

{

var integer v_Val;

if (isvalue(p_Val)) {

v_Val := bit2int(valueof(p_Val));

if ((v_Val >= bit2int('00000001'B)) and (v_Val <= bit2int('00111101'B))) {

return (256000 + (v_Val * 4000));

}

if ((v_Val >= bit2int('00111110'B)) and (v_Val <= bit2int('10100001'B))) {

return (500000 + ((v_Val - 61) * 10000));

}

if ((v_Val >= bit2int('10100010'B)) and (v_Val <= bit2int('11110110'B))) {

return (1500000 + ((v_Val - 161) * 100000));

}

}

// if any other value, including 0, just return previously calculated value

return p_CurrentBitRate;

}

/*

* @desc      To convert the Extended maximum bit rate for the Extended QoS

*            according to 24.301 table 9.9.4.30.1

* @param     p_Val

* @param     p_Unit

* @return    integer

*/

function fl_Extd_QoS_MaximumBitRate(B8_Type p_Unit, B16_Type p_Val) return integer

{

var integer v_Val := bit2int(p_Val);

var integer v_Multiplier := 0;

var integer v_Unit := 0;

select (bit2int(p_Unit)) {

case (0, 1) {

v_Multiplier := 200;

}

case (2, 3, 4, 5, 6) {

v_Multiplier := 1000;

}

case (7, 8, 9, 10, 11) {

v_Multiplier := 1000 * 1000;

}

case (12, 13, 14, 15, 16) {

v_Multiplier := 1000 * 1000 * 1000;

}

case (17, 18, 19, 20, 21) {

v_Multiplier := 1000 * 1000 * 1000 * 1000;

}

}

select (bit2int(p_Unit)) {

case (0, 1, 2, 7, 12, 17) {

v_Unit := 1;

}

case (3, 8, 13, 18) {

v_Unit := 4;

}

case (4, 9, 14, 19) {

v_Unit := 16;

}

case (5, 10, 15, 20) {

v_Unit := 64;

}

case (6, 11, 16, 21) {

v_Unit := 256;

}

}

return (v_Val * v_Unit * v_Multiplier);

}

## 2.2 Change 2

| Function name | fl_TC_10_2_2_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - As stated in Change 1, f_ConvertEPSQoS_ForATCommand will be extended to support Extended EPS QoS. - In current TTCN, Step 1 uses packet filter identifier = 3 while Step 8 uses packet filter identifier = 1. As test requirement only need to modify maximum bit rate, packet filter identifier in Steps 1 and 8 should be the same. |
| Summary of change | - Pass cs_Extd_EPS_QOS template to f_ConvertEPSQoS_ForATCommand - Set packet filer IEI to ‘2’H in Step 8 which will be converted to packet filter identifier = 3 in AT+CGTFT command. |
| TTCN module | ESM_ENDC_EUTRA |
| MCC160 Comment | - Accepted - Accepted |

Before change:

function fl_TC_10_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var ProcedureTransactionIdentifier v_EpsPti;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params;

var template (value) NAS_DL_Message_Type v_ModifyEPSBearerCxtReq;

var template (value)  NAS_MSG_Request_Type v_ActivateBearer;

var SRB_COMMON_IND v_ReceivedAsp;

var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; //@sic R5s190323 sic@

// Cause the UE to request bearer resource allocation of dedicated EPS bearer associated with first PDN connectivity

//@siclog "Step 1" siclog@

f_UT_RequestActivateDedicatedBearer(UT,

hex2int(tsc_EpsDedicatedBearerId),

hex2int(tsc_EpsDefaultBearerId2ndPDN),

f_ConvertEPSQoS_ForATCommand(cs_508_EPS_QoS_Dedicated_1),  // @sic R5-192186 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(2, tsc_EpsDedicatedBearerId));

// The UE transmits a SERVICE REQUEST

// The SS establishes SRB2 and DRB associated with default EPS bearer context obtained during the attach procedure.

//@siclog "Step 2 - 3" siclog@

v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(eutra_Cell1, MCG_SCG, RRC_IDLE);  //@sic R5s190323 sic@

f_EUTRA38_508_ENDC_RbEst_Steps3_10 (eutra_Cell1, omit, omit, omit, omit, omit, v_RadioResourceConfigDedicated); //@sic R5s190323 sic@

// Check : Does the UE transmits a Bearer Resource Allocation REQ ?

//@siclog "Step 4" siclog@

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_BearerResourceAllocReq(?,

tsc_EpsDefaultBearerId2ndPDN, //@sic R5s190323 sic@

cds_508_EPS_QoS(-, '00000001'B),// @sic R5-197006 sic@ @sic R5s200559 sic@

cr_Extd_EPS_QOS)))) -> value v_ReceivedAsp; // @sic R5-197006 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

// store PTI received

v_EpsPti := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.bEARER_RESOURCE_ALLOCATION_REQUEST.procedureTransactionIdentifier;

//Create the Activate Dedicated EPS Bearer Context REQ according to Table 10.2.2.1.3.3-2 in 38.523-1

v_ActivateBearer := f_EPS_GetActivateDedicatedBearer(tsc_EpsDedicatedBearerId,

cds_508_EPS_QoS(-, '00000001'B),// @sic R5-197006 sic@ @sic R5s200559 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId),cs_Extd_EPS_QOS);

v_ActivateBearer.Pdu.Msg.aCTIVATE_DEDICATED_EPS_BEARER_CONTEXT_REQUEST.procedureTransactionIdentifier := v_EpsPti;

// Update SgNB Keys:

v_Security_Params := f_EUTRA38_Security_Get (); // @sic R5s200045 sic@

// @siclog "Steps 5 - 6" siclog@

f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, MCG_SCG, v_Security_Params.SK_Counter, RRC_CONNECTED, {v_ActivateBearer}); //@sic R5s190323 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Steps 7" siclog@

// As per NOTE 2 CGCMOD is used

f_UT_RequestModifyDedicatedBearer(UT,

hex2int(tsc_EpsDedicatedBearerId),

hex2int(tsc_EpsDefaultBearerId2ndPDN),

f_ConvertEPSQoS_ForATCommand(cs_508_EPS_QoS_Dedicated_1),  // @sic R5-192186 sic@

f_EUTRA_Get_508_TrafficFlowTemplate_2_Modify(tsc_EpsDedicatedBearerId));

//@siclog "Step 8" siclog@

// Check : Does the UE transmit a BEARER RESOURCE MODIFICATION REQUEST message ?

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cdr_BearerResourceModReq(?,

tsc_EpsDedicatedBearerId,

omit,

cr_Tft(-),

cds_508_EPS_QoS('5B'O, '00000001'B),// @sic R5-197006 sic@ //@sic R5s190323 sic@ @sic R5s190920 sic@

cr_Extd_EPS_QOS(-,-,-))))) -> value v_ReceivedAsp; //@sic R5s190323 sic@ //@sic R5s190702 sic@ // @sic R5-197006 sic@ // @sic R5s191073 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

v_EpsPti := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.bEARER_RESOURCE_MODIFICATION_REQUEST.procedureTransactionIdentifier;

//Create the Modify EPS Bearer Context REQ to be sent at Step 9 according to Table 10.2.2.1.3.3-4 in 38.523-1

v_ModifyEPSBearerCxtReq := cs_ModifyEPSBearerCxtReq_AllParams(tsc_EpsDedicatedBearerId,

v_EpsPti,

cds_508_EPS_QoS('5B'O, '00000001'B),// @sic R5-197006 sic@ // @sic R5s191073 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId),

omit,

omit,

omit,

f_Get_508_PFI(),

cs_APN_AMBR,

omit,

omit,

omit,

omit,

cs_Extd_APN_AMBR,

cs_Extd_EPS_QOS ('00010000'B)); //@sic R5s190323 sic@ //@sic R5s190702 sic@

// @siclog "Steps 9" siclog@

SRB.send(cas_SRB2_NasPdu_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

v_ModifyEPSBearerCxtReq)));

//  Check:  Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message as specified?

//@siclog "Step 10" siclog@

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_ModifyEPSBearerCxtAccept(tsc_EpsDedicatedBearerId))));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 10");

} // End of function f_TC_10_2_2_1_ENDC_EUTRA_TestBody

After change:

function fl_TC_10_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var ProcedureTransactionIdentifier v_EpsPti;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params;

var template (value) NAS_DL_Message_Type v_ModifyEPSBearerCxtReq;

var template (value)  NAS_MSG_Request_Type v_ActivateBearer;

var SRB_COMMON_IND v_ReceivedAsp;

var template (omit) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; //@sic R5s190323 sic@

// Cause the UE to request bearer resource allocation of dedicated EPS bearer associated with first PDN connectivity

//@siclog "Step 1" siclog@

f_UT_RequestActivateDedicatedBearer(UT,

hex2int(tsc_EpsDedicatedBearerId),

hex2int(tsc_EpsDefaultBearerId2ndPDN),

f_ConvertEPSQoS_ForATCommand(cs_508_EPS_QoS_Dedicated_1, cs_Extd_EPS_QOS),  // @sic R5-192186 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(2, tsc_EpsDedicatedBearerId));

// The UE transmits a SERVICE REQUEST

// The SS establishes SRB2 and DRB associated with default EPS bearer context obtained during the attach procedure.

//@siclog "Step 2 - 3" siclog@

v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(eutra_Cell1, MCG_SCG, RRC_IDLE);  //@sic R5s190323 sic@

f_EUTRA38_508_ENDC_RbEst_Steps3_10 (eutra_Cell1, omit, omit, omit, omit, omit, v_RadioResourceConfigDedicated); //@sic R5s190323 sic@

// Check : Does the UE transmits a Bearer Resource Allocation REQ ?

//@siclog "Step 4" siclog@

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_BearerResourceAllocReq(?,

tsc_EpsDefaultBearerId2ndPDN, //@sic R5s190323 sic@

cds_508_EPS_QoS(-, '00000001'B),// @sic R5-197006 sic@ @sic R5s200559 sic@

cr_Extd_EPS_QOS)))) -> value v_ReceivedAsp; // @sic R5-197006 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

// store PTI received

v_EpsPti := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.bEARER_RESOURCE_ALLOCATION_REQUEST.procedureTransactionIdentifier;

//Create the Activate Dedicated EPS Bearer Context REQ according to Table 10.2.2.1.3.3-2 in 38.523-1

v_ActivateBearer := f_EPS_GetActivateDedicatedBearer(tsc_EpsDedicatedBearerId,

cds_508_EPS_QoS(-, '00000001'B),// @sic R5-197006 sic@ @sic R5s200559 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId),cs_Extd_EPS_QOS);

v_ActivateBearer.Pdu.Msg.aCTIVATE_DEDICATED_EPS_BEARER_CONTEXT_REQUEST.procedureTransactionIdentifier := v_EpsPti;

// Update SgNB Keys:

v_Security_Params := f_EUTRA38_Security_Get (); // @sic R5s200045 sic@

// @siclog "Steps 5 - 6" siclog@

f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, MCG_SCG, v_Security_Params.SK_Counter, RRC_CONNECTED, {v_ActivateBearer}); //@sic R5s190323 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Steps 7" siclog@

// As per NOTE 2 CGCMOD is used

f_UT_RequestModifyDedicatedBearer(UT,

hex2int(tsc_EpsDedicatedBearerId),

hex2int(tsc_EpsDefaultBearerId2ndPDN),

f_ConvertEPSQoS_ForATCommand(cs_508_EPS_QoS_Dedicated_1, cs_Extd_EPS_QOS ('00010000'B)),  // @sic R5-192186 sic@

f_EUTRA_Get_508_TrafficFlowTemplate_2_Modify(tsc_EpsDedicatedBearerId, -, '2'H));

//@siclog "Step 8" siclog@

// Check : Does the UE transmit a BEARER RESOURCE MODIFICATION REQUEST message ?

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cdr_BearerResourceModReq(?,

tsc_EpsDedicatedBearerId,

omit,

cr_Tft(-),

cds_508_EPS_QoS('5B'O, '00000001'B),// @sic R5-197006 sic@ //@sic R5s190323 sic@ @sic R5s190920 sic@

cr_Extd_EPS_QOS(-,-,-))))) -> value v_ReceivedAsp; //@sic R5s190323 sic@ //@sic R5s190702 sic@ // @sic R5-197006 sic@ // @sic R5s191073 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

v_EpsPti := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.bEARER_RESOURCE_MODIFICATION_REQUEST.procedureTransactionIdentifier;

//Create the Modify EPS Bearer Context REQ to be sent at Step 9 according to Table 10.2.2.1.3.3-4 in 38.523-1

v_ModifyEPSBearerCxtReq := cs_ModifyEPSBearerCxtReq_AllParams(tsc_EpsDedicatedBearerId,

v_EpsPti,

cds_508_EPS_QoS('5B'O, '00000001'B),// @sic R5-197006 sic@ // @sic R5s191073 sic@

f_EUTRA_Get_508_TrafficFlowTemplate(6, tsc_EpsDedicatedBearerId),

omit,

omit,

omit,

f_Get_508_PFI(),

cs_APN_AMBR,

omit,

omit,

omit,

omit,

cs_Extd_APN_AMBR,

cs_Extd_EPS_QOS ('00010000'B)); //@sic R5s190323 sic@ //@sic R5s190702 sic@

// @siclog "Steps 9" siclog@

SRB.send(cas_SRB2_NasPdu_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

v_ModifyEPSBearerCxtReq)));

//  Check:  Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message as specified?

//@siclog "Step 10" siclog@

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_ModifyEPSBearerCxtAccept(tsc_EpsDedicatedBearerId))));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 10");

} // End of function f_TC_10_2_2_1_ENDC_EUTRA_TestBody