3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210243

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1588 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC RRC test case 8.1.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC RRC test case 8.1.1.2.4 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC test case 8.1.1.2.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

3	Branches executed	6

4	Execution Log Files	6

4.1	Qualcomm SM8350 + SDX60	6

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.1.2.4 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.1.1.2.4.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM8350 + SDX60

Verification Status:	PASS

# Corrections required

# 2.1 Change 1

| Function name | f_TC_8_1_1_2_4_NR5GC() |
| --- | --- |
| Reason for change | Following change 2, UE will be in IDLE state following the test body |
| Summary of change | Change the postamble state from STATE_CONNECTED_3A to STATE_IDLE_1A |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#90e |

Before change:

function f_TC_8_1_1_2_4_NR5GC() runs on NR5GC_PTC

{ // RRC connection establishment / Extended and spare fields in SI

f_NR5GC_Init(NR_2);  //Initialised as NR-2 to get SIB2 initialised by default - scheduling updated in test  case body

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

// Preamble: The UE is switched off

f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0A);

f_NR_TestBody_Set(true);

fl_TC_8_1_1_2_4_TestBody();

f_NR_TestBody_Set(false);

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

After change:

function function f_TC_8_1_1_2_4_NR5GC() runs on NR5GC_PTC

{ // RRC connection establishment / Extended and spare fields in SI

f_NR5GC_Init(NR_2);  //Initialised as NR-2 to get SIB2 initialised by default - scheduling updated in test case body

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

// Preamble: The UE is switched off

f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0A);

f_NR_TestBody_Set(true);

fl_TC_8_1_1_2_4_TestBody();

f_NR_TestBody_Set(false);

f_NR_Postamble(nr_Cell1, STATE_IDLE_1A);

# 2.2 Change 2

| Template name | fl_TC_8_1_1_2_4_TestBody() |
| --- | --- |
| Reason for change | - The UE is switched off at the beginning of the test case body so it is not possible to check the UE release using f_NR_MobileInfo_NRCap_Get() as this information is cleared at switch off (call to f_NR5GC_MobileInfo_Init() at the end of f_NR_UE_DeRegisterOnSwitchOff()) - Since the UE is initially switched on at step 2, it will initiate a registration procedure instead of service request procedure. |
| Summary of change | - Remove check on accessstratumRelease - Expect REGITRATION REQUEST instead of SERVICE REQUEST and call Table 4.5.2.2-2 Procedure instead of 4.5.4.3-2 procedure Note : A prose CR on 38.523-1 will be raised at RAN5#90 |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment | 1.- Accepted 2.- Accepted conditional to prose CR agreement at RAN5#90e |

Before change:

function fl_TC_8_1_1_2_4_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var UE_NR_Capability v_NrCapability := f_NR_MobileInfo_NRCap_Get();

var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell1);

if (not match (v_NrCapability.accessStratumRelease, rel15))

{

f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 UE only"); // FFS: expected proseCR to correct the TC applicability at RAN5#90-e. Already taken into account.

}

//@siclog "Step 1" siclog@

//System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH.

v_NR_CellInfo.Sysinfo.BcchInfo.MIB.message_.mib.spare := '1'B;  // Table 8.1.1.2.4.3.3-1: MIB

//Add SIB11 with SIB2 in SI[0] as per Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1)

v_NR_CellInfo.Sysinfo.BcchInfo.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[1].sib11_v1610 := valueof(cs_SIB11_r16);

f_NR_CellInfo_Set(nr_Cell1, v_NR_CellInfo);

f_NR_SS_ConfigureSysinfo(nr_Cell1, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell1));

//@siclog "Step 2" siclog@

//The UE is switched on.

f_UT_SwitchOnUE(UT);

//@siclog "Step 3" siclog@

// Does the UE transmit an RRCSetupRequest message on Cell 1?

f_NR_RRC_SetupRequest_Def(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3");

//@siclog "Step 4" siclog@

// The SS transmits an RRCSetup message on Cell 1.

f_NR_RRCSetup_Def (nr_Cell1);

//@siclog "Step 5" siclog@

// Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Service Setup procedure by including the SERVICE REQUEST message?

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),

tsc_SHT_IntegrityProtected);

//@siclog "Step 6 - 10a2" siclog@

// Steps 5 to 9a2 of the Service Accept procedure described in TS 38.508 Table 4.5.4.2-3 are performed on Cell 1.

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg);

}

After change:

function fl_TC_8_1_1_2_4_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var UE_NR_Capability v_NrCapability := f_NR_MobileInfo_NRCap_Get();

var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell1);

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

//REMOVED

/*  if (not match (v_NrCapability.accessStratumRelease, rel15))

{

f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 UE only"); // FFS: expected proseCR to correct the TC applicability at RAN5#90-e. Already taken into account.

} */

//@siclog "Step 1" siclog@

//System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH.

v_NR_CellInfo.Sysinfo.BcchInfo.MIB.message_.mib.spare := '1'B;  // Table 8.1.1.2.4.3.3-1: MIB

//Add SIB11 with SIB2 in SI[0] as per Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1)

v_NR_CellInfo.Sysinfo.BcchInfo.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[1].sib11_v1610 := valueof(cs_SIB11_r16);

f_NR_CellInfo_Set(nr_Cell1, v_NR_CellInfo);

f_NR_SS_ConfigureSysinfo(nr_Cell1, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell1));

//@siclog "Step 2" siclog@

//The UE is switched on.

f_UT_SwitchOnUE(UT);

//@siclog "Step 3" siclog@

// Does the UE transmit an RRCSetupRequest message on Cell 1?

f_NR_RRC_SetupRequest_Def(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3");

//@siclog "Step 4" siclog@

// The SS transmits an RRCSetup message on Cell 1.

f_NR_RRCSetup_Def (nr_Cell1);

//@siclog "Step 5" siclog@

// Check: Does the UE transmit an RRCSetupComplete message to confirm the successful completion of the connection establishment and to initiate REGISTRATION REQUEST message?

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),

(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

//@siclog "Step 6 - 10a2" siclog@

// Steps 5 to 9a2 of the Service Accept procedure described in TS 38.508 Table 4.5.4.2-3 are performed on Cell 1.

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_NoSecurity);

}

# Branches executed

The test case was executed on NR band n78

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350+SDX60 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_8_1_1_2_4_LOG.html

- PICS/PIXIT parameter file:

TC_8_1_1_2_4_PIXIT.html

# References

| [1] | R5s210246 : NR5GC FR1 : Supporting information for addition of NR5GC RRC test case 8.1.1.2.4 |
| --- | --- |