3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210263

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1590 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to cads_NR_CellConfig_C_RNTI_REQ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently, the TTCN-3 Core Languague definition "ETSI ES 201 873-1"does not clearly differenciate between "explicit omit" / "implicit omit" context. But the TTCN-3 code in RAN5 ATS is written under the assumption that "implicit omit" is used. For other context, this may result in optional template fields being left uninitialized when template modification mechanism as per "ETSI ES 201 873-1 clause 6.2.1 – Record type and values" is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added “implicit omit” to template declarations to initialize optional template fields. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation not in line with TTCN-3 Core Language definitions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Nearly all, as common NR template definitions are affected (see cl. 1.2). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections	3

1.1	Correction to template cads_NR_CellConfig_C_RNTI_REQ	3

1.2	Similar changes are required for other templates	4

# Corrections

## Correction to template cads_NR_CellConfig_C_RNTI_REQ

| Template name | cads_NR_CellConfig_C_RNTI_REQ |
| --- | --- |
| Reason for change | In the template definition of cads_NR_CellConfig_C_RNTI_REQ modifying cas_NR_CellConfigAllOmit_REQ, parameters CellTimingInfo and InitialCellPower are not initialized. This result in optional template fields being left uninitialized. |
| Summary of change | Added “implicit omit” to template declarations to initialize optional template fields. |
| TTCN module | NR_CellCfg_Templates |
| MCC160 Comment | Accepted in principle but implemented differently. See updated objects below. |

Before Change:

<<SKIPPED CODE>> template (value) NR_SYSTEM_CTRL_REQ cads_NR_CellConfig_C_RNTI_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) RNTI_Value_Type p_RNTI_Value, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ //@sic R5-206369: added p_FollowOnFlag sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag, p_FollowOnFlag), Request := { Cell := { AddOrReconfigure := { StaticResourceConfig := omit, CellConfigCommon := { C_RNTI:= p_RNTI_Value } } } } }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) NR_SYSTEM_CTRL_REQ cads_NR_CellConfig_C_RNTI_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) RNTI_Value_Type p_RNTI_Value, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ //@sic R5-206369: added p_FollowOnFlag sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag, p_FollowOnFlag), Request := { Cell := { AddOrReconfigure := { StaticResourceConfig := omit, CellConfigCommon := { C_RNTI:= p_RNTI_Value } } } } } with { optional "implicit omit" }; <<SKIPPED CODE>>

## Similar changes are required for other templates

in module NR_CellCfg_Templates

- cads_NR_ServingCellConfig_REQ.  Not updated, not needed

- cads_NR_RachProcedureConfig_REQ.  Not updated, not needed

- cads_NR_CellConfigPhysicalLayerUplink.  Not updated, not needed

- cads_NR_UplinkBWP_ConfigCommon_REQ.  Not updated, not needed

- cads_NR_ULGrantAllocation_REQ. See updated code below

- cads_NR_PDSCH_Config_REQ.  Not updated, not needed

- cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ.  Not updated, not needed

- cads_NR_SearchSpaceUlDciAssignment_REQ. See updated code below

module NR_CommonTemplates_Beams

- cads_NR_SSB_BeamArray_REQ. See updated code below

- cads_NR_CSI_Config_REQ. See updated code below

module NR5GC_SysinfoPaging

- cads_NR_ModifySysinfo_Def_REQ.  Not updated, not needed

- cads_NR_DciTriggerShortMessage_REQ.  Not updated, not needed

module MAC_BWP_Common_NR

- cads_NR_BWP_CONFIG_DL_REQ. See updated code below

- cads_NR_BWP_CONFIG_UL_REQ. See updated code below

modeule MAC_TC_Common_NR

cads_NR_DownlinkBWP_ConfigCommon_REQ. See updated code below

cads_NR_UL_DL_BWP_ConfigCommon_REQ. See updated code below

cads_NR_DciTriggerPDCCHOrder_REQ.  Not updated, not needed

cads_NR_TDD_UL_DL_ConfigCommon_REQ.  Not updated, not needed

## MCC160 Implementation

module NR_CellCfg_Templates

template (value) NR_SYSTEM_CTRL_REQ cads_NR_CellConfig_C_RNTI_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) RNTI_Value_Type p_RNTI_Value, template (value) NR_InitialCellPower_Type p_InitialCellPower, template (value) CellTimingInfo_Type p_CellTimingInfo, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ //@sic R5-206369: added p_FollowOnFlag; R5s210263 added CellTimingInfo and InitialCellPower sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag, p_FollowOnFlag), Request := { Cell := { AddOrReconfigure := { StaticResourceConfig := omit, CellConfigCommon := { C_RNTI:= p_RNTI_Value, CellTimingInfo := omit, InitialCellPower := omit } } } } }; template (value) NR_SYSTEM_CTRL_REQ cads_NR_CellConfigPhysicalLayerUplink(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template(value) NR_CellConfigPhysicalLayerUplink_Type p_Uplink, template (omit) boolean p_CnfFlag := tsc_NoCnfReq ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ //@sic R5s210263 added omit IEs Common and Downlink sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag), Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := omit, Uplink := p_Uplink } } } } }; template (value) NR_SYSTEM_CTRL_REQ cads_NR_ULGrantAllocation_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_DciUlInfo_Type p_DciUlInfo := omit, template (omit) NR_UplinkTimeAlignment_Synch_Type p_PUCCH_Synch := omit, template (omit) UL_GrantConfig_Type p_UL_GrantConfig := omit, template (omit) NR_ServingCellConfig_Type p_NR_ServingCellConfig := omit, template (omit) boolean p_CnfFlag := omit ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //@sic R5s210263 added omit IEs DL, DrxCtrl and MeasGapCtrl sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag), //@sic R5s200161 sic@ Request := { Cell := { AddOrReconfigure := { DcchDtchConfig := { DL := omit, UL := { SearchSpaceAndDci := { DciInfo := p_DciUlInfo }, PUCCH_Synch := p_PUCCH_Synch, GrantConfig := p_UL_GrantConfig }, DrxCtrl := omit, MeasGapCtrl := omit }, ServingCellConfig := p_NR_ServingCellConfig } } } }; template (value) NR_SYSTEM_CTRL_REQ cas_NR_UplinkTimeAlignment_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_UplinkTimeAlignment_Synch_Type p_PUCCH_Synch) := /* @status APPROVED (IMS, NR5GC, NR5GC_IRAT, POS) */ cads_NR_ULGrantAllocation_REQ(p_NR_CellId, p_TimingInfo, -, p_PUCCH_Synch); template (value) NR_SYSTEM_CTRL_REQ cads_NR_SearchSpaceUlDciAssignment_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_SearchSpaceUlDciAssignment_Type p_NR_SearchSpaceUlDciAssignment := omit, template (omit) boolean p_CnfFlag := omit ) modifies cas_NR_CellConfigAllOmit_REQ := { //@sic R5s210263 added omit IEs DL, DrxCtrl and MeasGapCtrl sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag), Request := { Cell := { AddOrReconfigure := { DcchDtchConfig := { DL := omit, UL := { SearchSpaceAndDci := p_NR_SearchSpaceUlDciAssignment, PUCCH_Synch := omit, GrantConfig := omit }, DrxCtrl := omit, MeasGapCtrl := omit } } } } };

module NR_CommonTemplates_Beams

template (value) NR_SYSTEM_CTRL_REQ cads_NR_SSB_BeamArray_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_SSB_BeamArray_Type p_SSB_BeamArray) modifies cas_NR_CellConfigAllOmit_REQ := { //@sic R5s210263 added omit IEs sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Downlink := { SSPbchBlock := { BurstConfig := { BlockPattern := omit, PositionsInBurst := omit, BeamArray := p_SSB_BeamArray } } } } } } } }; template (value) NR_SYSTEM_CTRL_REQ cads_NR_CSI_Config_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_CSI_Config_Type p_NR_CSI_Config) modifies cas_NR_CellConfigAllOmit_REQ := { //@sic R5s210263 added omit IEs sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := omit, CsiConfig := p_NR_CSI_Config } } } } } };

module MAC_BWP_Common_NR

template (value) NR_SYSTEM_CTRL_REQ cads_NR_BWP_CONFIG_DL_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, BWP_Id p_ActiveBWP, template (value) NR_AssignedBWPs_Type p_AssignedBWPs, template (omit) NR_DciDlInfo_Type p_DciDlInfo, template (omit) NR_DownlinkBWP_List_Type p_DownlinkBWP_List := omit) modifies cas_NR_CellConfigAllOmit_REQ := { //@sic R5s210263 added omit IEs sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := { ActiveBWP := p_ActiveBWP, BwpArray := p_DownlinkBWP_List }, CsiConfig := omit }, Uplink := omit }, DcchDtchConfig := { DL := { SearchSpaceAndDci := { AssignedBWPs := p_AssignedBWPs, SearchSpaceType := omit, DciInfo := p_DciDlInfo } }, UL := omit, DrxCtrl := omit, MeasGapCtrl := omit } } } } }; template (value) NR_SYSTEM_CTRL_REQ cads_NR_BWP_CONFIG_UL_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_ActiveUplinkBWP_Id_Type p_ActiveUplinkBWP_Id, template (value) NR_AssignedBWPs_Type p_AssignedBWPs, template (value) NR_DciUlInfo_Type p_DciUlInfo, template (omit) NR_UplinkBWP_List_Type p_UplinkBWP_List := omit) modifies cas_NR_CellConfigAllOmit_REQ := { //@sic R5s210263 added IE AddOrReconfigure->PhysicalLayer->Common sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := omit, Uplink := { Uplink := { Config:= { FrequencyInfoUL := omit, BWPs := { ActiveBWP := p_ActiveUplinkBWP_Id, BwpArray := p_UplinkBWP_List }, RACH_ConfigDedicated := omit, SI_RequestConfig := omit } } } }, DcchDtchConfig := { DL := omit, UL := { SearchSpaceAndDci := { AssignedBWPs := p_AssignedBWPs, SearchSpaceType := omit, DciInfo := p_DciUlInfo }, PUCCH_Synch := omit, GrantConfig := omit }, DrxCtrl := omit, MeasGapCtrl := omit } } } } };

module MAC_TC_Common_NR

template (value) NR_SYSTEM_CTRL_REQ cads_NR_DownlinkBWP_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_DownlinkBWP_List_Type p_NR_DownlinkBWP_List) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status */ //@sic R5s210263 added omit IEs sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := { ActiveBWP := tsc_NR_BWP_Id, BwpArray := p_NR_DownlinkBWP_List }, CsiConfig := omit } } } } } }; template (value) NR_SYSTEM_CTRL_REQ cads_NR_UL_DL_BWP_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_DownlinkBWP_List_Type p_NR_DownlinkBWP_List_Type, template (value) NR_UplinkBWP_List_Type p_NR_UplinkBWP_List) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status */ //@sic R5s210263 added omit IEs sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink:= { BWPs := { ActiveBWP := omit, BwpArray := p_NR_DownlinkBWP_List_Type } }, Uplink := { Uplink := { Config:= { FrequencyInfoUL := omit, BWPs := { ActiveBWP := omit, BwpArray := p_NR_UplinkBWP_List }, RACH_ConfigDedicated := omit, SI_RequestConfig := omit } } } } } } } };