3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210271

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1594 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 8.1.3.1.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In step9, Measurement Report Quantity is infinity. So SS will receive the Measurement Reports continuously. If we don’t ignore the Measurement Reports in step9, it will cause failure in step10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture and ignore Measurement Reports in step9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.23.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_3_1_23_TestBody() |
| --- | --- |
| Reason for change | In step9, Measurement Report Quantity is infinity. So SS will receive the Measurement Reports continuously. If we don’t ignore the Measurement Reports in step9, it will cause failure in step10. |
| Summary of change | Capture and ignore Measurement Reports in step9. |
| TTCN module | RRC_Measurement_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_3_1_23_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var NR_AbsoluteCellPower_Type v_CellPower_ServingCell_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_NeighbourCell_FR1 := -91; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (omit) MeasGapConfig v_MeasGapUE := omit; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var float v_TimerValue_5s := 5.0; var default v_MyDefaultVar := null; //@sic R5s201450 sic@ //Power levels of NR cell 1 and cell 2 FR1 acc. Table 8.1.3.1.23.3.2-1, Table 8.1.3.1.23.3.2-2 FFS /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -85 ¦ -91 * */ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_ServingCell_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, v_CellPower_NeighbourCell_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR periodic measurement reporting. //The UE transmits an RRCReconfigrationComplete message v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, cs_38508_ReportConfigNR_Periodical)}; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE ); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 2A" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-200803 sic@ //@siclog "Step 3" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 3"); //@siclog "Step 4" siclog@ //The SS transmits an RRCRelease message with suspendConfig. //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s201450 sic@ f_NR_RRC_InactiveDef(nr_Cell1); //@sic R5s201450 sic@ deactivate(v_MyDefaultVar); //@sic R5s201450 sic@ //@siclog "Step 5" siclog@ //The SS waits for 5 sec transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 6" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@siclog "Step 7" siclog@ //The SS transmits an RRCResume message without including MeasConfig. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 8" siclog@ //The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 8A" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-201356 sic@ //@siclog "Step 9" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 9"); //@siclog "Step 10" siclog@ //The SS transmits an RRCRelease message with suspendConfig f_NR_RRC_InactiveDef(nr_Cell1); //@siclog "Step 11" siclog@ //The SS waits for 5 sec and transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); //@sic R5-204441 sic@ f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@sic R5-204441 sic@ //@siclog "Step 12" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@sic R5-204441 sic@ //@siclog "Step 13" siclog@ //The SS transmits an RRCResume message including MeasConfig. v_MeasIdConfigList := {cs_NR_MeasId_Config_id2_obj1_conf1}; //@sic R5-204441 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE, -, -, {tsc_NR_MeasId1}); //@sic R5s201450, R5-204441 sic@ f_NR_SendRRCResume_Def(nr_Cell1, -, -, omit, omit, v_MeasConfig); //@sic R5-204441 sic@ //@siclog "Step 14" siclog@ //The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell1); //@sic R5-204441 sic@ //@siclog "Step 15" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-204441 sic@ //@siclog "Step 16" siclog@ //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId2, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450, R5-204441 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 16"); //@sic R5-204441 sic@ //@siclog "Step 17" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@sic R5-204441 sic@ }

After Change:

function f_TC_8_1_3_1_23_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT0; var NR_AbsoluteCellPower_Type v_CellPower_ServingCell_FR1 := -85; var NR_AbsoluteCellPower_Type v_CellPower_NeighbourCell_FR1 := -91; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var template (omit) MeasGapConfig v_MeasGapUE := omit; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var float v_TimerValue_5s := 5.0; var default v_MyDefaultVar := null; //@sic R5s201450 sic@ //Power levels of NR cell 1 and cell 2 FR1 acc. Table 8.1.3.1.23.3.2-1, Table 8.1.3.1.23.3.2-2 FFS /* ¦ Cell 1 ¦ Cell 2 * ------------------------------- * T0 ¦ -85 ¦ -91 * */ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_ServingCell_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, v_CellPower_NeighbourCell_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR periodic measurement reporting. //The UE transmits an RRCReconfigrationComplete message v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, cs_38508_ReportConfigNR_Periodical)}; v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE ); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 2A" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-200803 sic@ //@siclog "Step 3" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(nr_Cell2); // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 3"); //@siclog "Step 4" siclog@ //The SS transmits an RRCRelease message with suspendConfig. //The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@sic R5s201450 sic@ f_NR_RRC_InactiveDef(nr_Cell1); //@sic R5s201450 sic@ deactivate(v_MyDefaultVar); //@sic R5s201450 sic@ //@siclog "Step 5" siclog@ //The SS waits for 5 sec transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@siclog "Step 6" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@siclog "Step 7" siclog@ //The SS transmits an RRCResume message without including MeasConfig. f_NR_SendRRCResume_Def(nr_Cell1); //@siclog "Step 8" siclog@ //The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell1); //@siclog "Step 8A" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-201356 sic@ //@siclog "Step 9" siclog@ //Wait for 30 seconds to ensure that the UE performs a periodical intra/inter-frequency reporting for NR Cell 2 //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 9"); v_MyDefaultVar := activate(a_NR5GC_ReceiveMeasReport(nr_Cell1, v_PhysCellIdServing, v_PhysCellIdNeighbour)); //@siclog "Step 10" siclog@ //The SS transmits an RRCRelease message with suspendConfig f_NR_RRC_InactiveDef(nr_Cell1); deactivate(v_MyDefaultVar); //@siclog "Step 11" siclog@ //The SS waits for 5 sec and transmits a Paging message including a matched identity (correct full-RNTI). f_Delay(v_TimerValue_5s); //@sic R5-204441 sic@ f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); //@sic R5-204441 sic@ //@siclog "Step 12" siclog@ //Check: UE transmits RRCResumeRequest message //f_NR_RRC_ResumeRequest_Def(nr_Cell1, tsc_NR_ShortI_RNTI_Value1, mt_Access); //to be checked f_NR_RRC_ResumeRequest_Def(nr_Cell1, ?, mt_Access); //@sic R5-204441 sic@ //@siclog "Step 13" siclog@ //The SS transmits an RRCResume message including MeasConfig. v_MeasIdConfigList := {cs_NR_MeasId_Config_id2_obj1_conf1}; //@sic R5-204441 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapUE, -, -, {tsc_NR_MeasId1}); //@sic R5s201450, R5-204441 sic@ f_NR_SendRRCResume_Def(nr_Cell1, -, -, omit, omit, v_MeasConfig); //@sic R5-204441 sic@ //@siclog "Step 14" siclog@ //The UE transmits an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell1); //@sic R5-204441 sic@ //@siclog "Step 15" siclog@ //The SS waits and ingnores MeasurementReport messages for 3 seconds to allow UE to measure NR Cell 2 f_NR_IgnorePeriodicalMeasurmentReport(3.0); //@sic R5-204441 sic@ //@siclog "Step 16" siclog@ //Check: Does the UE transmit at least 10 MeasurementReport messages within the next 30 seconds? // 63 Reports for event A4/5 to ensure ca 30 sec (62*0.48ms=30.24s) f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId2, 63, 48.0, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour)})); //@sic R5s201450, R5-204441 sic@ f_NR_PreliminaryPass (__FILE__, __LINE__, "Step 16"); //@sic R5-204441 sic@ //@siclog "Step 17" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@sic R5-204441 sic@ }